3GPP TSG-WG SA2 Meeting #156e	S2-2304263R05

# April 17th – 21th, 2023, Electronic Meeting					                   (revision of S2-23XXXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2087 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New feature for 5G-RG to support NSWO procedure to authorize UE behind RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For 23.700-17 has concluded that “in order to support NSWO (as defined in clause 5.42 of TS 23.501) for 3GPP UE, 5G-RG shall support the SWa' interface to NSWOF but this aspect is in the scope of BBF and/or Cable Labs specifications.” Besides, in TS 23.501 clause 4.2.15, UE is allowed to use 5G credentials based on NSWO to authenticate to the 5GC to first obtain local IP address and then to use untrusted Non-3GPP access to register to the 5GC. Similarly, when UE is behind RG, the UE should also be able to use NSWO to first authenticate to 5GC. So propose a feature for 5G-RG to support NSWO procedure to authorize UE behind RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - propose a feature for 5G-RG to support NSWO procedure to authorize UE behind RG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G-RG does not support 5G NSWO procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10, 4.10d (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.10	UE behind 5G-RG and FN-RG

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5GC capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a UE, the control and user plane packets of the UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an IWF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 4.10-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

NOTE 1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered in this specification as it is assumed that FN-RG does not support EAP-based access control (e.g. 802.1X).

Figure 4.10-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via 5G-RG, NG-RAN or via both accesses.

The TNGF and Ta reference point are defined in TS 23.501 [2].

NOTE 2:	The reference architecture in figure 4.10-1 only shows the architecture and the network functions directly connected to W-5GAN or TNGF, and other parts of the architecture are the same as defined in clause 4.2 of TS 23.501 [2].

NOTE 3:	The reference architecture in figure 4.10-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 4:	The two N2 instances in Figure 4.10-1 apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and W-5GAN.

# * * * * Second change * * * *

## 4.10d	Support of NSWO for 3GPP UE behind a 5G-RG

In order to support NSWO defined in clause 5.42 of TS 23.501 [2] for 3GPP UE, 5G-RG may support the WLAN Access functionality defined in TS 23.501 [2], clause 4.2.15 and clause 5.42. The WLAN Access functionality includes the support of the SWa' interface to NSWOF as shown in Figure 4.10d-1 and Figure 4.10d -2. In this case the user plane packets of the UE are not transported via the 5G-RGs PDU Session and are also not traversing the UE’s 5GC. The user plane for NSWO may be terminated on a BNG or on a W-AGF where NWSO traffic is offloaded without traversing the 5G network c.

Editor’s note: whether and how to support W-AGF AAA proxy function requires further considerations

As specified in TS 23.501, clause 4.2.15, the SWa’ reference point corresponds to SWa reference point defined in TS 23.402 but where the EAP user ID is a SUCI and not an IMSI.

The SWa’ support in Wireline access network has no impact on 3GPP specifications.

NOTE: 	The specification of support of functionalities for NSWO in the wireline access network is under BBF and Cablelabs responsibility. This includes potential support of AAA proxy functionality as shown in TS 23.501, clause 4.2.15 allowing to carry AAA flows between the 5G-RG and the NSWOF over the network access of the 5G-RG.

Figure 4.10d-1: Reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Figure 4.X.1-2: Service based reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Editor’s Note: The specification in this clause may be further revised based on feedback from BBF and Cable Labs

Editor’s Note: the support of NSWO for FN-RG requires further considerations

# * * * * End of changes * * * *