3GPP TSG-SA2 Meeting #156	S2-2304283r03

# Elbonia, April, 17th – 21st 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3986 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to assistance for UE member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Assistance for UE member selection and some corresponding filtering criteria have been specified. To maintain the service quality of AIML services, the AF may indicate the NEF more filtering criteria for recommend candidate selected or deselected UEs, e.g. the unacceptable abnormal UE behaviours, the required registration and connection status of the UEs, and the acceptable jitter etc. During the AIML operation, if some members cannot fulfil the filtering criteria, they might be discarded from the service. In another case, due to the model variation by AF or UE mobility etc., the AF may need more UEs to join an ongoing operation. However, according to the current specified procedures and services, it is not clear how to update the members, which NF will trigger the member update, how to inform the AF of the updated members, whether the member update will be perfomed periodically or not etc. Therefore, the corresponding services, procedures and parameters of member update should be defined to fully support the AIML operation. Futhermore, the current specified procedures for UE current location filtering criteria cannot obtain the UE location in the finer granularity than cell level, if required. Therefore, GMLC should be added as a source to collect UE current location related data finer than cell ID level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the title of clause 4.15.13 to align the terminology with Clause 5.46 of TS 23.501. - Introduce new filtering criteria to assist the NEF in selecting and deselecting members; - Introduce Nnef_UEMemberSelectionAssistance_Discard service operation. - Introduce ‘Periodicity’ as an optional input of Nnef_UEMemberSelectionAssistance_Subscribe service operation to support periodic member updates. - Introduce member update procedures. - Introduce GMLC as a source to collect UE current location related data finer than cell level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - 5GC cannot inform AF the UEs that fulfil or cannot meet the requirement of AIML services. - the 5GC cannot support UE member update; as a result, AIML services may not operate. - the 5GC cannot collect UE current location related data finer than cell leve for filtering, if required. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13, 4.15.13.0, 4.15.13.1, 4.15.13.2, 4.15.13.3.2, 5.2.6.32.2, 5.2.6.32.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 4.15.13	Assistance for UE member selection

#### 4.15.13.0	General

AF provides a UE list and at least one UE member filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information from 5GC NFs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the UE member filtering criteria requested by the AF.

The member selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

AF leverages the 5GC network exposure for UE member selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how AF to leverage the 5GC network exposure for UE member selection can be referred to informative Annex I.

Editor's note:	The term "UE member selection" is FFS.

#### 4.15.13.1	UE member selection general information flow

This clause describes the procedures that are generally applicable independently of the UE member filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to UE Member selection and update

1.	AF subscribes the member selection assistance functionality by sending Nnef_UEMemberSelectionAssistance_subscribe request including a list of target UE(s), one or more UE member filtering criteria listed in the Table 4.15.13.2-1, and optionally, time window(s).

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executes the corresponding service operation based on the UE member filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

3.	NEF interacts with different 5GC network functions to collect the required information. The set of interactions between NEF and among 5GC NFs are dependent on the UE member filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UE(s) which fulfil the UE member filtering criteria in the AF request.

Editor's note:	whether the NEF stores any filtering information for member UE selection after providing candidate UEs to NEF is FFS.

5.	NEF sends a Nnef_UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information.

6	[optional] An update of the filtering criteria may be required. AF may send Nnef_UEMemberSelectionAssistance_Subscribe request including one or more updated member UE filtering criteria listed in the Table 4.15.13.2-1. If the AF does not indicate a list of target UE(s) to the NEF, the NEF uses the a list of target UE(s) received in Step 1 for the member UE update

7	[optional] NEF interacts with different 5GC network functions to collect the required information for the updated filtering criteria. The set of interactions between NEF and among 5GC NFs depend on the UE member filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

8.	[optional] Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of the candidate selected UEs and/or candidate deselected UEs which fulfil the UE member filtering criteria in the AF request.

9.	[optional] NEF sends a Nnef_UEMemberSelectionAssistance_Notify and/or Nnef_UEMemberSelectionAssistance_NotifyDiscard request to the AF including the list(s) of the candidate UEs and/or UEs that are not recommended to be selected, and possibly additional information.

#### 4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure or Ngmlc_Location service Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.3 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.3 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.3 |
| UE registration state, connectivity state and reachability | Indicate the registration state (Registered or Deregistered), connectivity state (IDLE or CONNECTED), and/or UE reachability (RRC state) of the selected UEs. | Service operation: Namf_EventExposure service Filter: a list of GPSI(s) or SUPI(s) Event ID: Reachability Filter | This can be provided by existing service operations, events, and procedures. |
| Variation of UE latency | Indicate the maximum variation of the transmission time (jitter) among the selected UEs. | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time Nsmf_EventExposure/ Nupf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | Clause 6.18 of TS 23.288 [50]. Editor's note: additional services for UE latency variation information collection are FFS. |
| UE Abnormal behaviour | Indicate the Exceptions for UE Abnormal behaviour, e.g. if the a UE abnormal behaviour is out of AF requirements, this UE will not be recommended by the NEF. | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = Abnormal behaviour Filter information may also include the Exceptions of different Exception IDs | Clause 6.7.5 of TS 23.288 [50]. |

Editor's note:	The list of UE Member filtering criteria and their corresponding actions are FFS.

Editor's note:	Each Filter Criteria may add a new clause where the specific procedures for such filter criteria are described. For example:"4.15.13.3	Specific procedure for criterial foo".

* * * Next Change * * *

##### 4.15.13.3.2	Specific procedure for the 5GC assistance to member selection based on the UE's current location, historical location and direction.

Figure 4.15.13.3.2-1: 5GC assistance to Member selection based on the UE's travel history, direction and separation distance

1.	AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

3a-4a.	If the required UE location is a finer granularity area than the Cell level, the NEF invokes Ngmlc_Location service operation with the UE IDs, the current AoI and event ID = Location report. GMLC will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from GMLC within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list received from step 4.

6-7. The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction as requested by AF. The NEF notifies AF for such UE candidate list.

10.and 10a.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify or Ngmlc_Location_LocationUpdateNotify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11(a)-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage and recommend deselecting the UE from the member UE list.

* * * Next Change * * *

##### 5.2.6.32.2	Nnef_UEMemberSelectionAssistance_Subscribe service operation

Service operation name: Nnef_UEMemberSelectionAssistance_Subscribe

Description: The NF consumer subscribes to receive the UE member selection assistance information, or the subscription is updated if the same subscription is already defined in NEF.

Inputs, Required: Target of Member Selection Assistance Reporting (GPSI or a list of GPSIs), Notification Target Address (+ Notification Correlation ID), at least one filtering criteria shown in Table 4.15.13.2-1.

Inputs, Optional: Application ID, Subscription Correlation ID (in the case of modification of the existing subscription), Expiry time, a set of UE member filtering criteria shown in the Table 4.15.13.2-1, time window(s) for selecting the candidate UEs, specific parameters depending on the UE member filtering criteria, Periodicity (the periodicity of member update), updated filtering criteria.

NOTE x:	Updated filtering criteria parameter is only included when the value of a filtering criterion previously provided by the AF needs to be updated.

Editor's note:	It is FFS whether the Target of Member Selection Assistance Reporting is mandatory when updated filtering criteria are provided.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

* * * Next Change * * *

##### 5.2.6.32.y	Nnef_UEMemberSelectionAssistance_NotifyDiscard service operation

Service operation name: Nnef_UEMemberSelectionAssistance_NotifyDiscard

Description: NEF reports the UEs not recommended to be selected to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information, one or more list(s) of UE(s) recommended not to be selected.

Inputs, Optional: reasons for discarding the corresponding UE(s).

Outputs, Required: Operation execution result indication.

* * *End of Changes * * *