3GPP TSG-SA WG2 Meeting #156-e	S2-2304287r01

# Elbonia, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0748 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service data from AF for E2E data volume transfer time analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-03-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The present version of the CR lacks clarity regarding the event ID required to collect data listed in "Table 6.18.2-3: Service Data from AF for E2E data volume transfer time analytics." Furthermore, it is unclear the measurement of data volume and its corresponding transmission time can pertain to UL or DL. The proposed CR addresses these issues by specifying the event ID necessary to gather data from AF and clarifying that the measurement can apply to either UL or DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding event ID to gather data from AF Clarifying UL/DL measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear which event ID should be used to gather data from AF, and there is ambiguity in the definition of the measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3987 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

### 6.18.2	Input Data

The NWDAF supporting analytics on E2E data volume transfer time shall be able to collect E2E data volume transfer time information from AF, OAM and 5GC NFs.

More detailed information collected by the NWDAF from the OAM is defined in the Table 6.18.2-1, from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.18.2-2, and from AF is defined in Table 6.18.2-3.

Table 6.18.2-1: Input data from OAM related to E2E data volume transfer time

| Information | Source | Description |
| --- | --- | --- |
| RAN part delay | OAM TS 28.554 [10] | Average packet transmission delay through the RAN part to the UE, per timeslot, per 5QI and per S-NSSAI. |
| Timestamp | OAM | A time stamp associated with the collected information. |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per QCI per UE packet delay as specified in clause 5.2.1.1 of TS 37.320 [20] and per DRB per UE packet delay as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per QCI per UE packet loss rate as specified in clause 5.2.1.1 of TS 37.320 [20] and the per DRB per UE packet loss rate as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| Distribution of UL/DL/round trip packed delay between PSA UPF and UE | OAM | The distribution of UL/DL/round trip packet delay between PSA UPF and UE as captured in TS 28.552 [8]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.18.2-1 by using the services provided by OAM as described in clause 6.2.3.

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s)).

NOTE 2:	User consent checking from UDM can apply for these analytics.

Table 6.18.2-2: Service Data from 5GC NFs for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | Location of the UE(s) needs to be selected via AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be selected via GMLC instead. |
| UE ID | AMF | (list of) SUPI(s). |
| 5QI | SMF | A reference to 5G QoS characteristics. |
| QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions. |

Table 6.18.2-3: Service Data from AF for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AF | Timestamp of the collected information. |
| Application ID | AF | Identifier of the application at the AF. |
| UE ID(s) | AF | Internal or External UE IDs (i.e. SUPI or GPSI, respectively). |
| Transmitted UL/DL data volume | AF | The volume of the transmitted UL/DL data. |
| UL/DL transmission time duration | AF | The time duration (start and end time) needed for sending the volume of UL/DL data. |
| Server location | AF | The location where the server (AF) is located. |

Editor's note:	Which AF event can be used to collect Server location is FFS.

NWDAF subscribes to the service data from AF in the 6.18.2-3 (except "Server location") either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = E2E data volume transfer time information, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)), or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = E2E data volume transfer time information, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)) as defined in TS 23.502 [3].

********** End of Changes **********