3GPP TSG-SA WG2 Meeting #156-e	S2-2304307r05

# 17 – 21 April, 2023, Electronic Meeting					                       (revision of S2-23XXXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0250 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of Support for Emergency Relaying |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Address remaining EN in clause 5.4.4.1 regarding indication of support for emergency relaying. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the indication is either existing serving RAN SIB indicating emergency support (as specified in TS 38.331 [16]) for 5G ProSe Layer-2 UE-to-Network Relay (see clause 5.4.4.2) or existing emergency service support indicator from the AMF in the latest Registration Accept message (see clause 5.4.4.3). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity on indication of support for emergency relaying. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.1, 5.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.4.4	Support of emergency service from 5G ProSe Remote UE via 5G ProSe UE-to-Network Relay

#### 5.4.4.1	General

When a 5G ProSe enabled UE does not have direct connection to the network for emergency service, the UE may attempt to obtain emergency service via 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay.

NOTE:	Direct connection refers to the UE connected to the network via Uu or WLAN. No direct connection to the network for emergency service includes also the case that the RAN broadcast SIB indicates no emergency support as specified in TS 23.122 [14].

A 5G ProSe enabled UE acting as 5G ProSe UE-to-Network Relay shall have a normal registration (including also normal registration for a 5G ProSe Relay enabled UE in Non-Allowed Area). A 5G ProSe enabled UE in limited-service state shall not act as 5G ProSe UE-to-Network Relay. Mobility Restrictions that are overruled for UE requesting direct emergency service are overruled also for 5G ProSe UE-to-Network Relay that is relaying emergency service as specified in clause 5.4.3.

A 5G ProSe enabled UE shall only advertise its support for relaying emergency service when the serving network has indicated support of relaying of emergency service. The indication is either existing serving RAN SIB indicating emergency support (as specified in TS 38.331 [16]) for 5G ProSe Layer-2 UE-to-Network Relay (see clause 5.4.4.2) or existing emergency service support indicator from the AMF in the latest Registration Accept message for 5G ProSe Layer-3 UE-to-Network Relay (see clause 5.4.4.3).

Dedicated RSC(s) for emergency service needs to be provisioned in the 5G ProSe enabled UEs with capability of 5G ProSe UE-to-Network Relay and/or 5G ProSe Remote UE as specified in clause 5.1.4.

The dedicated RSC(s) are used by the 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE during discovery and PC5 link establishment.

A PC5 link associated with a dedicated emergency RSC is only used for emergency service.

If the 5G ProSe UE-to-Network Relay needs to establish RRC Connection when the 5G ProSe Remote UE has requested emergency service over the PC5 link, the 5G ProSe UE-to-Network Relay shall use RRC establishment cause "emergency".

The existing positioning function as applicable is reused for the 5G ProSe Remote UE. If no other information is available, the location of the 5G ProSe UE-to-Network Relay can be used as Remote UE location estimate.

For more information on UE specific behavior and location handling, refer to TS 23.167 [31].

Editor’s Note:	Whether and how PC5 security is used for emergency services is to be determined as part of SA3 alignment.

#### 5.4.4.2	Emergency service from 5G ProSe Remote UE via via 5G ProSe Layer-2 UE-to-Network Relay

For a 5G ProSe Layer-2 UE-to-Network Relay to advertise its support of emergency service, the existing serving NG-RAN support of emergency services (as specified in TS 38.331 [16]) is used. The 5G ProSe Layer-2 Remote UE may select a different PLMN from the 5G ProSe Layer-2 Relay UE-to-Network Relay.

**** END OF CHANGES ****