3GPP TSG-WG SA2 Meeting #156-e	S2-2304328r01

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0220 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update group message delivery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Service announcement can be done by MBSF or AF. In case of AF providing service announcement to the UE, the AF itself shall include the parameters for the MBS session distribution, e.g., Session Description parameters (which includes TMGI), user plane traffic flow parameters and so on. Therefore in the Nnef_MBSGroupMsgDelivery_Create Response, such parameters for MBS User Service Announcement need to be included so as to enable AF to providing such. It is proposed to update the related clauses to reflect the above-mentioned issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update procedures and services for group message delivery to include the parameters for MBS session distribution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the AF fetch the parameters |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, 9.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.5	MBS procedures for Group Message Delivery

7.5.1	Group Message Delivery via MBS Broadcast

Figure 7.5.1-1: Group Message Delivery via MBS Broadcast

NOTE 1:	The steps in this clause for group message delivery to the UE(s) follow the procedures specified in TS 26.502 [18], where the NEF acts as an MBS Application Provider.

1.	The AF sends Nnef_MBSGroupMsgDelivery_Create Request to the NEF. The Request message contains the Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time and External Group Identifier.

2.	The NEF checks authorization of the AF. If geographical area information or civic address information was provided by the AF as MBS service area, the NEF translates the MBS service area to Cell ID list or TAI list. The NEF may further check the MBS capability within the MBS service area.

NOTE 2:	The NEF is mandated for group message delivery.

3.	The NEF transforms the group message payload into a file, and determines the meta data information of the file (e.g. File URL, etc.). The NEF assigns a Group Message Correlation ID that identifies this group message delivery request.

If Application Service Provisioning specified in TS 26.502 [18] has not been performed, step 4 to step 8 are executed. Otherwise, they can be skipped.

4.	The NEF performs Application Service Provisioning by invoking Nmbsf_MBSUserService_Create and Nmbsf_MBSUserDataIngestSession_Create towards the MBSF using Object Distribution Method as specified in TS 26.502 [18].

5.	The MBSF performs MBS Session Creation as specified in clause 7.1.1.2 or clause 7.1.1.3.

6.	The MBSF performs Distribution Session Provisioning as specified in TS 26.502 [18]. The MBSF invokes Nmbstf_MBSDistributionSession_Create on the MBSTF, passing the parameters of the MBS Distribution Session received in step 4 to the MBSTF.

7.	The MB-SMF initiates the MBS Session Start for Broadcast procedure as specified in step 2 to step 9 in clause 7.3.1.

8.	If the MBSF performs the service announcement, it initiates the MBS User Service Announcement as specified in TS 26.502 [18]. The application may receive the appropriate information through the MBS-6 API from the MBS Client (see TS 26.502 [18]). The NEF may receive the service announcement information via Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation (see TS 26.502 [18]).

9.	The NEF sends Nnef_MBSGroupMsgDelivery_Create Response (Group Message Correlation ID, Acceptance Status, Cause) to the AF. The Acceptance Status indicates whether the group message delivery request is accepted or not. If not, the Cause is included indicating the appropriate failure reason. If the AF performs the service announcement, the NEF includes in the Group Message Response the service announcement information which contains information such as TMGI, Session Description parameters and the file meta data as specified in TS 26.502 [18] . The NEF may further include the area where MBS is not supported within the MBS service area in the Nnef_MBSGroupMsgDelivery_Create response to the AF.

If the AF knows the UEs that are located in the area where MBS is not supported, AF may use unicast to send the group message to those UEs.

10.	If the AF needs to perform the Service Announcement, the AF sends the application service announcement to the UE as specified in TS 26.502 [18].

11.	The NEF performs the User Data Ingestion towards the MBSTF as specified in TS 26.502 [18]. The NEF may push the file to the MBSTF or let MBSTF pull the file from the NEF.

12.	The MBSTF performs packetization and optionally FEC encoding as specified in TS 26.502 [18].

13.	The MBSTF delivers the packets to the MB-UPF to NG-RAN, and NG-RAN broadcast to the UE as specified in step 13 to step 15 in clause 7.3.1.

14.	Based on the service announcement information received in step 8 or step 10, when the UE receives the packets, the MBS client performs FEC decoding to restore the file and gets the group message from the file, as specified in TS 26.502 [18]. The MBS Client can expose the file towards the application in the UE using the MBS-7 API (see TS 26.502 [18]).

15.	The MBSTF notifies the MBSF of User Data distribution status, and the MBSF then notifies the MBSF of the User Data ingestion status as specific in TS 26.502 [18].

16.	The NEF sends Nnef_MBSGroupMsgDelivery_StatusNotify to the AF, containing Group Message Correlation ID, Delivery Status. The Delivery Status indicates whether delivery of Group Message Payload is successful or not. The delivery status information is received by the NEF via Nmbsf_MBSUserDataIngestSession_StatusNotify service operation (see TS 26.502 [18]).

# * * * * Second change * * * *

9.4.4.2	Nnef_MBSGroupMsgDelivery_Create service operation

Service operation name: Nnef_MBSGroupMsgDelivery_Create

Description: This service is used by the NF Service Consumer to request group message delivery.

Inputs, Required: Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time, External Group Identifier.

Inputs, Optional: None.

Outputs, Required: Group Message Correlation ID, Delivery Status, service announcement information.

Outputs, Optional: Cause, service area without MBS capability.

# * * * * End of changes * * * *