3GPP TSG-SA2 Meeting #156E	S2-2304338r01

April 17-21, 2023, Electronic meeting		                                      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4275 | Rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N3IWF selection for emergency services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | S2-2303930/C1-231181 asks clarifications about the UE selecting N3IWF for emergency service when it has SNPN credentials. According to the TR conclusion and current description, SA2 agreed that the UE shall select an N3IWF in a PLMN instead of SNPN for emergency sessions when the UE is not located in the same country as the subscribed SNPN. However, the current wording is a little misleading since there is no N3IWF selection procedure for emergency services for UE not equipped with valid SNPN credentials. - Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with valid SNPN credentials, in the same way as defined in clause 6.3.6.4.2 for a UE not equipped with a UICC. If the UE is not equipped with valid SNPN credentials, then the UE should be equipped with valid PLMN credentials or not be equipped with any credentials. In this case, the UE selects an N3IWF in a PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that in the above scenario, the UE follows the N3IWF selection procedure for Emergency services as if the UE is not equipped with valid SNPN credentials; Clarify the scenario when the UE is not equipped with valid SNPN credentials. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing descriptions may cause different understanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.30.2.12	Access to SNPN services via Untrusted non-3GPP access

Access to SNPN services via Untrusted non-3GPP access network follows the specification in the previous 5.30.2 clauses with the differences as specified in this clause.

N3IWF selection is supported as follows:

-	When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a.

Emergency services are supported as follows:

-	UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following:

-	If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection.

-	Otherwise, the UE follows the N3IWF selection procedure for Emergency services as if the UE is not equipped with valid SNPN credentials, in the same way as defined in clause 6.3.6.4.2 for a UE not equipped with a UICC.

UE onboarding is supported as follows:

-	When UE registers to SNPN over Untrusted non-3GPP access for UE Onboarding, UE may select an N3IWF in the SNPN which supports UE Onboarding by using a pre-configured N3IWF FQDN used for Onboarding.

-	If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without being connected to an N3WIF) with a PVS to obtain the SNPN credentials.

END CHANGE