3GPP TSG-SA2 Meeting #156E	S2-2304373

# 17th – 21th April, 2023, Elbonia													   revision of S2-xxxxxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4287 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN ID needs to be supported by 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe identifers for PIN. The 5G System already supports group identification and group membership management through External Group ID and Internal Group ID. A PIN also has exactly the same requirements that can be already fullfilled using this group management feature. It is proposed that a PIN is identified externally by a PIN ID. It is also proposed that External Group ID and Internal Group ID is used to identify and manage the PEGC UEs that are part of a PIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the PIN is identified and how the PEGCs that are part of a PIN are managed in 5GC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.44.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all text are new) * * * *

## 5.44	Support for Personal IoT Networks

### 5.44.x	PIN Identifiers

A PIN, pre-configured or created by an AF, is identified by a PIN ID. The PEGCs in a PIN may be managed dynamically using the 5GC group management feature. The AF for PIN uses the External Group ID to identiy the list of PEGCs in a PIN.

# * * * * End of change * * * *