SA WG2 Meeting #156E (e-meeting)	                                           S2-2304392r03

April 17 – 21, 2023									                                        (revision of S2-230XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4294 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Monitoring and 5GS information exposure update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is ENs unfixed: Editor's note: It is FFS whether all QoS monitoring requests can be addressed with the limitation of one QoS Monitoring control information per PCC. Further, the reported congested information used for exposure is also used for ECN marking for L4S in PSA UPF as defined in 5.37.4. However, the current reported congestion information is different. - Congestion information (i.e. a percentage of congestion level for exposure) - congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) However, the motivation of the RAN reported congestion information is to satisfy both ECN marking for L4S in PSA UPF and congestion information exposure. It is proposed to clarify the reported percentage is common for the two mechanisms, but the two mechanism PSA UPF can understand the percentage on its own. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fix the EN related to multiple QoS parameters are required to be measured for a given service data flow in 5.37.4. - QoS monitoring refinement in 5.45, including that - PCF calculate Round Trip delay calculation based on two QoS flows’ packet delay monitoring result, - PDV based on packet delay monitoring result, - Reported congestion information clarification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for for 5GS exposure, QoS monitoring is incomplete and sufficiently correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.3.3, , 5.45.1, 5.45.2, 5.45.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.45	QoS Monitoring

### 5.45.1	General

QoS monitoring comprises of measurements of QoS parameters for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF.

The AF may request for one or more of the following QoS parameters may trigger QoS monitoring for  service data flow(s):

-  UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-  Congestion, see clause 5.45.3.

-  Data Rate, see clause 5.45.4.

-  Packet Delay Variation, see clause 5.37.7.

- Round trip packet delay when UL and DL are on different QoS flows, see clause 5.37.4.

The PCF may generate the authorized QoS Monitoring policy for a service data flow based on the QoS Monitoring request received from the AF (as described in clause 6.1.3.21 of TS 23.503 [45]). The PCF includes the authorized QoS Monitoring policy in the PCC rule and provides it to the SMF.

The QoS parameter(s) to be measured by means of QoS monitoring follow. The QoS monitoring policy in PCC rule i(described in clause 6.3.1 of TS 23.503 [45]) may include the following:

-  UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-  Congestion, see clause 5.45.3.

-  Data Rate, see clause 5.45.4.

NOTE:     The AF requested QoS parameters to be measured and the PCF requested QoS parameters to be measured based on the AF request may be different.

The SMF configures the UPF to perform QoS monitoring for the QoS Flow and to report the monitoring results as described in clause 5.8.2.18 with parameters determined by the SMF based on the authorized QoS Monitoring policy received from the PCF and/or local configuration. The SMF may also configure RAN based on the authorized QoS Monitoring policy received from the PCF and/or local configuration.

The following clauses describe the QoS parameters which can be measured and any specific actions or constraints for their measurement.

NOTE:	The QoS parameter which can be measured are parameters which describe the QoS experienced in the 5GS by the application, i.e. they are not restricted to the 5G QoS Parameters defined in clause 5.7.2.

### 5.45.2	Packet delay monitoring

QoS Monitoring for packet delay allows for the measurement of UL packet delay, DL packet delay or round trip packet delay between UE and PSA UPF. The details of the QoS Monitoring for packet delay are described in clause 5.33.3.

NOTE:     The PCF may calculate Packet Delay Variation (clause 5.37.7) and the round trip packet delay when UL and DL are on different QoS flows (clause 5.37.4) based on packet delay monitoring results of QoS flows.

### 5.45.3	Congestion information monitoring

The NG-RAN may be required to provide UPF with the UL and/or DL QoS Flow congestion information ) based on configuration from the SMF. The UPF may be required to monitor the UL and/or DL QoS Flow congestion information reported from the NG-RAN based on a QoS monitoring configuration with congestion to be measured from the SMF.

The UPF reported congestion information is based on the information provided by NG-RAN, which is a percentage of packets that UPF would use for ECN marking for L4S in case of active ECN marking for L4S in PSA UPF (see clause 5.37.3). SMF may configure NG-RAN to report this information, which can be used both for ECN marking for L4S in PSA UPF (see clause 5.37.3.3) and for congestion information monitoring). For a given QoS flow with congestion to be measured, the PSA UPF reports congestion information as requested based on this information provided by NG-RAN

QoS monitoring request to the NG-RAN and NG-RAN reporting for UL and/or DL QoS Flow congestion information to PSA UPF is as defined in 5.37.3. The PSA UPF reports the received UL and/or DL QoS Flow congestion information to the target NF as instructed by the QoS Monitoring request from the SMF.

* * * Next Changes * * *

* * * Next Changes * * *

#### 5.37.3.3	Support of ECN marking for L4S in PSA UPF

To enable ECN marking for L4S by a PSA UPF, a QoS Flow level ECN marking for L4S indicator may be sent by SMF to PSA UPF over N4. SMF also configures NG-RAN to report the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF. This NG RAN reported information may be used for both, ECN marking for L4S in PSA UPF and for congestion information monitoring as defined in clause 5.45.3. If there is no UL packet when the report for DL and/or UL needs to be provided, NG-RAN may generate an UL Dummy GTP-U Packet for such a reporting.

Upon successful activation of congestion information reporting towards NG-RAN for UL and/or DL, PSA UPF uses information sent by NG-RAN in GTP-U header extension (see TS 38.415 [116] and TS 38.300 [27]) to perform ECN bits marking for L4S for the corresponding direction.

* * * End of Changes * * *