3GPP TSG-SA2 Meeting #156E	 S2-2304404r01

E-Meeting April 17 - 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4300 | Rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N3IWF selection for emergency services for UE not equipped with valid SNPN credentials |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 5.30.2.12 specifies that if UE is in visited country, UE follows “the N3IWF selection procedure for Emergency services for UE not equipped with valid SNPN credentials, in the same way as defined in clause 6.3.6.4.2 for a UE not equipped with a UICC” Clause 6.3.6.4.2 is used for UE to gain emergency services over PLMN untrusted non-3GPP access, and clause 5.30.2.12 is about UE accesses SNPN service. It causes different intereptation if the reference (6.3.6.4.2) mentioned by 5.30.2.12 is intended to let such UE (with SNPN subscription) to go for PLMN for emergency services(i.e. UE deactivates SNPN access mode) when in visited country, or is it intended to let such UE (with SNPN subscription) to perform similar procedure (but modified for SNPN scenario) as in 6.3.6.4.2 to access emergency services still in an SNPN. 6.3.6.4.2 states that “the UE may construct the Operator Identifier based N3IWF FQDN format based on a PLMN ID obtained via implementation specific means.” But if a UE that intends to access an SNPN should use an SNPN ID as to be able to find SNPNs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that similar procedure as 6.3.6.4.2 but modified is used for UE to use SNPN for emergency service, and if no SNPN availiable for emergency service over untrusted non-3GPP, then the UE falls back to use PLMN for emergency services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different intereptation of the spec, and the selection of SNPN would fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.30.2.12	Access to SNPN services via Untrusted non-3GPP access

Access to SNPN services via Untrusted non-3GPP access network follows the specification in the previous 5.30.2 clauses with the differences as specified in this clause.

N3IWF selection is supported as follows:

-	When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a.

Emergency services are supported as follows:

-	UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE in SNPN access mode performs the following:

-	If the UE determines that it is located in the same country as the configured N3IWF of the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection.

-	Otherwise, the UE performs a DNS query using the Visited Country Emergency N3IWF FQDN for SNPN, as specified in TS 23.003 [19] to determine which SNPNs in the visited country support emergency services in untrusted non-3GPP access via N3IWF; and:

Editor's note:	The Visited Country Emergency N3IWF FQDN for SNPN is assumed to take the form:

"sos.n3iwf.snpn-5gc.mcc<MCC>.visited-country.pub.3gppnetwork.org "

Once this FQDN has been added to TS 23.003, this Editor's note will be removed.

-	If the DNS response contains one or more records, the UE selects an SNPN that supports emergency services in untrusted non-3GPP access via N3IWF based on UE implementation specific methods. Each record in the DNS response shall contain the identity of an SNPN (i.e. SNPN ID) in the visited country supporting emergency services in untrusted non-3GPP access via N3IWF.

NOTE :	Self-assigned NIDs are not supported, since a DNS cannot be properly configured for multiple SNPNs using the same self-assigned NID (i.e. in collision scenarios).

-	Once the UE has selected an SNPN, the UE selects a N3IWF for the selected SNPN by constructing Operator Identifier based emergency SNPN N3IWF FQDN.

Editor's note:	The Operator Identifier based emergency SNPN N3IWF FQDN is assumed to take the form:

"sos.n3iwf.5gc.nid<NID>.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org"

Once this FQDN has been added to TS 23.003, this Editor's note will be removed.

- When an N3IWF has been selected, the UE initiates an Emergency Registration. If the Emergency Registration fails, the UE shall select another SNPN supporting emergency services in untrusted non-3GPP access.

- If the DNS response of the Visited Country Emergency N3IWF FQDN for SNPN does not contain any record, or if the DNS response contains one or more records but the UE fails to select an SNPN that supports emergency services in untrusted non-3GPP access, or if the Emergency Registration procedure has failed for all SNPNs supporting emergency services in untrusted non-3GPP access, the UE deactivates the SNPN access mode over NWu and attempts emergency services via PLMN untrusted non-3GPP access, by following the N3IWF selection procedure as defined in clause 6.3.6.4.2.

UE onboarding is supported as follows:

-	When UE registers to SNPN over Untrusted non-3GPP access for UE Onboarding, UE may select an N3IWF in the SNPN which supports UE Onboarding by using a pre-configured N3IWF FQDN used for Onboarding.

-	If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without being connected to an N3WIF) with a PVS to obtain the SNPN credentials.

END CHANGE