3GPP TSG-SA2 Meeting #156-e	S2-2304439

# Online,  17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0598 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rating untrusted AF data sources |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo Future Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 04-06-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces enhhancements for enabling rating of untrusted AF data sources based on the conclusions for KI#1 in the TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to introduce a procedure for enabling the NWDAF to rate untrusted AF data sources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.X, 6.2.X.1, 6.2.X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 6.2.x	Rating untrusted AF data sources

#### 6.2.X.1	General

When using an untrusted AF as data source, NWDAF may consider the data source rating results. The rating of untrusted AF is based on the quality of data collected.

Such rating can be triggered when the accuracy check based on the calculation between the predicted and ground-truth data indicates low perfromance, while the untrusted data source rating can be performed using a private logic, i.e., an NWDAF contained AnLF internal operation. In the selection of the appropriate data sources, the NWDAF containing AnLF can also use the rate of untrusted AF data sources as a criterion to calculate the expected confidence degree, i.e., that relates the outcome result with the input data sources.

#### 6.2.X.2	Procedure for rating untrusted AF data sources

The process of rating untrusted AF data sources is depicted in Figure 6.2.x.2. For realizing potential issues, the NWDAF containing AnLF subscribes to the NWDAF containing MTLF, which perfroms an accuracy calculation based on the predicted and ground-truth data or alternatively the NWDAF containing AnLF can calculate the accuracy locally by comparing the predicted and ground-truth data.

Figure 6.2.x.2: NWDAF containing AnLF-based unstrusted AF data source rating

1. NWDAF containing AnLF subscribes to NWDAF containing MTLF for obtaining an ML model using the Nnwdaf_ModelProvision_Subscribe service operation. The NWDAF containg AnLF may include a threshold (as described in 6.2E.2) to indicate when the NWDAF containing MTLF needs to execute the accuracy monitoring operations.

Option 1: Accuracy report from NWDAF containing MTLF

2a. NWDAF containing MTLF evaluates the ML model correctness as per 6.2E.2

2b. An accuracy report is sent to the NWDAF containing AnLF, e.g., when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

Option 2: NWDAF containing AnLF computes accuracy

2c. NWDAF containing AnLF calculates the accuracy by comparing the predictions with ground truth data.

2d. NWDAF containing AnLF is aware that the ML Model used has a low accuracy and determines that it needs to check further the data sources and compute data source rating. The decision conditions upon which it needs to initiate data source rating for a data source is based on NWDAF containing AnLF implementation.

3a-3b.  NWDAF containing AnLF initiates rating of a data source by requesting and receiving supplementary data, i.e., via Nnwdaf_DataManagement_Fetch / Ndccf_DataManagement_Notify, from different data sources (if available) to verify the data source quality or correctness. Such data can be for example performance data from the OAM which are supplementary to the data from untrusted AFs, or data from UPF supplementary to the data from untrusted AFs.

4.  NWDAF containing AnLF updates the rating for the sources where untrusted data is deviated from the supplementary trusted data per Event ID, or in case step 3 is not implemented the rating is based on the accuracy reporting in 2b.

NOTE: An NWDAF containing AnLF determines the rating of an untrusted AF data source based on internal operations.

The NWDAF containing AnLF stores the untrusted AF data source rating locally or in the UDSF or in both.

5a. NWDAF containing AnLF stores the untrusted AF data source rating locally.

5b. NWDAF containing AnLF sends the untrusted AF data source rating into the UDSF for storage into an external storage, if available. In this release, only UDSF is considered as external storage for the data source rating results. NWDAF containing AnLF uses the Nudsf_ UnstructuredDataManagement_Create service operation.

NOTE: To avoid an untrusted AF to be permanently excluded as a data source the NWDAF containing AnLF can: i) introduce a timer option to delete it or ii) delete it explicitly using the Nudsf_ UnstructuredDataManagement_Delete service operation once the related Event ID is completed.

6. A NWDAF consumer subscribes to a certain Analytics ID = “xx”, using Nnwdaf_AnalyticsSubscription_Subsribe service operation.

The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources required either locally step 7a or from the UDSF step 7b, before collecting the data needed for the subscribed Analytic ID.

7a. The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources locally

7b. The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources from the UDSF using the to use Nudsf_ UnstructuredDataManagement_Query service operation.

8. If the rating of one or more data sources is below a threshold (i.e., that is pre-configured), then the NWDAF containing AnLF can: (i) select an alternative data source with higher rating or (ii) request supplementary data from other trusted data sources.

9.  The NWDAF containing AnLF subscribes to a new data source to receive alternative or supplementary data if a new data source is selected in step 8. For subscribing to data from DCCF, the NWDAF containing AnLF needs to use Ndccf_DataManagement_Subscribe service operation while the DCCF uses the Ndccf_DataManagement_Notify service operation to provide the new data. Alternatively, subscribing to data from other 5G core data sources the Nnf_EventExposure_Subscribe / Nnf_EventExposure_Notify service operations are used.

NOTE: Data from other 5G core data source can also be collected, e.g., from ADRF, using the respective operation services. Data from the OAM can also be collected using the respective 3GPP SA5 data exposure operations.

10. The NWDAF containing AnLF obtains the data and the corresponding data source rating and determines the impact on the confidence level of the respective analytics output.

11.  The NWDAF containing AnLF provides the analytics output to the analytics consumer, using the Nnwdaf_AnalyticsSubscription_Notify service operation.

* * * * End of Change * * * *