3GPP TSG-SA2 Meeting #156-e	S2-2304439r03

# Online,  17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0598 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rating untrusted AF data sources |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo Future Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 04-06-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces enhhancements for enabling rating of untrusted AF data sources based on the conclusions for KI#1 in the TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to introduce a procedure for enabling the NWDAF to rate untrusted AF data sources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.X, 6.2.X.1, 6.2.X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 6.2.x	Rating untrusted AF data sources

#### 6.2.X.1	General

When using an untrusted AF as data source, NWDAF may consider the data source rating results. The rating of untrusted AF is based on the quality of data collected.

Such rating may be triggered when the accuracy check based on the calculation between the predicted and ground-truth data indicates low perfromance, while the untrusted data source rating may be performed based onNWDAF  internal logic. In the selection of the appropriate data sources, the NWDAF  may use the rate of untrusted AF data sources as a criterion to calculate the expected confidence degree, i.e., that relates the outcome result with the input data sources.

#### 6.2.X.2	Procedure for rating untrusted AF data sources

The process of rating untrusted AF data sources is depicted in Figure 6.2.x.2. For realizing potential issues, the NWDAF containing AnLF subscribes to the NWDAF containing MTLF, which perfroms an accuracy calculation based on the predicted and ground-truth data or alternatively the NWDAF containing AnLF can calculate the accuracy locally by comparing the predicted and ground-truth data.

Figure 6.2.x.2: NWDAF containing AnLF-based unstrusted AF data source rating

1. NWDAF containing AnLF subscribes to NWDAF containing MTLF for obtaining an ML model using the Nnwdaf_ModelProvision_Subscribe service operation. The NWDAF containg AnLF may include a threshold (as described in 6.2E.2) to indicate when the NWDAF containing MTLF needs to execute the accuracy monitoring operations.

Option 1: Accuracy report from NWDAF containing MTLF

2a. NWDAF containing MTLF evaluates the ML model correctness as per 6.2E.2

2b. An accuracy report is sent to the NWDAF containing AnLF, e.g., when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

Option 2: NWDAF containing AnLF computes accuracy

2c. NWDAF containing AnLF calculates the accuracy by comparing the predictions with ground truth data.

2d. NWDAF containing AnLF is aware that the ML Model used has a low accuracy either by receiving the accuracy report in step 2b or monitoring the accuracy by itself in step 2c. NWDAF containing AnLF determines that it needs to check further the data sources and compute data source rating. The decision conditions upon which it needs to initiate data source rating for a data source is based on NWDAF containing AnLF implementation.

3a-3b.  NWDAF containing AnLF initiates rating of a data source by requesting and receiving supplementary data, i.e., via Nnwdaf_DataManagement_Fetch / Ndccf_DataManagement_Notify, from different data sources (if available) to verify the data source quality or correctness. Such data can be for example performance data from the OAM which are supplementary to the data from untrusted AFs, or data from UPF supplementary to the data from untrusted AFs.

4.  NWDAF containing AnLF updates the rating for the sources where untrusted data is deviated from the supplementary trusted data per Event ID.

NOTE: An NWDAF containing AnLF determines the rating of an untrusted AF data source based on internal operations.

5a. NWDAF containing AnLF stores the untrusted AF data source rating locally.

5b. NWDAF containing AnLF may send the untrusted AF data source rating into the UDSF for storage into an external storage, if available. In this release, only UDSF is considered as external storage for the data source rating results. NWDAF containing AnLF uses the Nudsf_ UnstructuredDataManagement_Create service operation.

NOTE: To avoid an untrusted AF to be permanently excluded as a data source, the NWDAF containing AnLF can re-rate the untrusted AF based on its internal logic. For example, it can rate the untrusted AF after some timer expired.

6. A NWDAF consumer subscribes to a certain Analytics ID = “xx”, using Nnwdaf_AnalyticsSubscription_Subsribe service operation.

The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources required either locally step 7a or from the UDSF step 7b, before collecting the data needed for the subscribed Analytic ID.

7a. The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources locally

7b. The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources from the UDSF using the to use Nudsf_ UnstructuredDataManagement_Query service operation.

8. If the rating of one or more untrusted AF are below a threshold (i.e., that is pre-configured), then the NWDAF containing AnLF can: (i) select an alternative untrusted AF (if available) with higher rating or (ii) request supplementary data from other trusted data sources.

9.  The NWDAF containing AnLF subscribes to a new data source to receive alternative or supplementary data if a new data source is selected in step 8.

10. The NWDAF may use the rate of untrusted AF data sources as a criterion to calculate  the confidence level of the respective analytics output.

11.  The NWDAF containing AnLF provides the analytics output to the analytics consumer, using the Nnwdaf_AnalyticsSubscription_Notify service operation.

* * * * End of Change * * * *