SA WG2 Meeting #156e	S2-2304475

# Elbonia, April 17th – 21st, 2023               	  	       		 (revision of S2-230)

Title:	[draft] LS on Wi-Fi Alliance communication on methods for consistent QoS across 5G and Wi-Fi networks

Response to:	S2-2303915

Release:

Work Item:

Source:	Nokia, Will be SA2

To:	WI-FI Alliance

Cc:

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks WI-FI Alliance for your LS. SA2 have discussed it and would like to provide following answer:

- About your item “1) Consideration of DSCP markings at UPF and UE”

Current 3GPP specifications allow the Session Manager (SMF) to control User Plane Function (UPF) classification as follows: The traffic classification filters provided to the UPF for downlink traffic (in PDR) and to the UE for uplink traffic (QoS rules) and to gNB (in QoS profile) may include the DSCP along with other packet filtering criteria. The QoS provided (PDR, QoS rule and QoS profile) provide the map between the traffic matching traffic filter  to the associated QoS parameter, e.g. 5QI (the resulting matching is name QoS flow) service data flows to 5G QoS flows with desired 5Qis. This may be controlled by PCF Policy Control policies policies which provides to SMF the traffic filter and the associated QoS to be applied. The 3GPP specifications allows the PLMN to associate any DSCP to any 5QI based on its consideration and needs, as  considered suitable

- About your item 2) “QoS Mapping at CPEs “

In the scenario of N3IWF/TNGF/TWIF the mapping of DL traffic from CPE towards the device connected behind and the UL traffic from CPE to UPF is currently under specification in Rel-18. For more details refer to TS 23.316 from version 18.2.0 onward

The mapping of traffic in previous releases for the scenario of wireline access where the CPE corresponds to the 5G-RG is specified in TS 23.316. In other scenario previous to release 18 the mapping between 3GPP QoS and DSCP marking when a UE supports WLAN interface is left to implementation and configuration and considered outside 3GPP scope.

2. Actions:

To WI-FI Alliance:

ACTION: 	SA2 kindly asks WI-FI Alliance to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#157 | May 22-26th, 2023 | Berlin | Germany |
| SA2#158 | August 22-26th, 2023 | Gothenburg | Sweden |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/