SA WG2 Meeting #156e	S2-2304475r03

# Elbonia, April 17th – 21st, 2023               	  	       		 (revision of S2-230)

Title:	[draft] LS on Wi-Fi Alliance communication on methods for consistent QoS across 5G and Wi-Fi networks

Response to:	S2-2303915

Release:

Work Item:

Source:	Nokia, Will be SA2

To:	WI-FI Alliance

Cc:

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks WI-FI Alliance for your LS. SA2 have discussed it and would like to provide following answer:

- About your item “1) Consideration of DSCP markings at UPF and UE”

Current 3GPP specifications allow the Session Manager (SMF) to control User Plane Function (UPF) classification as follows: The traffic classification filters provided to the UPF for downlink traffic (in PDR) and to the UE for uplink traffic (QoS rules) may include the DSCP along with other packet filtering criteria. The QoS provided (PDR, QoS rule) provide the mapping between the traffic matching traffic filter and the associated QoS parameter, e.g. 5QI (the resulting matching is named QoS flow) i.e. map service data flows to 5G QoS flows with desired 5QIs. This may be controlled by PCF Policy Control policies which provide to SMF the traffic filter and the associated QoS to be applied. The 3GPP specifications allows the PLMN to associate any DSCP to any 5QI based on its consideration and needs,

- About your item 2) “QoS Mapping at CPEs “

3GPP is currently working on following feature for its R18: when the SMF provides the 5G-RG with QoS flow descriptions, the SMF may additionally signal Non-3GPP QoS Assistance Information (N3QAI) for each QoS flow to the 5G-RG. Based on the N3QAI together with QoS rule information, the 5G-RG may reserve resources in the non-3GPP network. N3QAI consists of the following QoS information: QoS characteristics, GFBR/MFBR, Maximum Packet Loss Rate, ARP and Periodicity (if available at the SMF).

the mapping between 3GPP QoS and DSCP marking at the WLAN interface is left to implementation and configuration and considered outside 3GPP scope.

2. Actions:

To WI-FI Alliance:

ACTION: 	SA2 kindly asks WI-FI Alliance to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#157 | May 22-26th, 2023 | Berlin | Germany |
| SA2#158 | August 22-26th, 2023 | Gothenburg | Sweden |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/