3GPP TSG-WG SA2 Meeting #156E	S2-2304479r06

# E-meeting, April 17–21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4322 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolving for KI#8 Except TSCAC and clarifications on AF inputs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.37.8 of TS 23.501, there is one statement saying AF provide peridicty if it is trusted. However, being trusted or not is not the direct reason regarding to whether AF provide periodicity or not. This should be clarified. Also, there is one EN about UPF to SMF reporting as follows which can be removed by adding a N4 session level report procedure in TS 23.502. Editor's note: The method used to report the Periodicity information from the UPF to the SMF is FFS. Another EN about new RTP header as follows which should not be an EN and should be resovled Editor’s note: Support of new RTP header extension for End of Data Burst indication by the application server depends on progress in SA WG4 5G_RTP WI. In addition, some editorial changes on N6 jitter are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removing an EN about UPF reporting proceudre to SMF and make corresponding changes in 5.8.5 and 5.37.8 - Resolve the EN and add reference - Some editorial changes to N6 jitter/jitter, NG RAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN not resolved and some conceptual confusions remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.8.5.11, 5.8.5.12, 5.37.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	Void.

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	3GPP TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[73]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[74]	IETF RFC 3162: "RADIUS and IPv6".

[75]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[76]	3GPP TS 26.238: "Uplink streaming".

[77]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156"; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[80]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[81]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[82]	IETF RFC 8803: "0-RTT TCP Convert Protocol".

[83]	IEEE Std 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[84]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[86]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[87]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[88]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[89]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[90]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[91]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[92]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[93]	BBF TR-456 issue 2: "AGF Functional Requirements".

[94]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The reference to BBF WT-457 will be revised when finalized by BBF.

[95]	IEEE Std 802.1Qcc-2018: "IEEE Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[96]	Void.

[97]	IEEE Std 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[98]	IEEE Std 802.1Q-2018: "IEEE Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[99]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[100]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[101]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[102]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; stage 2".

[103]	3GPP TS 29.563: "5G System (5GS); HSS services for interworking with UDM; Stage 3".

[104]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[105]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[106]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[107]	IEEE Std 1588-2008: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[108]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[109]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[110]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[111]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[112]	3GPP TR 38.824: "Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)".

[113]	IEEE: "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

[115]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[116]	3GPP TS 38.415: "PDU Session User Plane Protocol".

[117]	3GPP TS 24.535: "Device-side Time-Sensitive Networking (TSN) Translator (DS-TT) to network-side TSN Translator (NW-TT) protocol aspects; Stage 3".

[118]	3GPP TS 32.274: "Charging Management; Short Message Service (SMS) charging".

[119]	3GPP TS 23.008: "Organization of subscriber data".

[120]	3GPP TS 38.314: "NR; Layer 2 measurements".

[121]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[122]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[123]	3GPP TS 32.254: "Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[124]	3GPP TS 33.535: "Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (5GS)".

[125]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[126]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[127]	ST 2059-2:2015: "SMPTE Standard - SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications".

[128]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[129]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[130]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[131]	IEEE Std 802.3: "Ethernet".

[132]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[133]	3GPP TS 29.513: "Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[134]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[135]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[136]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[137]	GSMA NG.116: "Generic Network Slice Template".

[138]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[139]	3GPP TS 24.539: "5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3".

[140]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[141]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[142]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

[143]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[144]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[145]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

[146]	IEEE Std P802.1Qdj-d0.3: "IEEE Draft Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment XX: Configuration Enhancements for Time-Sensitive Networking".

[147]	3GPP TS 23.700-05: "Study on architecture enhancements for vehicle-mounted relays".

[TS26522]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

# * * * * Next change* * * *

5.8.5.11	Session Reporting Rule

The following table describes the Session Reporting Rule (SRR) that defines the detection and reporting events that the UPF shall report, that are not related to specific PDRs of the PDU Session, as follows:

-	Per QoS Flow per UE QoS Monitoring Report, as specified in clause 5.33.3.2.

-	Change of 3GPP or non-3GPP access availability, for an MA PDU session.

-	Per QoS Flow Traffic Parameter Measurement Report.

Table 5.8.5.11-1: Attributes within Session Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this SRR. |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting. |
| QoS Monitoring per QoS Flow Control Information | Indicates the UPF to apply perform the QoS Monitoring report for one or more QoS Flows. | The IE is defined in clause 7.5.2.9 of the TS 29.244 [65]. See NOTE 1. |
| Access Availability Control Information | Indicates the UPF to report when an access type becomes available or unavailable for an MA PDU Session. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. |
| Traffic Parameter Measurement Control Information | Indicates the UPF to report Traffic parameter for one QoS Flow, including UL/DL periodicity and/or N6 jitter information associated with a DL Periodicity in downlink direction. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. |
| NOTE 1: The QoS Monitoring per QoS Flow Control Information may contain an Indication of local direct event notification and. The Indication of local event notification includes a Notification Target Address (the details are described in clause 5.8.2.18) that identifies the recipient of the information being notified by the UPF (Local NEF/AF). The Indication of local direct event notification also indicates that the UPF reports the information to the NF indicated by the Target of reporting via Nupf_EventExposure_Notify service operation. |  |  |

# * * * * Next change * * * *

5.8.5.12	Session reporting generated by UPF

The UPF sends the session report to inform the SMF the detected events for a PDU Session that are related to an SRR. The UPF may support notification to the AF possibly via local NEF as described in clause 6.4 of TS 23.548 [130].

Table 5.8.5.12-1: Attributes within Session Reporting

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to the SRR which triggered the report. |  |
| Rule ID | Unique identifier to identify the Session Reporting Rule within a session which triggered the report. | Used by UPF when reporting. |
| QoS Monitoring Report | Indicates the QoS Monitoring result for one or more QoS Flows. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |
| Access Availability Report | Indicates the change of 3GPP or non-3GPP access availability, for an MA PDU session. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |
| Traffic Parameter Measurement Report | Indicates the Traffic Parameter measurement result for one QoS Flow, including the UL/DL periodicity and/or N6 jitter for a DL Periodicity. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |

# * * * * Next change * * * *

5.37.8	UE power saving management

5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

5.37.8.2	Periodicity and N6 Jitter Information associated with Periodicity

At AF Session Establishment with QoS, the 5G System may be provided with Periodicity information for NG-RAN to configure UE power management for connected mode DRX. The Periodicity information is provided by the AF to the PCF via NEF or directly to the PCF when the AF is trusted.

Editor's note:	The method used to provide the Periodicity information to SMF is FFS.

The PCF should send the Periodicity information received from the AF and includes an indication for SMF to request UPF to perform Traffic Parameter(s) measurement (i.e. UL/DL periodicity, if not provided by the AF, and/or DL periodicity associated N6 jitter range) within PCC Rules, in accordance with the operator's local policies.

Upon reception of PCC rule, the SMF determines the TSCAI and forwards it to the NG-RAN. If the PCC rule indicates to perform N6 jitter measurements, the SMF also sends the DL Periodicity to the UPF if available. The SMF requests the UPF to monitor and periodically report the Traffic Parameter(s) (i.e. UL/DL periodicity, if not provided by the AF, and/or the N6 jitter range associated with the DL Periodicity) using the N4 Session Modification procedure, see clause 5.8.5.11. The UPF reports the measured Traffic Parameter(s)  to SMF via N4 interface.

NOTE 1:	How the UPF derives the N6 jitter is implementation dependent.

NOTE 2:	How the UPF derives periodicity when not provided by the AF is implementation dependent.

At reception of  measured Traffic Parameter(s) from the UPF in the N4 Session Level Report, the SMF includes the N6 jitter range together with the associated DL periodicity in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the N6 jitter is larger than a threshold.

5.37.8.3	End of Data Burst Indication

An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX.

The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.

SMF should request the UPF to detect the last PDU of the data burst and the mark the End of Data burst in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies.

According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the protocol header as defined in Protocol Description or UPF implementation and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst (see TS 26.522 [TS26522]).

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

# * * * * End of changes * * * *