3GPP TSG-WG SA2 Meeting #156-e 	S2-2304510

Elbonia, April 17 – 21, 2023	(revision of S2-2202655)

Source:	Huawei, HiSilicon

Title:	Procedure of UE Positioning using Sidelink Positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the Procedure of UE Positioning using Sidelink Positioning based on the KI#5 conclusion in TR 23.700-86.

1. Introduction/Discussion

This paper proposes to add the Procedure of UE Positioning using Sidelink Positioning based on the KI#5 conclusion in TR 23.700-86.

Based on the following description in TS 38.305, the used term "Network assisted Sidelink Positioning" is not correct since the LMF can make the positioning calculation in the TR conclusion. This paper proposes to use "UE Positioning using Sidelink Positioning".

The suffixes "-based" and "-assisted" refer respectively to the node that is responsible for making the positioning calculation (and which may also provide measurements) and a node that provides measurements (but which does not make the positioning calculation).

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change (all new)* * * *

### 6.X	Procedure of standalone Sidelink Positioning for Target UE

This procedure is used to acquire absolute location of Target UE using Located UE, where both of the Target UE and Located UE are out of coverage. The Located UE is assumed to already know its absolution location.

Procedures for Ranging/Sidelink Positioning control (UE-only operation) defined in clause 6.7 applies with the following adaptations:

- UE1 is the Target UE, and UE2 is the Located UE

- Step1, the service request from Target UE application layer indicates that absolute location of the Target UE is required

- Step 7, absolute location of the Target UE is calculated.

# * * * * End of changes * * * *