3GPP TSG-WG SA2 Meeting #156-e	S2-2304523

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0977 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URSP handling for legacy UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, LG Electronics, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As discussed last meeting, URSP can be used for energy efficiency, i.e. to save the signalling exchange to update the Configure NSSAI for legacy UEs. In details, it is noticed that there is validation criteria in the URSP rule and the UE is able to make its own decision based on the application layer/OS Layer (traffic status) and release the PDU Sessions by itself. In details, the URSP rule can fulfil the above requirement as described in TS 23.503: - The UE will not establish new PDU Sessions if the UE is out of Time Window or Location Criteria. The Route Selection Descriptor of a URSP rule shall be only considered valid if all of the following conditions are fulfilled: - If a Time Window is present and the time matches what is indicated in the Time Window. - If a Location Criteria is present and the UE location matches what is indicated in the Location Criteria. The Route Selection Descriptor of a URSP rule shall be only considered valid if all of the following conditions are fulfilled: - If a Time Window is present and the time matches what is indicated in the Time Window. - If a Location Criteria is present and the UE location matches what is indicated in the Location Criteria. The UE mayrelease the existing PDU Session in a timely manner if the UE is out of Time Window or Location Criteria. When URSP rules are updated or their validity according to the conditions above change, the association of existing applications to PDU Sessions may need to be re-evaluated. The UE may also re-evaluate the application to PDU Session association due to the following reasons: - periodic re-evaluation based on UE implementation; - an existing PDU Session that is used for routing traffic of an application based on a URSP rule is released; - The expiration of Time Window in Route Selection Validation Criteria, i.e. the expiration of Time Window, or UE's location no longer matches the Location Criteria. If the re-evaluation leads to a change of the application to PDU Session association, e.g. the application is to be associated with another PDU Session or a new PDU Session needs to be established, the UE may enforce such changes in a timely manner based on implementation, e.g. immediately or when UE enters CM-IDLE state. When URSP rules are updated or their validity according to the conditions above change, the association of existing applications to PDU Sessions may need to be re-evaluated. The UE may also re-evaluate the application to PDU Session association due to the following reasons: - periodic re-evaluation based on UE implementation; - an existing PDU Session that is used for routing traffic of an application based on a URSP rule is released; - The expiration of Time Window in Route Selection Validation Criteria, i.e. the expiration of Time Window, or UE's location no longer matches the Location Criteria. If the re-evaluation leads to a change of the application to PDU Session association, e.g. the application is to be associated with another PDU Session or a new PDU Session needs to be established, the UE may enforce such changes in a timely manner based on implementation, e.g. immediately or when UE enters CM-IDLE state. It is proposed to add related description on URSP for legacy UEs.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is porposed to add a NOTE that If the validity time as described in TS 23.501 [2] clause 5.15.6 and/or location availability information as described in TS 23.501 [2] clause 5.15.8 is available for one S-NSSAI in the PCF then such information can be used by the PCF to generate Time Window and/or Location Criteria of the URSP rule.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Temporary S-NSSAI for legacy UEs is not efficient. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.1.3	Policy control subscription information management

The PCF may request subscription information at PDU Session establishment, PDU Session modification, during AM Policy Association Establishment procedure and during the UE Policy Association Establishment procedure.

The PCF may receive notifications on changes in the subscription information. Upon reception of a notification, the PCF shall make the policy control decisions necessary to accommodate the change in the subscription and shall update the SMF and/or the AMF if needed.

NOTE 1:	How the PCF provisions/retrieves information related with policy control subscription data is defined in TS 23.501 [2].

The policy control subscription profile information provided by the UDR during the UE Policy Association Establishment procedure using Nudr service for Data Set "Policy Data" and Data Subset "UE context policy control data" is described in Table 6.2-1:

Table 6.2-1: UE context policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |
| Tracing Requirements | Tracing requirements as defined in TS 32.421 [18] | Optional |
| PEI | The Permanent Equipment Identifier of the UE. | Optional |
| OSId | Identifies the operating system supported by UE. | Optional |
| Indication of UE support for ANDSP | Indicates the UE support for ANDSP. | Optional |
| Indication of URSP Provisioning Support in EPS | Indicates the UE support for URSP Provisioning in EPS. | Optional |
| S-NSSAI subscription information | Contains the list of subscribed S-NSSAIs, its associated subscribed DNNs. For each DNN, it includes the Allowed PDU Session types, the Allowed SSC modes and the ATSSS information (NOTE 1) (NOTE 2). | Optional |
| Subscriber spending limits control | Indicates whether the PCF must enforce UE policies based on subscriber spending limits. | Optional |
| NOTE 1: ATSSS information is defined in TS 23.502 [3] Table 5.2.3.3.1-1 and Indicates whether MA PDU Session establishment is allowed. NOTE 2: If the validity time as described in TS 23.501 [2] clause 5.15.6 and/or location availability information as described in TS 23.501 [2] clause 5.15.8 is available for one S-NSSAI in the PCF then such information can be used by the PCF to generate Time Window and/or Location Criteria of the URSP rule. |  |  |

Editor's note:	It is FFS whether CHF address may be added as an optional parameter to Table 6.2-1.

NOTE 2:	S-NSSAI subscription information can be part of UE context policy control subscription information and Session Management Subscription data/Slice Selection Subscription data. UDR implementation and the provisioning system are responsible for keeping the consistency of this information when both Data Sets are stored in the same UDR. The provisioning system is responsible for keeping the consistency of this information when both Data Sets are stored in different UDRs.

The policy control subscription profile information provided by the UDR at PDU Session establishment, using Nudr service for Data Set "Policy Data" and Data Subset "PDU Session policy control data" is described in Table 6.2-2.

Table 6.2-2: PDU Session policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Allowed services | List of subscriber's allowed service identifiers | Optional |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |
| Subscribed GBR | Maximum aggregate bitrate that can be provided across all GBR QoS Flows for the DNN and S-NSSAI. | Optional |
| ADC support | Indicates whether application detection and control can be enabled for a subscriber | Optional |
| Subscriber spending limits control | Indicates whether the PCF must enforce session management related policies based on subscriber spending limits | Optional |
| IP index information | Information that identifies the IP Address allocation method during PDU Session establishment | Optional |
| Background Data Transfer Reference ID(s) | Reference ID(s) for Background Data Transfer Policies that apply to the UE. | Optional |
| Local routing indication | Indication on whether AF influence on traffic routing is allowed or not allowed | Optional |
| Subscribed UE-Slice-MBR | Subscribed UE-Slice-MBR value per S-NSSAI | Conditional (NOTE 2) |
| Charging related information | This part defines the charging related information in the policy control subscription profile |  |
| Default charging method | Default charging method for the PDU Session (online / offline) | Optional |
| CHF address | The address of the Charging Function and optionally the associated CHF instance ID and CHF set ID (see clause 6.3.1.0 of TS 23.501 [2]) | Optional |
| Usage monitoring related information | This part includes a list of usage monitoring profiles associated with the subscriber. Each usage monitoring profile is logically associated with a particular operator offer, and includes the following elements |  |
| Monitoring key | An identifier to a usage monitoring control instance that includes one or more PCC rules | Conditional (NOTE 1) |
| Usage monitoring level | Indicates the scope of the usage monitoring instance (PDU Session level or per Service) | Optional |
| Start date | Start date and time when the usage monitoring profile applies | Optional |
| End date | End date and time when the usage monitoring profile applies | Optional |
| Volume limit | Maximum allowed traffic volume | Optional |
| Time limit | Maximum allowed resource time usage | Optional |
| Reset period | Time period to reset the remaining allowed consumed usage for periodic usage monitoring control (postpaid subscriptions) | Optional |
| MPS subscription data | This part defines the MPS subscription information in the policy control subscription profile |  |
| MPS priority | Indicates subscription to MPS priority service; priority applies to all traffic on the PDU Session | Conditional (NOTE 1) |
| IMS signalling priority | Indicates subscription to IMS signalling priority service; priority only applies to IMS signalling traffic | Conditional (NOTE 1) |
| MPS priority level | Relative priority level for multimedia priority services | Conditional (NOTE 1) |
| MCS priority | Indicates subscription to MCS priority service; priority applies to all traffic on the PDU Session | Conditional (NOTE 1) |
| MCS priority level | Relative priority level for MCS services | Conditional (NOTE 1) |
| NOTE 1: The information is mandatory if the specific part is included in the subscription information (e.g. the monitoring key is mandatory if the usage monitoring information part is included). NOTE 2: The information is used in PCF as described in clause 6.2.1.9 when the monitoring of the UE-Slice-MBR for an S-NSSAI is performed at the PCF. There may be a UE-Slice-MBR value for each S-NSSAI, if applicable. |  |  |

NOTE 3:	Subscribed UE-Slice-MBR can be part of the Access and Mobility Subscription data as described in clause 5.2.3.3.1 of TS 23.502 [3] and can be part of the PDU Session policy control subscription information as described in Table 6.2-2. UDR implementation and the provisioning system are responsible for keeping the consistency of this information when both Data Sets are stored in the same UDR. The provisioning system is responsible for keeping the consistency of this information when both Data Sets are stored in different UDRs.

Table 6.2-3: Remaining allowed usage subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Remaining allowed usage related information | This part includes a list of Remaining allowed usage associated with the subscriber. |  |
| Monitoring key | An identifier to a usage monitoring control included one or more PCC rules | Conditional (NOTE 1) |
| Usage monitoring level | Iindicates the scope of the usage monitoring (PDU Session level or service level) | Optional |
| Volume usage | Remaining allowed traffic volume | Optional |
| Time usage | Remaining allowed resource time usage | Optional |
| NOTE 1: The information is mandatory if the specific part is included in the subscription information (e.g. the monitoring key is mandatory if the usage monitoring information part is included). |  |  |

The Allowed services may comprise any number of service identifiers allowed for the subscriber in the PDU Session. The PCF maps those service identifiers into PCC rules according to local configuration and operator policies.

The Subscriber category may comprise any number of identifiers associated with the subscriber (e.g. gold, silver, etc.). Each identifier associates operator defined policies to the subscriber that belong to that category.

The Usage monitoring related information may comprise any number of usage monitoring control instances associated with the subscriber. In each usage monitoring control instance is mandatory to include the Monitoring key. The Reset period only applies to usage monitoring control instances that periodically reset the allowed usage (e.g. daily, monthly, etc.). If the Reset period is not specified, the usage monitoring control instance ends when the allowed data is consumed or when the End date is reached. The usage monitoring related information is used by the PCF instead of the respective information for the subscriber category.

The policy subscription profile may be extended with operator-specific information. Operator-specific extensions may be added both to any specific part of the policy control subscription information (e.g. to the subscriber category part), or as a new optional information block.

Handling of operator specific policy data by the PCF is out of scope of this specification in this release.

The latest list of PSIs and its content delivered to the UE provided by the UDR during the UE Policy Association Establishment procedure using Nudr service for Data Set "Policy Data" and Data Subset "Policy Set Entry" is described in Table 6.2-4.

Table 6.2-4: Policy Set Entry

| Information name | Description | Category |
| --- | --- | --- |
| Policy Set Entry | List of PSIs and content for each PSI. Content may be Access Network Discovery & Selection Policy Information or UE Route Selection Policy information or both. | Optional |

The network slice specific policy control information is per S-NSSAI information stored by the UDR and updated by the PCF during PDU Session Establishment or Modification procedure using Nudr service for Data Set "Policy Data" and Data Subset "Network Slice Specific Control Data" is described in Table 6.2-5:

Table 6.2-5: Network slice specific policy control information

| Information name | Description | Category |
| --- | --- | --- |
| Maximum Slice Data Rate for UL (per S-NSSAI) | The maximum uplink data rate for the specific network slice | Optional (NOTE 2) |
| Maximum Slice Data Rate for DL (per S-NSSAI) | The maximum downlink data rate for the specific network slice | Optional (NOTE 2) |
| Remaining Maximum Slice Data Rate for UL (per S-NSSAI) | The remaining maximum uplink data rate for the specific network slice (NOTE 1). | Optional (NOTE 3) |
| Remaining Maximum Slice Data Rate for DL (per S-NSSAI) | The remaining maximum downlink data rate limited for a specific network slice (NOTE 1). | Optional (NOTE 3) |
| NOTE 1: The initial value is set to the Maximum Slice Data Rate for UL/DL value. NOTE 2: The information is only used for limitation of data rate per network slice with assistance of the NWDAF. NOTE 3: The information is only used for limitation of data rate per network slice with PCF based monitoring. |  |  |

The policy control subscription profile information is per SUPI information, provided by the UDR using Nudr service for Data Set "Policy Data" and Data Subset " Access and Mobility policy control data" is described in Table 6.2-6:

Table 6.2-6: Access and Mobility policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Subscriber spending limits control | Indicates whether the PCF must enforce Access and Mobility management related policies based on subscriber spending limits. | Optional |

Editor's note:	It is FFS whether CHF address may be added as an optional parameter to Table 6.2-6.

# * * * * End of changes * * * *