3GPP TSG-WG SA2 Meeting #156-e	S2-2304549

Elbonia, April 17th – 21st 2023	(revision of S2-xxxxxx)

Source:	Samsung

Title:	Procedure for service exposure to AF

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This contribution specifies ranging and sidelink positioning service exposure to AF.

1. Introduction/Discussion

This contribution specifies ranging and sidelink positioning service exposure to AF.

2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change * * * *

5.6.3	Service Exposure to the AF

The 5GS Ranging/SL Positioning service may be exposed to the AF either via direct interaction with the GMLC or via the NEF for an AF in untrusted domain. Thus, the ranging AF may contact GMLC directly or via NEF to provide a ranging or sidelink positioning operation request to the 5GS.

The service exposure in clause 5.5 of TS 23.273 [8] is reused with enhancements. The request may indicate with corresponding events if the server requires relative distance/direction of target UEs, absolute positioning of target UEs, or both. The request shall include one or multiple target UEs and optionally it may include additional information such as the SL reference UE for relative distance/direction, the required positioning QoS, etc. For relative distance/direction, at least two UE’s identities are included as attribute in the location service requests and accuracy and latency for distance and/or direction measurements are included in Requested QoS information. The result is returned by the 5GC to the AF via NEF, if needed. The detailed procedure is specified in clause 6.1.X.

In addition, the LCS architecture specified in TS 23.273 [8] is re-used and the 5GC-MT-LR and 5GC-MO-LR procedures there specified are taken as baseline. LMF is the responsible 5GC NF to forward the Ranging/Sidelink Positioning request to the UE, and one Ranging/SL Positioning capable LMF is selected by the AMF for a Ranging/Sidelink Positioning session. The AMF serves either the Target UE or the SL Reference UE or both. In case the selected LMF does not have the context information for all involved UEs, a similar context transfer as specified in TS 23.273 [8] can be performed.

The Ranging/SL Positioning result calculation is performed by LMF or UEs. When absolute positioning is requested, LMF and target UE may decide whether SL positioning or Uu positioning needs to be performed. LMF may combine Ranging/Sidelink Positioning results and Uu positioning results before exposing the results to the AF.

The 5GS Ranging/SL Positioning service request may be initiated by a 5GC NF. In this case, the 5GC NF initially interacts with GMLC.

# * * * * Second change (all new text) * * * *

6.X.Y	Procedures for Service Exposure to the AF

SL-MT-LR procedure defined in clause 6.y of TS 23.273 [8] is performed for Service Exposure to the 5GC NF, AF or the LCS Client.

# * * * * End of changes * * * *