3GPP TSG-SA2 Meeting # 156E (e-meeting)	S2-2304551r8

# Elbonia, April 17th -21st , 2023																			(new CR)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0765 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service operations update to general procedure for Federated Learning between NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, ETRI, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on agreed S2-2303536 in SA2#155 meeting, new service is introduced to support ML model information exchange for Federated Learning in 5GC. The changes proposed in this CR includes the new service operation in the General procedure for Federated Learning between NWDAFs addressing the following EN: Editor's note: The services (i.e., Nnwdaf_MLModelTraining, Nnwdaf_MLModelTrainingInfo) in step 1, 3, 5a and 6 will be updated in the Figure 6.2C.2.1-1. Step 0b is introduced to clarify that the FL Client NWDAF selection by FL Server NWDAF as defined in clause 6.2C.2.1 is used. An EN is included in step 5b since the consumer sends Nnwdaf_MLModelProvision subscription message (stop or continues the training process) which requires new parameters to be defined for Nnwdaf_MLModelProvision service: Editor’s Note: The enhancement with respect to the new input and output parameters required to support Federated Learning in the Nnwdaf_MLModelProvision service operations is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes : - Includes the new service operation in the General procedure for Federated Learning between NWDAFs - Step 0b is introduced to clarify that the FL Client NWDAF selection by FL Server NWDAF as defined in clause 6.2C.2.1 is used. - An EN is included in step 5b |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete procedure defined for Federated Learning support in 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to NWDAF containing MTLF to retrieve/train an ML model, using Nnwdaf_MLModelProvision service (if consumer is NWDAF containing AnLF) as defined in clause 7.5 or Nnwdaf_MLModelTraining_Subscribe Service (if consumer is NWDAF containing MTLF) as defined in clause 7.10 or Nnwdaf_MLModelTrainingInfo_Request(if consumer is NWDAF containing MTLF) as defined in 7.X, including Analytics ID, ML model metric (e.g., ML model Accuracy/Precision/Recall, Accuracy reporting intervalpre-determined status (ML model Accuracy threshold,g. Total training time).

NOTE 1:	The ML model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process, and the FL server NWDAF may stop the training process when the ML model Accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF) provides the time when the ML model  is needed, the FL Server NWDAF can take this information into account to decide the maximum response time for its FL Client NWDAFs.

1. FL Server NWDAF containing MTLF selects NWDAF(s) containing MTLF (FL Client NWDAF) as described in clause 6.2C.2.1.

2.	FL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or  Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF containing MTLF(FL Client NWDAF) that participates in the Federated learning to perform the local model training based on the input parameter in the request from FL Server NWDAF, including model metric. The request also includes the maximum response time before which the FL Client NWDAF has to report the interim local ML model information to the FL Server NWDAF.

3.	Each FL Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

4.	During Federated Learning training procedure, each FL Client NWDAF further trains the ML model provided by the FL Server NWDAF based on its own data, and reports the interim local ML model information to the FL Server NWDAF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Response. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Response may also include the local ML model metric computed by the FL Client NWDAF  and other information about local training in the FL Client NWDAF, e.g. characteristics of local training dataset (e.g., area, sampling ratio, max/min of value of each dimension of data), etc.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The ML model, which is sent from the FL Client NWDAF(s) to the FL Server NWDAF during the FL training process, is the information needed by the FL Server NWDAF to build the aggregated model based on the locally trained ML model(s).

If the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the FL Server NWDAF, the FL Client NWDAF shall  notify the FL Server NWDAF on the delay event with proper cause information (e.g. local ML model training failure, more time necessary for local ML model training) and the expected time to complete the training if available before the maximum response time elapses.

4a.	[Optional]If FL Server NWDAF receives notification that the FL Client NWDAF is not able to complete the training within the maximum response time, the FL Server NWDAF may send to the FL Client NWDAF an extended maximum response time in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request, before which the FL Client NWDAF has to report the interim local ML model information to the FL Server NWDAF. Otherwise, the FL Server NWDAF may indicate FL Client NWDAF to skip reporting for this iteration. FL Server NWDAF includes the current iteration round ID in the message to indicate that the request is to modify the training parameters of the current interation round.

5.	The FL Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model. The FL Server NWDAF may also compute the global model metric, e.g. based on the local model metric(s) or by applying the global model on the validation dataset (if available).

If the FL Server NWDAF provides the maximum response time for the FL Client NWDAFs to provide the interim local ML model information in step 1, or the extended maximum response time in step 4a, the FL Server NWDAF decides either to wait for the FL Client NWDAFs which have not yet provided their interim local ML model within the (extended) maximum response time or aggregates only the retrieved local ML model information instances to update global ML model. The FL Server NWDAF makes this decision, considering the notification/response from the FL Client NWDAF or, if the notification is not received, based on local configuration.

6a.	[Optional] Based on the consumer request in step 0, the FL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message or Nnwdaf_MLModelTraining_Notify Service or Nnwdaf_MLModelTrainingInfo_Response to update the global model metric to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

6b.	[Optional] The consumer decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer re-invoke the same service operation as used in step 0 (i.e., either Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operation) to stop or continue the training process.

Editor’s Note: The enhancement with respect to the new input and output parameters required to support Federated Learning in the Nnwdaf_MLModelProvision service operations is FFS.

6c.	[Optional] Based on the subscription request sent from the consumer in step 6b, the FL Server NWDAF updates or terminates the current FL training process.

If the FL Server NWDAF received a request in step 6b to stop the Federated Training process, steps 7 nd 8 are skipped.

7.	If the FL procedure continues, FL Server NWDAF determines FL Client NWDAF as described in clause 6.2C.2.2 and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the aggregated ML model information to selected FL Client NWDAF(s) for next round of Federated Training.

8.	Each FL Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the FL Server NWDAF at step 7.

NOTE 3:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the Federated Training procedure is complete, the FL Server NWDAF requests the FL client NWDAF(s) to terminate the FL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the FL process has finished and optionally with the final aggregated ML model information. Then the FL client NWDAF(s) terminate the local model training, and if the final aggregated ML model information is received from the FL server NWDAF, the FL client NWDAF(s) can store it for further use.

After the training procedure is complete, the FL Server NWDAF may send Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelTraining_Notify Service or Nnwdaf_MLModelTrainingInfo_Response that includes the globally optimal ML model information to the consumer.

* * * End of Changes * * *