3GPP TSG-SA WG2 Meeting #156e	S2-2304583r02

# April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0194 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource efficiency in MOCN network sharing for location dependent MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When multiple broadcast MBS Sessions are established via different CNs and a shared NG-RAN to deliver the same location-dependent broadcast MBS service, how to achieve resource efficiency is to be decided. The following options are considered: Option 1: Pure NG-RAN based solution. For the same location dependent broadcast MBS service, it is assumed that the same content is delivered in the MBS service areas which are mapped to the same geographical area and belong to the different CNs. After identifying the MBS service areas from different CNs for the same geographical area, the NG-RAN node can decide to apply resource efficient mechanism for delivering the same content in those MBS service areas. Option-2: AF provides enhanced Associated Session ID containing SSM+ Content ID. There is no consensus reached during meeting discussion, and in Show of Hands, Option-1 got more support. This CR reflects the SOH result. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of resource efficiency in MOCN network sharing for location dependent MBS service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource efficiency in MOCN network sharing for location dependent MBS service is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.3.4a	Support of resource sharing across broadcast MBS sessions during network sharing for location dependent broadcast service

When multiple location dependent broadcast MBS sessions are established in different CNs for the same broadcast MBS service, to apply the function of resource sharing across broadcast MBS sessions for location dependent broadcast service as described in clause 6.18,

in addition to clause 7.3.4, the following enhancements or differences apply:

- The MBS session creation procedure is performed with the following additions:

- The AF(s) provides MBS service areas that contain shared radio cells, when requesting to create the associated location dependent MBS sessions towards the participating PLMNs.

NOTE 1:	The MBS service areas of the associated location dependent broadcast MBS sessions can also contain non-shared radio cells among the participating PLMNs.

- The MBS session establishment procedure is performed as defined in clause 7.3.1a with the following additions:

- Step 4: The NG-RAN identifies the associated broadcast MBS sessions which deliver the same location dependent broadcast MBS service, additionally based on MBS service areas received from the MB-SMFs for the broadcast MBS sessions.

- Step 6: The NG-RAN responds with the Area Session ID(s) and N3mb DL Tunnel Info if unicast transport applies over N3mb between MB-UPF(s) and the NG-RAN.

- -	The MBS session release procedure is performed as defined in clause 7.3.2a.

* * * End of Changes * * * *