3GPP TSG-SA WG2 Meeting #156e	S2-2304584r07

# April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0770 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving editor's notes in procedures for analytics exposure in roaming case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Qualcomm, Nokia, Nokia-Shanghai-Bell, vivo, NTT DOCOMO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures for analytics exposure in roaming case have been specified, with the following ENs yet to solve: Editor’s note: it is FFS if both H-NWDAF and V-NWDAF need to have roaming entry capability. Editor’s NOTE: If the HNWDAF with roaming entry capability can not support to generate the requested analytics, then how to exposure the requested analytics is FFS. Editor’s NOTE: If the VNWDAF with roaming entry capability can not support to generate the requested analytics, then how to exposure the requested analytics is FFS. Editor's note: It is FFS whether there is a need to restrict the parameters compared to the non-roaming case. Handling of NSSAIs is FFS. Does source or target PLMN perform a mapping, and are both mapped and original S-NSSAIs required?. It is FFS how HPLMN-ID and VPLMN ID can be provided in a safe manner. It is ffs whether an indication about inbound or outbound roaming scenario is required. Editor's note: How to indicate requesting roaming analytics (e.g. using Analytics Filter Information as below) by the analytics consumer is FFS. Editor's note: Whether and how the V-NWDAF aggregates the analytics from HPLMN and the analytics derived in VPLMN is FFS. Editor's note: Whether and how the H-NWDAF aggregates the analytics from VPLMN and the analytics derived in HPLMN is FFS. For the 1st EN, it is proposed that both H-NWDAF and V-NWDAF have roaming entry capability as it is approved in S2-2301990 for data exchange between PLMNs: This procedure may be used by an NWDAF in the VPLMN to subscribe/unsubscribe to notifications about input data from the HPLMN for inbound roaming users (from the VPLMN perspective). H-NWDAF and V-NWDAF in the procedure are the NWDAFs with roaming entry capability. For the 2nd and 3rd ENs, it is proposed to get the unsupported analytics from another NWDAF in the PLMN if it is available For the 4th and 5th ENs, it is proposed that: 1) parameters related to analytics aggregation, analytics transfer or ML model provisioning should not be included (i.e. those cases are not supported for roaming); 2) mapped S-NSSAI in the HPLMN/VPLMN is optionally provided together with the S-NSSAI in the VPLMN/HPLMN, if available in the consumer NF or NWDAF with roaming entry capability; 3) PLMN ID is provided for the NWDAF to authorize the roaming analytics request/subscription. The security issue (if any) is up to SA WG3; 4) the NWDAF can decide whether inbound or outbound roaming analytics is requested based on the PLMN ID. For the 6th EN, it is proposed that if the consumer NF also indicates request or subscription to VPLMN(/HPLMN) analytics in step 1, the V-NWDAF(/H-NWDAF) simply sends the VPLMN analytics together with the HPLMN(/VPLMN) analytics to the consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve Editor's notes in the procedures for analytics exposure in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved problems/issues in the procedures for analytics exposure in roaming case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5, 6.1.5.1, 6.1.5.2, 6.1.5.3, 6.1.5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

### 6.1.5	Analytics Exposure in Roaming Case

#### 6.1.5.1	General

Based on operator's policy and local regulations (e.g. privacy), analytics may be exchanged between PLMNs (i.e.HPLMN and VPLMN of a UE served by the NWDAF analytics consumer). In this case, an NWDAF is used as entry point in a PLMN to exchange analytics in roaming scenario with other PLMNs. It authorizes the analytics request according to operator policies, and filters the information exposed to other PLMNs. The roaming architecture is defined in clause 4.3.

The H-NWDAF with roaming entry capability may provide analytics to the V-NWDAF with roaming entry capability. The V-NWDAF with roaming entry capability may provide analytics to the H-NWDAF with roaming entry capability.

NOTE X:	It depends on the deployment whether analytics exchange between PLMNs is supported, and thus whether H-NWDAF and V-NWDAF with roaming entry capability are provided in a PLMN.

The H-NWDAF or V-NWDAF with roaming entry capability provides the Nnwdaf_RoamingAnalytics and Nnwdaf_RoamingData services for that purpose. NWDAF with roaming entry capability is the only consumer of these services, i.e., both H-NWDAF and V-NWDAF need to have roaming entry capability when used as entry point to exchange analytics in roaming scenario.

NOTE 1:	The access to the Nnwdaf_AnalyticsSubscription service and the Nnwdaf_AnalyticsInfo service is expected to be restricted by the NRF to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Editor's note:	Analytics exchange between HPLMN and VPLMN may be updated based on feedback from GSMA, e.g. related to user related analytics.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

V-NWDAF may request or subscribe to HPLMN analytics from the H-NWDAF as described in clause 6.x.2, and then the analytics can be leveraged by the 5GC NF in the VPLMN, for example:

Editor's note:	It is FFS whether an analytics profile is required in addition to analytics.

-	In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for network slice selection and SMF selection for PDU Session management.

-	UE-related analytics (e.g. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs.

NOTE Y:	Analytics that rely on input data from the VPLMN are preferably not provided from H-NWDAF to V-NWDAF, but generated by the V-NWDAF.

H-NWDAF may request or subscribe to VPLMN analytics from the V-NWDAF as described in clause 6.x.3, and then the analytics can be leveraged by the 5GC NF in the HPLMN, for example:

-	In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

NOTE Z:	Analytics that rely on input data from the HPLMN are preferably not provided from V-NWDAF to H-NWDAF, but generated by the H-NWDAF.

#### 6.1.5.2	Analytics Exposure from HPLMN to VPLMN

Figure 6.1.5.2-1 shows the procedure that the V-NWDAF requests network analytics (i.e. slice load level analytics, NF load analytics, service experience analytics, etc.) in HPLMN from the H-NWDAF, upon receiving an analytics information/subscription request from the NF (e.g. AMF) in VPLMN.

Figure 6.1.5.2-1: Procedure for analytics exposure from HPLMN to VPLMN

1.	Consumer NF in VPLMN (e.g. AMF) discovers a V-NWDAF with roaming entry capability as described in clause 5.2 and sends an Analytics request/subscribe (Analytics ID, Analytics Filter Information) to V-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. V-NWDAF with roaming entry capability determines based on operator configuration and nature of the requested analytics whether analytics or input data from the HPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the HPLMN are required. If input data from the HPLMN are required, the procedures in Clause 6.2.11 apply.

2.	V-NWDAF checks the policies and regulatory constraints between related to analytics from HPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps are skipped.

V-NWDAF discovers the H-NWDAF with roaming entry capability as described in clause 5.2. V-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information) to H-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in VPLMN.

3a.	H-NWDAF checks the HPLMN operator policies and regulatory constraints between the HPLMN and the VPLMN, and user consent for analytics as defined in clause 6.2.9 if needed, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is rejected, the following steps are skipped.

If H-NWDAF supports to generate the requested analytics, it collects data from the NF(s) and/or OAM in HPLMN and derives the requested analytics; otherwise steps 3b and 3c are executed.

3b-3c. [Optional] If the H-NWDAF with the roaming entry capability does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the HPLMN (if available) for the analytics and get corresponding response/notification.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

4.	H-NWDAF sends the HPLMN analytics information to the V-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or  Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices.

5.	[Optional] If the consumer NF also indicates request or subscription of VPLMN analytics in step 1, V-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested analytics. These steps can be executed in parallel with steps 3-6.

6.	V-NWDAF sends analytics information derived from the HPLMN analytics information received in step 4, and/or possibleinformation obtained in step 5, to the Consumer NF in VPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE:	It is up to implementation and possibly dependent on the nature of the requested analytics information whether and how input data from the VPLMN and analytics from the HPLMN are combined. It is also up to implementation and possibly dependent on the nature of the requested analytics information whether analytics information can be derived locally if no analytics information from the HPLMN can be obtained.

#### 6.1.5.3	Analytics Exposure from VPLMN to HPLMN

Figure 6.1.5.3-1 shows the procedure that the H-NWDAF requests network analytics (i.e. service experience analytics, slice load level analytics, etc.) in VPLMN from the V-NWDAF, upon receiving an analytics information/subscription request from the NF (e.g. PCF) in HPLMN.

Figure 6.1.5.3-1: Procedure for analytics exposure from VPLMN to HPLMN

1.	Consumer NF in HPLMN (e.g. H-PCF) discovers a H-NWDAF with roaming entry capability as described in clause 5.2 and sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to H-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. H-NWDAF with roaming entry capability determines based on operator configuration and nature of the requested analytics whether analytics or input data from the VPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the VPLMN are required. If input data from the VPLMN are required, the procedures in Clause 6.2.10 apply.

2.	The H-NWDAF checks user consent for analytics as defined in clause 6.2.9. H-NWDAF checks the policies and regulatory constraints between the HPLMN and the VPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps are skipped.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

The H-NWDAF discovers the V-NWDAF with roaming entry capability as described in clause 5.2.

H-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information) to V-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in HPLMN.

3a.	V-NWDAF with roaming entry capability checks the policies and regulatory constraints between the HPLMN and the VPLMN, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is rejected, the following steps are skipped.

If V-NWDAF supports to generate the requested analytics, it collects data from the NF(s) and/or OAM in VPLMN and derives the analytics; otherwise steps 3b and 3c are executed.

3b-3c. [Optional] If the V-NWDAF with the roaming entry capability does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the VPLMN (if available) for the analytics and get corresponding response/notification.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

4.	V-NWDAF sends the VPLMN analytics information to the H-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices.

5.	[Optional] If the consumer NF also indicates request or subscription of HPLMN analytics in step 1, H-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the analytics. These steps can be executed in parallel with steps 3-6.

6.	H-NWDAF sends analytics information derived from the VPLMN analytics information received in step 4, and/or possible information obtained in step 5, to the Consumer NF in HPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE:	It is up to implementation and possibly dependent on the nature of the requested analytics information whether and how input data from the HPLMN and analytics from the VPLMN are combined. It is also up to implementation and possibly dependent on the nature of the requested analytics information whether analytics information can be derived locally if no analytics information from the VPLMN can be obtained.

* * * Next Change * * * *

### 6.1.5.x	Contents of Analytics Exposure in roaming case

When requesting or subscribing to analytics involving one or more roaming UEs, the NWDAF service consumer shall send the request or subscription to an NWDAF belonging to the same PLMN as the consumer NF. The NWDAF service consumer may indicate the following parameters in the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation:

-	The parameters as defined in clause 6.1.3, with the following differences:

-	Parameters related to analytics aggregation, analytics transfer or ML model selection should not be included;

-	if Target of Analytics Reporting is included in Analytics Filter Information and indicates specific UEs or a group of UEs, these UEs shall belong to the same HPLMN (if HPLMN analytics are subscribed/requested) or be served by the same VPLMN (if VPLMN analytics are subscribed/requested)

-	Additionally, the following parameters:

-	If the NWDAF service consumer in a VPLMN requests/subscribes to HPLMN analytics, in Analytics Filter Information:

-	[OPTIONAL] PLMN ID of the HPLMN;

NOTE 1:	If PLMN ID of the HPLMN is not provided by the NWDAF service consumer, the V-NWDAF derives it from SUPIs of UEs provided as target for analytics.

-	[OPTIONAL] mapped S-NSSAI of the HPLMN.

-	If the NWDAF service consumer in a HPLMN requests/subscribes to VPLMN analytics, in Analytics Filter Information:

-	[OPTIONAL] PLMN ID of the VPLMN

NOTE 2:	If PLMN ID of the VPLMN is not provided by the NWDAF service consumer, the H-NWDAF inquires it at the UDM for the UEs indicated as target of analytics reporting.

NOTE 3:	In this release of the specification, only one PLMN ID (of HPLMN or VPLMN) is supported in the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation..

Based on the analytics request or subscription from the NWDAF service consumer in the HPLMN, and local configuration, the H-NWDAF with roaming entry capability may indicate the following parameters in the Nnwdaf_RoamingAnalytics_Request or Nnwdaf_RoamingAnalytics_Subscribe service operation to the V-NWDAF with roaming entry capability, for requesting/subscribing to analytics in the VPLMN:

Editor's note:	The parameters are FFS and should be aligned with the service definition for roaming analytics.

-	Analytics ID;

-	PLMN ID of the HPLMN;

NOTE 4:	It is up to SA3 to decide if a special protection of the PLMN ID of the HPLMN is required.

-	Analytics Filter Information:

-	[OPTIONAL] HPLMN S-NSSAI

NOTE 5:	V-NWDAF maps that S-NSSAI to an S-NSSAI of the VPLMN which will be used in the  Analytics Filter .

-	other Analytics Filter Information (e.g. AOI in the VPLMN) as provided by NWDAF service consumer in the HPLMN, if applicable in the VPLMN.

-	other parameters as provided by NWDAF service consumer in the HPLMN, if applicable in the VPLMN.

Based on the analytics request or subscription from the NWDAF service consumer in the VPLMN, and local configuration, the V-NWDAF with roaming entry capability may indicate the following parameters in the Nnwdaf_RoamingAnalytics_Request or Nnwdaf_RoamingAnalytics_Subscribe service operation to the H-NWDAF with roaming entry capability, for requesting/subscribing to analytics in the HPLMN:

Editor's note:	The parameters are FFS and should be aligned with the service definition for roaming analytics.

-	Analytics ID;

-	PLMN ID of the VPLMN;

NOTE 6:	It is up to SA3 to decide if a special protection of the PLMN ID of the VPLMN is required.

-	Analytics Filter Information:

[OPTIONAL] HPLMN S-NSSAI

NOTE 7:	If an S-NSSAI but no mapped S-NSSAI is provided by the NWDAF service consumer in the Analytics Filter Information, the V-NWDAF maps the S-NSSAI to the S-NSSAI of the HPLMN and provides that mapped S-NSSAI of the HPLM as the Analytics Filter Information.

-	other Analytics Filter Information (e.g. AOI in the VPLMN) as provided by NWDAF service consumer in the VPLMN, if applicable in the HPLMN.

-	other parameters as provided by NWDAF service consumer in the VPLMN, if applicable in the HPLMN.

The NWDAF with roaming entry capability provides the following output information to the consumer NWDAF of the Nnwdaf_RoamingAnalytics_Request or Nnwdaf_RoamingAnalytics_Notify service operation:

-	The output information as defined in clause 6.1.3, with the following differences:

-	Information related to analytics aggregation (i.e. analytics metadata information) should not be included.

NOTE 2:	the output of the analytics depends on the operator policies and local regulations.

* * * End of Changes * * * *