3GPP TSG-WG SA2 Meeting #156-e	S2-2304608r01

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0771 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Model storage and retrieval from ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new service that Nnwdaf_MLModelTraining_Subscribe has been defined for an NWDAF containing MTLF to retrieve ML model from another NWDAF containing MTLF. For this reason, a clarification is added to the step 1 in clause 6.2B.7; Similar as software, ML Model with a specific ML Model ID may has different versions. Although the ML Model ID for an ML model keeps the same, but the version number can be updated when the ML model is developed during its life cycle. Thus, apart from the ML Model identifier, ML model version number is added as an additional index to store/retrieve ML Model from the ADRF. Besides the model ID and address of Model file, actually the storage size required for the ML model is also an important information to store this ML model in the ADRF. Without indicating it, the ADRF will not know how much storage resource sholud be prepared for this ML Model, then it may fail when it downloads this model file from the address of model file. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add ML model version number as an index to store/retrieve ML Model from the ADRF apart from the ML Model identifier; - Add Storage size required for the ML model as a new input to store an ML Model to the ADRF. - Clarify the Nadrf_MLModelManagement_StorageRequest service could also be used to update ML Model which has been stored in the ADRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Important information missing for Model storage/retrieval from ADRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.7, 10.3, 10.3.2, 10.3.3, 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2B.7	ML model retrieval from ADRF

The procedure depicted in Figure 6.2B.7-1 is used by consumers (NWDAF containing MTLF) to retrieve ML models from an ADRF.

Figure 6.2B.7-1: Procedure for ML model(s) retrieval from ADRF

1.	The NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) subscribes/requests for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Request service operation.

2.	The NWDAF containing MTLF determines whether the set of ML Model(s) associated with a/an (set of) Analytics ID(s) should be retrieved from the ADRF.

When NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF directly, steps 3 and 4 is skipped.

If NWDAF containing MTLF determines that the set of ML Model(s) corresponding Analytics ID(s) requested in step 1 needs to be retrieved from ADRF and the NF consumer is agnostic to where the ML model(s) is stored, then Steps 3 and 4 is performed.

Editor's note:	How NWDAF containing MTLF and ADRF authorizes the NF Consumer is in scope of SA WG3.

3.	The ADRF service consumer (NWDAF containing MTLF) requests for the ML model stored in ADRF by invoking the Nadrf_MLModelManagement_Retrieve request (Analytics ID(s), Storage Transaction Identifier, one or more tuples of unique ML model Identifier, ML model version number) service operation.

4.	The ADRF sends Nadrf_MLModelManagement_Retrieve response (address e.g., URL or FQDN of Model file stored in ADRF) service operation.

5.	The NWDAF containing MTLF notifies/ response to the NWDAF service consumer with the tuple Analytics ID, one or more tuples of unique ML Model identifier, ML model version number and ML Model Information. The ML model information may contain the ML model file address (e.g. URL or FQDN) or ADRF (Set) ID. When ADRF (Set) ID is provided, a Storage Transaction Identifier may be provided. The ADRF(Set) ID in included only when the NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF in step 2.

6.	If in step 5, the NWDAF service consumer (NWDAF containing MTLF) received ADRF ID (where the ML model(s) requested in step 1 is stored), then the NWDAF containing MTLF may invoke the Nadrf_MLModelManagement_Retrieve Subscribe/Request (Analytics ID(s), Storage Transaction Identifier or one or more tuples of unique ML Model identifier and ML model version number stored in ADRF) service operation to get the ML model stored in ADRF.

7.	The ADRF sends Nadrf_MLModelManagement_Retrieve notify/ response (address (e.g., URL or FQDN) of Model file stored in ADRF to the NWDAF containing MTLF.

Editor's note:	Whether the ADRF sends ML model(s) when requested the NF Consumer is FFS.

# * * * * Next change * * * *

## 10.3	Nadrf_MLModelManagement service

### 10.3.1	General

Service Description: This service enables the consumer to store and update and remove ML model(s) or ML model address(es) from an ADRF.

### 10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML model(s). ML model(s) or ML model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: ML model file or Set of:

-	the tuple (NF instance ID of the NWDAF containing MTLF, address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF and Storage size required for the ML model), when multiple ML models is not supported; or

-	the tuple (NF instance ID of the NWDAF containing MTLF, one or more tuples of unique ML Model identifier, ML model version number, address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF and Storage size required for the ML model).

NOTE:	The ADRF can download ML model files based on the ML model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML model and locally stores the ML model is left for implementation.

Inputs, Optional: Validity period, Spatial validity.

Outputs Required: Result Indication, one of the following:

-	address (e.g. URL or FQDN) of Model file stored in ADRF, when multiple ML models is not supported; or

-	one or more tuples of unique ML Model identifier, ML model version number and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

# * * * * Next change * * * *

### 10.3.3	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML model file(s) or ML model address(es).

Inputs, Required: When multiple ML models is not supported:

-	Storage Transaction Identifier; or

-	address (e.g. URL or FQDN) of Model file stored in ADRF, when multiple ML models is not supported.

When multiple ML models is supported:

-	one or more tuples of unique ML Model identifier, ML model version number and address (e.g. URL or FQDN) of Model file stored in ADRF.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

### 10.3.4	Nadrf_MLModelManagement_Retrieval Request service operation

Service operation name: Nadrf_MLModelManagement_Retrieval Request

Description: The consumer NF uses this service operation to request the ADRF to get the ML model(s) stored in ADRF.

Inputs, Required:

-	Analytics ID(s);

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier and ML model version number stored in ADRF.

Inputs, Optional: None.

Outputs Required:

-	address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Validity Period.

# * * * * End of changes * * * *