3GPP TSG-SA2 Meeting #156-EMeeting	S2-2304630r01

# Elbonia, 17  - 20 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4364 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Rate Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The QoS Monitoring includes measurement and reporting of the QoS parameters. Text has been clariffied and improved |  |  |  |  |  |  |  |  |
| Summary of change: |  | The QoS Monitoring includes measurement and reporting of the QoS parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measured UL and/or DL data rate can not be used by the AF to perform the bitrate changing for the XRM service data flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.1, 5.45.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 5.45	QoS Monitoring

### 5.45.1	General

QoS monitoring comprises of measurements of QoS parameters and reports of the measured values of QoS parameters for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF.

The PCF may generate the authorized QoS Monitoring policy for a service data flow based on the QoS Monitoring request received from the AF (as described in clause 6.1.3.21 of TS 23.503 [45]). The PCF includes the authorized QoS Monitoring policy in the PCC rule and provides it to the SMF.

The SMF may configure the RAN and/or UPF to perform QoS monitoring for the QoS Flow.

The SMF may configure the UPF to report the monitoring results as described in clause 5.8.2.18 with parameters determined by the SMF based on the authorized QoS Monitoring policy received from the PCF and/or local configuration.

The following clauses describe the QoS parameters which can be measured and any specific actions or constraints for their measurement.

NOTE:	The QoS parameter which can be measured are parameters which describe the QoS experienced in the 5GS by the application, i.e. they are not restricted to the 5G QoS Parameters defined in clause 5.7.2.

### 5.45.2	Packet delay monitoring

QoS Monitoring for packet delay allows for the measurement of UL packet delay, DL packet delay or round trip packet delay between UE and PSA UPF. The details of the QoS Monitoring for packet delay are described in clause 5.33.3.

### 5.45.3	Congestion information monitoring

The NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information (i.e. a percentage of congestion level for exposure). The UPF may be required to monitor the UL and/or DL QoS Flow congestion information reported from the NG-RAN.

QoS monitoring request to the NG-RAN and NG-RAN reporting for UL and/or DL QoS Flow congestion information to PSA UPF is as defined in 5.37.3. The PSA UPF reports the received UL and/or DL QoS Flow congestion information to the target NF as instructed by the QoS Monitoring request from the SMF.

### 5.45.4	Data rate monitoring

The data rate is the measured traffic UL and/or DL data rate, for a QoS flow.

Either the NG-RAN or the UPF may be required to provide the QoS Monitoring on the measured traffic UL and/or DL data rate . The SMF or the PSA UPF may be required to monitor data rate report from the NG-RAN.

The QoS Monitoring on the UL and/or DL traffic data rate measurement is per QoS flow level, or per PDU Session level. And it can be applied to a Non-GBR or GBR QoS flow.

According to the QoS Monitoring request for UL and/or DL data rate from SMF, either the UPF or the NG-RAN is required to initiate data rate measurement for a QoS Flow based on the QoS Monitoring request from SMF and may report the measured data rate to the SMF as instructed by the SMF.

Editor's note:	It is pending for RAN WGs to confirm whether providing data rate information for QoS Flow to the CN can be included or not in Release-18.

### 5.45.5	QoS notification monitoring

QoS monitoring of QoS notification allows that QoS notification control information reported by NG-RAN can be exposed directly by UPF. The QoS Notification Control for a GBR QoS flow is as defined in clause 5.7.2.4. When the NG-RAN is instructed to perform QoS Notification Control and report the QoS notification (e.g. events of "GFBR can no longer be guaranteed" and "GFBR can be guaranteed again") via the tunnel(s) between the NG RAN and UPF as defined in clause 5.7.2.4, the PSA UPF is required by SMF to monitor the event report from the NG-RAN by the SMF.

The QoS Monitoring on the Events of "GFBR can no longer be guaranteed" and "GFBR can be guaranteed again" is per GBR QoS flow level.

Editor's note:	It is for pending to RAN WG to confirm whether providing QoS Notification for GBR QoS Flow to the CN via User Plane can be included or not in Release-18.

Editor's note:	It is FFS whether Direct exposure of QoS Notification does need to be part of QoS Monitoring of rather more as an extension to 5.7.2.4 and a new UPF Event.

## END of CHANGES