3GPP TSG-SA2 Meeting #156-EMeeting	S2-2304630r04

# Elbonia, 17  - 20 April 2023														(Revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4364 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Rate Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The QoS Monitoring includes measurement and reporting of the QoS parameters. The data rate for a QoS Flow can be different types. One type of data rate is the measured traffic data rate of the QoS Flow, if there is no user traffic, there is no data rate (i.e. the measured traffic data rate = 0). Another type of data rate is the measured available data rate of the QoS Flow. Even if there is no user traffic, there is still available radio resource that can be used by the QoS Flow to transport the PDUs, in such case, the measured traffic data rate =0 but the measured available data rate can be used by the AF adaptively to change the XR stream codec below the measured available data rate. The NG-RAN and PSA UPF can measure the measured traffic data rate. But only the NG-RAN can measure the measured available data rate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The QoS Monitoring includes measurement and reporting of the QoS parameters. Change the data rate into two types of data rate, one is the measured traffic UL and/or DL data rate, another is the measured available UL and/or DL data rate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measured UL and/or DL data rate can not be used by the AF to perform the bitrate changing for the XRM service data flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.45.4	Data rate monitoring

The data rate is the measured traffic UL and/or DL data rate, and/or the measured available UL and/or DL data rate for a QoS flow.

Either the NG-RAN or the UPF may be required to provide the QoS Monitoring on the measured traffic UL and/or DL data rate . The PSA UPF may be required to monitor data rate report from the NG-RAN.

The NG-RAN may be required to provide the QoS Monitoring on the measured available UL and/or DL data rate to the SMF.

The QoS Monitoring on the UL and/or DL traffic or available data rate measurement is per QoS flow level, or per PDU Session level. And it can be applied to a Non-GBR or GBR QoS flow.

According to the QoS Monitoring request for UL and/or DL data rate from SMF, either the UPF or the NG-RAN is required to initiate data rate measurement for a QoS Flow based on the QoS Monitoring request from SMF and may report the measured data rate to the SMF as instructed by the SMF.

Editor's note:	It is pending for RAN WGs to confirm whether providing measured traffic and or available data rate information for QoS Flow to the CN can be included or not in Release-18.

END of CHANGES