3GPP TSG-WG SA2 Meeting #156e	S2-2304634

# E-Meeting April 17 – 21, 2023		(revision of S2-2303026)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0321 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MT-LR for Ranging and Sidelink Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is concluded in FS_Ranging_SL study (TR 23.700-86) that the LMF may be used to support network assisted SL Positioning when one of the UEs involved in Ranging and Sidelink Positioning is in coverage. This CR provides the MT-LR based procedure for network assisted Ranging and Sidelink Positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MT-LR based procedure for network assisted Ranging and Sidelink Positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete network assisted Ranging and Sidelink Positioning support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 3.2; (new) 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LPI	LCS Privacy Indicator

LRF	Location Retrieval Function

MBSR	Mobile Base Station Relay

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

SL-MT-LR	Sidelink Mobile Terminating Location Request

# * * * * next change * * * *

## 6.x	Network assisted Ranging/SL Positioning procedures

## 6.x.y	SL-MT-LR Procedure

Figure 6.x.y-1 illustrates a procedure to enable an LCS Client or AF to obtain Ranging/Sidelink Positioning location results for a UE, designated UE1 (i.e. Target UE in TS 23.586) using one or more other UEs designated UE2, UE3, … UEn (n ≥ 2) (i.e. Reference/Located UEs in TS 23.586). The Ranging/Sidelink Positioning location results may include absolute locations, relative locations or ranges and directions related to the UEs, based on the service request.

Figure 6.x.y-1: SL-MT-LR Procedure

Precondition:

At least one of the n UEs is in coverage and registered with a serving PLMN.

1.	The LCS Client or the AF (via NEF) sends an LCS service request to the (H)GMLC for Ranging/Sidelink Positioning location results for the n UEs which may each be identified by a GPSI or a SUPI. The request may include the required QoS, the required location results (e.g. absolute locations, relative locations or distances and/or directions related to the UEs), the SL reference UE(s) in case of relative locations, distance, or direction. The service request may also indicate threshold(s) for distance and/or direction events as well other optional events such as time to trigger, direction orientation, threshold matching direction, measurement frequency, preferred accuracy and other attributes. The (H)GMLC or NEF authorizes the LCS Client or the AF for the usage of the LCS service. If the authorization fails, the remaining steps are skipped and the (H)GMLC or NEF responds to the LCS Client or the AF with the failure of the service authorization. In some cases, the (H)GMLC derives the GPSIs or SUPIs of the n UEs and possibly the QoS from either subscription data or other data supplied by the LCS Client or AF.

If the LCS service request contains pseudonyms of the n UEs (i.e. neither GPSI nor SUPI) and the (H)GMLC cannot resolve the PMD address from the pseudonym, the (H)GMLC itself determines the GPSI or SUPI of the n UEs. If the (H)GMLC can resolve the address of PMD from the pseudonym, the HGMLC requests the GPSI or SUPI from its associated PMD. If (H)GMLC is not able to obtain the GPSI or SUPI of any of the n UEs, the (H)GMLC shall cancel the location request. The (H)GMLC proceeds with step 2 for each of the n UEs for which a GPSI or SUPI is available (i.e. m UEs where m≤n).

NOTE 1:	How the LCS Client or AF determines before step 1 that the n UEs are all in proximity to one another and obtains their Application Layer IDs is outside the scope of this specification.

2.	The (H)GMLC invokes a Nudm_SDM_Get service operation towards the UDM of each of the n UEs to get the privacy settings of the UE identified by its GPSI or SUPI. The UDM returns the UE Privacy setting of the UE. The (H)GMLC checks the UE LCS privacy profile. If any of the m UEs are not allowed to be located, steps 3-19 are skipped for these UEs.

3.	The (H)GMLC invokes a Nudm_UECM_Get service operation towards the UDM of each of the m UEs (for which GPSI or SUPI is available), one at a time, using the GPSI or SUPI of each UE. If any of the m UE is in coverage and has registered with a serving PLMN, the UDM returns the network addresses of the current serving AMF and additionally the address of a VGMLC (for roaming case). The (H)GMLC checks the country codes of the serving AMF addresses. If the (H)GMLC finds the current AMF is out of the service coverage of the (H)GMLC, the (H)GMLC returns an appropriate error message to the LCS client or AF (via NEF). The (H)GMLC stops the Nudm_UECM_Get service operations towards the UDM as soon as the UDM returns serving PLMN information for one of the m UEs.

NOTE 2:	The UDM is aware of the serving AMF address at UE registration on an AMF as defined in clause 4.2.2.2.2 of TS 23.502 [19]. The UDM is aware of a serving VGMLC address at UE registration on an AMF as defined in clause 4.2.2.2.2 of TS 23.502 [19].

4.	For a non-roaming case, this step is skipped. In the case of roaming, the HGMLC may receive an address of a VGMLC (together with the network address of the current serving AMF) from the UDM in step 3, otherwise, the HGMLC may use the NRF service in the HPLMN to select an available VGMLC in the VPLMN, based on the VPLMN identification contained in the AMF address received in step 3. The HGMLC then sends the location request to the VGMLC by invoking the Ngmlc_Location_ProvideLocation service operation towards the VGMLC. In the cases when the HGMLC did not receive the address of the VGMLC, or when the VGMLC address is the same as the HGMLC address, or when both PLMN operators agree, the HGMLC sends the location service request message to the serving AMF. In this case, step 4 is skipped. The H-GMLC also provides the LCS client type of AF, if received in step 1, or LCS client type of LCS client and other attributes to be sent to AMF in step 5.

5.	In the case of roaming, the VGMLC first authorizes that the location request is allowed from this HGMLC, PLMN or from this country. If not, an error response is returned. The (H)GMLC or VGMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request sidelink positioning/ranging location results of the m UEs. The service operation includes the SUPI of UE1, the Application layer IDs of the m UEs, the client type and may include the required LCS QoS, the required location results (e.g. absolute locations, relative locations or ranges and directions related to the UEs) and other attributes as received or determined in step 1.

NOTE 3:	The location request forwarded at step 4 and step 5 may also carry the result of the privacy check in step 2 and an indication of a privacy related action.

6.	If UE1 is in CM IDLE state, the AMF initiates a network triggered Service Request procedure to establish a signalling connection with UE1.

If signalling connection establishment fails, steps 7-17 are skipped.

7.	If the indicator of privacy check related action indicates that UE1 must either be notified or notified with privacy verification and if UE1 supports LCS notification (according to the UE capability information), a notification invoke message is sent to UE1, indicating the identity of the LCS client, the type of location and whether privacy verification is required.

8.	UE1 notifies the user of the location request and, if privacy verification was requested, waits for the user to grant or withhold permission. UE1 then returns a notification result to the AMF indicating, if privacy verification was requested, whether permission is granted or denied for the current LCS request. If the user does not respond after a predetermined time period, the AMF shall infer a "no response" condition. The AMF shall return an error response in step 18, and if roaming VGMLC in step 19, to the HGMLC if privacy verification was requested and either the UE1 user denies permission or there is no response with the indication received from the (H)GMLC indicating barring of the location request and steps 9~17 are skipped.

9.	The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Ranging/Sidelink Positioning) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information received at step 5.

10.	The LMF sends an SL-MT-LR request to the serving AMF as a supplementary services message, using the Namf_Communication_N1N2MessageTransfer service operation, and a Correlation ID identifying the LMF. The SL-MT-LR request includes the application layer IDs of the other UEs 2 to m, the conditions for cancelling the location request when not all UEs 2 to m can be discovered and the types of Ranging/Sidelink Positioning location result needed.

11.	The serving AMF forwards the SL-MT-LR request and a Routing ID equal to the Correlation ID to UE1 using a DL NAS TRANSPORT message.

12.	UE1 attempts to discover the other UEs 2 to m using their application IDs and if not already discovered using procedure defined in 6.4 of TS 23.586[40]. If not all UEs 2 to m are discovered and if the conditions for cancelling the location request when not all m UEs can be discovered are met, UE1 returns a negative response at step 14 and the procedure terminates after the LMF returns the negative response to the LCS Client or AF via the AMF and GMLC(s).

13.	UE1 obtains (e.g. requests and receives) the sidelink positioning capabilities of the discovered UEs if not already obtained.

14.	UE1 returns a supplementary services SL-MT-LR response to the serving AMF in an UL NAS TRANSPORT message and includes the Routing ID received in step 11. The SL-MT-LR response indicates if the SL-MT-LR request can be supported and which of UEs 2 to n have been discovered and are available for positioning.

15.	The serving AMF forwards the SL-MT-LR response to the LMF indicated by the Routing ID received at step 14 and includes a Correlation ID equal to the Routing ID.

16.	Ranging/Sidelink Positioning of UE1 and the other discovered UEs occurs as for an SL-MO-LR as described for steps 10-19 of Figure 6.x.z-1 with the difference that Ranging/Sidelink Positioning location results are always obtained and the LMF indicates to UE1 at step 13 or step 14 of Figure 6.x.z-1 whether the Ranging/Sidelink Positioning location results will be calculated by the LMF (at step 19) or by UE1 (at step 17).

17-20.	The LMF returns the sidelink positioning/ranging location results to the LCS Client or AF as in steps 13-15 and step 24 of clause 6.1.2.

# * * * * End of changes * * * *