3GPP TSG-WG SA2 Meeting #156e	S2-2304644r01

# E-Meeting April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4367 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Open issue resolutions for MBSR support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clasue 5.35A introduced the MBSR support. However, there are still open issues identified by the editor notes. These editor notes needs to be addressed to allow the 5G system working properly with the MBSR. This CR provides the resolutions to the open issues based on the feedbacks from RAN WGs and SA3 WGS. For the CAG mechanism, it was agreed that the location restriciton is not supported in this release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve open issues identified by the editor notes in clause 5.35A. Remove the suport of locaiton restriciton for CAG control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of MBSR operation is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.35A	Support for Mobile Base Station Relay (MBSR)

### 5.35A.1	General

The MBSR uses the IAB architecture as defined in clause 5.35, and operates as an IAB node (with an IAB-UE and gNB-DU) with mobility when integrated with the serving PLMN. The architecture described in clause 5.35 applies unless specific handling is specified in clause 5.35A. Additionally, the following limitations apply to the MBSR:

-	the MBSR has a single hop to the IAB-donor node;

-	NR Uu is used for the radio link between a MBSR and served UEs, and between MBSR and IAB-donor node.

Regulatory requirements (e.g. emergency services, priority services) are supported when UEs access 5GS via a MBSR. LCS framework as defined in TS 23.273 [87] is used for providing the location service to the served UEs, with additional enhancements described in clause 5.35A.5.

Roaming of the MBSR is supported, i.e. a MBSR can integrated with a VPLMN's IAB-donor node. The corresponding enhancements to support MBSR roaming are described in clause 5.35A.4.

CAG mechanism as defined in clause 5.30 can be used for the control of UE's access to the MBSR. Optional enhancements to the CAG mechanism for MBSR use are described in clause 5.35A.7.

For a MBSR node, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization as described in clause 5.35A.4.

After the IAB-UE performs registration procedure in 5GS, further mobility procedure can be performed to change the IAB-donor-DU, the IAB-donor-CU as specified in TS 38.401 [42]. The mobility support of UEs served by the MBSR is specified in clause 5.35A.3

Editor's note:	Description of RRC indication and full migration procedure will be updated after coordination with RAN WGs.

### 5.35A.2		Configuration of the MBSR

In order for an MBSR to operate as a mobile IAB node, it receives configuration from the OAM system of the serving PLMN as specified in TS 38.401 [42]. The MBSR IAB-UE establishes a secure and trusted connection to the OAM server only if it is authorized to operate as MBSR in the serving PLMN.

In addition, the MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN to perform PLMN selection as specified in TS 23.122 [17].

When a PDU session is used for the MBSR to access the OAM server, the MBSR establishes a dedicated PDU session for the OAM traffic. The serving PLMN provides an Allowed NSSAI and establishes the PDU session for the OAM server access, considering the S-NSSAI and DNN requested by MBSR and/or the default values in subscription data. The MBSR can be (pre-)configured with UE policy or provisioned using existing UE Policy mechanism as defined in TS 23.503 [45] including the OAM access PDU session parameters for the authorized PLMNs.

### 5.35A.3	Mobility support of UEs served by MBSR

#### 5.35A.3.1	UE mobility between a fixed cell and MBSR cell

The procedure of Inter-gNB-DU Mobility as defined in TS 38.401 [42] or the handover procedure using the Xn/N2 reference points as defined in TS 23.502 [3] can be used.

For UEs in RRC_IDLE and RRC_INACTIVE state when a MBSR goes out-of-service, procedure for cell (re-)selection as specified in TS 38.304 [50] for RRC_IDLE and RRC_INACTIVE is used.

For UEs in RRC_CONNECTED state, if the MBSR goes out-of-service due to e.g. MBSR moves to an area where the MBSR is not allowed to provide the relay service, the procedure for IAB node release as specified in TS 38.401 [42] is used.

The IAB-donor-CU triggers handover procedure when it is possible for the UEs accessing emergency service and being served by the MBSR, if MBSR is about to become unavailable to provide the services.

#### 5.35A.3.2	UE mobility between MBSR cells

Similar to the behaviours described in clause 5.35A.3.1, UEs use existing procedures defined in TS 38.401 [42], TS 23.502 [3], or TS 38.304 [50] to handle the mobility between MBSR cells.

#### 5.35A.3.3	UE mobility when moving together with a MBSR cell

The TAC broadcasted by the MBSR cell(s) can be configured by the OAM or donor-CU. When MBSR moves to a serving cell with a different TAC, the TAC to be broadcasted by the MBSR may also change.

For a UE served by a MBSR cell, it may observe change of TAC and/or cell IDs, even if it is still connected to the same MBSR. This can trigger mobility registrations, as defined in TS 23.502 [3], if the new TAC is not in the TAI list in the RA.

Editor's note:	How to configure the TAC and further updates will be added based on RAN WG inputs.

### 5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods. The subscription information for the MBSR is defined in TS 23.502 [3] clause 5.2.3.3.1.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the 5GC can make use of it for authorization of MBSR in VPLMN. MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

The MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN for the MBSR operation.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to NG-RAN. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation. The indication for MBSR is different from that of the IAB indication described in clause 5.35.

Editor's note:	Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined.

NOTE 1:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.1.

The AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR IAB node as part of registration procedure, and in this case the AMF does not include MBSR authorization indication to donor-gNB. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN).

Editor's note:	In what conditions and how (e.g. UCU or deregistration with re-registration) an MBSR is informed when there is a change in the MBSR authorization information is FFS.

NOTE 2:	The mechanism applies to both roaming and non-roaming MBSR operations.

### 5.35A.5	Location Service Support of UEs served by MBSR

When a UE accesses 5GS via a MBSR, it can use the location service as defined in TS 23.273 [87]. However, in order to provide accurate estimation of the UE location, LMF needs to take the location of the MBSR into account. Enhancements to the LCS framework for MBSR support is described in clause 5.9 of TS 23.273 [87].

### 5.35A.6	Providing cell ID/TAC of MBSR for services

Editor's note:	The description of how to provide the MBSR cell ID/TAC for certain services will be enhanced if needed based on RAN WG work progress.

The TAC and cell ID broadcasted by the MBSR cell(s) are configured as specified in TS 38.470 [165].

### 5.35A.7	Control of UE access to MBSR

CAG Identifier is used to control the access of UE via MBSR (i.e. mobile IAB-node) and existing CAG mechanism defined in clause 5.30.3 can be used for managing UE's access to MBSR, with the following additional considerations:

-	When the MBSR is allowed to operate as an IAB node for a PLMN, the MBSR is configured, either during the communication with the serving PLMN OAM or (pre-)configuration mechanism, with a CAG identifier which is unique within the scope of this PLMN. If the MBSR is (pre-)configured with the PLMN list in which the MBSR is allowed to operate as MBSR, the corresponding CAG Identifier per PLMN is also configured in the MBSR.

-	NG-RAN and 5GC support the UE access control based on the CAG identifier associated with the MBSR cell and the allowed CAG identifiers for the UE that supports CAG functionality.

-	For the UE that does not support CAG functionality, NG-RAN and 5GC are allowed to use not only CAG mechanism but also the other existing mechanism e.g. forbidden Tracking Area, to manage its access to MBSR.

NOTE 1:	If CAG for both MBSR and fixed private network are supported in a PLMN, the CAG identifier value of the MBSR cell(s) need to be different from the CAG identifier(S) of fixed cell(s) and unique within the scope of a PLMN ID.

-	Time duration restriction may be provided together with the CAG Identifier(s) for the MBSR(s) that the UE can access. The enhanced Allowed CAG list will be provided to UE and AMF for enforcement, to make sure that UE not accessing the MBSR cell outside of the time duration. For example, if the time when a certain CAG is allowed for a UE is up, the CAG for the UE is revoked from the network.

Editor's note:	Further description on the efficient UE configuration approach based on time.

NOTE 2:	Control of the MBSR access to the serving network is based on normal mobility restriction management based on subscription data form MBSR (i.e. IAB-UE).

# * * * * End of changes * * * *