SA WG2 Meeting #156e	S2-2304650r01

# Elbonia, April 17 – 21, 2023           	  		 		  		                        (revision of S2-2302751)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0100 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of inter-PLMN edge relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR aims at specifying support of edge relocation for accessing EHE when roaming as concluded in TR 23.700-48 clause 8.1.2 as following: “For edge relocation in roaming scenarios, the AF sends PLMN ID corresponding to target EAS to the network, and an inter-PLMN relocation indicator is transferred between V-SMF and H-SMF to indicate that the EAS relocation is between HPLMN and VPLMN.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add general description for edge relocation of inter-PLMN. - Add procedure for edge relocation of inter-PLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of edge relocation for accessing EHE when roaming as concluded in TR 23.700-48 clause 8.1.2 would not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 4016, TS 23.503 CR 0868 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.3.1	General

Edge Relocation refers to the procedures supporting EAS changes and/or PSA UPF relocation.

Edge Relocation may be triggered by an AF request (e.g. due to the load balance between EAS instances in the EHE) or by the network (e.g. due to the UE mobility).

With Edge Relocation, the user plane path may be re-configured to keep it optimized. This may be done by PDU Session re-establishment using SSC mode 2/3 mechanisms or Local PSA UPF relocation using UL CL and BP mechanisms. The corresponding procedures are defined in TS 23.501 [2] and TS 23.502 [3].

Due to Edge Relocation, the UE may need to re-discover a new EAS and establish the connectivity to the new EAS to continue the service. The re-discovery of EAS is specified in clause 6.2.

Edge Relocation may result in AF relocation, for example, as part of initial PDU Session Establishment, a central AF may be involved. However, due to Edge Relocation another AF serving the Edge Applications is selected.

The trigger of Edge Relocation by the network is specified in clause 4.3.6.3 of TS 23.502 [3]. Some EAS (re-)Discovery procedures in clause 6.2 may also trigger Edge Relocation.

For the edge relocation in roaming scenarios, the AF may trigger EAS relocation between VPLMN and HPLMN.

This clause further describes the following procedures:

-	Edge Relocation involving AF change.

-	Edge Relocation using EAS IP replacement.

-	AF request for simultaneous connectivity for source and target PSA.

-	Packet buffering for low Packet Loss.

-	Edge Relocation considering User Plane Latency Requirements.

-	Edge Relocation triggered by AF

-	Edge Relocation for a set of UEs for common DNAI.

-	inter-PLMN Edge Relocation

Annex F describes example procedure for EAS Relocation on Release 16 capabilities.

For HR-SBO PDU Sessions specified in clause 6.7, the AF may send to V-NEF an AF request to influence traffic routing for supporting Edge Relocation (e.g. for the purpose of subscription to UP path management events, especially for the change of local PSA UPF in VPLMN).

SECOND CHANGE

6.3.X	inter-PLMN Edge relocation

6.3.X.1	General

When roaming, the edge relocation may be triggered from HPLMN to VPLMN.

The AF provides  inter-PLMN EAS relocation indication in AF request as specified in clause 5.6.7 of TS 23.501[2], which indicates the EAS is relocated to the VPLMN. The PCF includes the inter-PLMN EAS relocation indication in PCC rule and send it to H-SMF to determine target DNAI for V-SMF selection. Then an inter-PLMN relocation information is transferred between V-SMF and H-SMF to indicate that the EAS relocation is from HPLMN to VPLMN. The EAS IP replacement mechanism as described in clause 6.3.3 is reused for traffic routing to target EAS in VPLMN.

Editor's note:	How to support AF triggered edge relocation via  interacting with VPLMN is FFS.

6.3.X.2	EAS relocation from HPLMN to VPLMN

Figure 6.3.X.2-1: EAS relocation from HPLMN to VPLMN

0.	The UE establishes HR PDU Session and the UE accesses the source EAS in HPLMN.

1.	When the EAS relocation is triggered, AF determines the source EAS should be relocated to a target EAS in VPLMN, which is serving the UE (e.g. based on event notification about the new PLMN Identifier for the PDU session and EAS in VPLMN is available). Based on the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3], AF invokes Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update service operation to send the request message to NEF, or invokes Npcf_PolicyAuthorization_Create or Npcf_PolicyAuthorization_Update to send the request message to the PCF directly. The request message includes the inter-PLMN EAS relocation indication, which indicates the EAS is relocated to the VPLMN .

The EAS IP replacement information (i.e. source EAS IP address and port number, target EAS IP address and port number) is provided as specified in clause 6.3.3. The AF may also provide target DNAI.

2.	If the PCF authorizes the traffic routing in the VPLMN is allowed, the PCF creates and sends the PCC rule to the H-SMF by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The PCC rule includes the inter-PLMN EAS relocation indication and target DNAI (if the target DNAI is provided by the AF).

3.	When H-SMF receives the policy information (including inter-PLMN EAS relocation indication), the H-SMF authorizes the request for HR-SBO based on SM subscription data (i.e. HR-SBO authorization indication) and retrieves the VPLMN Specific Offloading Policy from H-PCF as described in clause 6.7.2.2. H-SMF determines the target DNAI as follows:

-	the H-SMF uses the target DNAI if received in step 2.

-	If no target DNAI has been received, the H-SMF determines the target DNAI based on the EAS deployment information and IP address of target EAS.

If current (old) V-SMF cannot serve the target DNAI, the H-SMF sends the target DNAI information to old V-SMF by invoking Nsmf_PDUSession_StatusNotify service operation, and old V-SMF sends target DNAI information to the AMF by invoking Nsmf_PDUSession_SMContextStatusNotify service operation. Then the AMF selects a new V-SMF based on the target DNAI  as described in clause 4.23.5.4 of TS 23.502 [3] by replacing I-SMF with V-SMF, SMF with H-SMF.

4.	Based on the inter-PLMN EAS relocation indication and VPLMN Specific Offloading Policy, the H-SMF sends an inter-PLMN relocation information to (new) V-SMF by Nsmf_PDUSession_Update Request. The inter-PLMN relocation information is to indicate the EAS needs to be relocated from HPLMN to VPLMN (target PLMN), and it includes IP range(s) to indicate traffic(s) authorized for traffic routing to target EAS in VPLMN.

5.	The V-SMF selects the V-PSA UPF based on target DNAI and inserts a UL-CL/BP UPF as specified in clause 4.3.5.4 of TS 23.502 [3].  Based on the inter-PLMN relocation information and EAS IP replacement information, the V-SMF configures V-PSA UPF with EAS IP replacement information and configures the UL-CL UPF for traffic routing towards V-PSA UPF if the destination address is the IP address of source EAS.

6.	The V-SMF sends the acknowledgement message to the H-SMF.

7.  In the case of IPv6 multi-homing, the V-SMF notifies the UE the new IP prefix @ V-PSA UPF and may re-configure the UE for the original IP prefix @ H-PSA UPF as specified in step 7 and 8 of clause 4.3.5.4 of TS 23.502[3].

8.  UE interacts with target EAS deployed in VPLMN via V-PSA UPF using IP prefix @ V-PSA UPF (for BP case) or original IP address (for UL/CL case).

NOTE:	Based on the established HR-SBO PDU session, the UE can access EAS(es) deployed in VPLMN and HPLMN simultaneously, so it is possible for the UE to access EAS deployed in HPLMN if the target EAS is relocated to HPLMN again.

END OF CHANGES