3GPP TSG-WG SA2 Meeting #156e	S2-2304665r04

# April 17th – 21st, 2023, Electronic		(was_S2-2302759, 0857)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0662 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID for traffic flow use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-03-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-81 v2.0.0 has documented the conclusion for key issue#9 “Enhancement of NWDAF with finer granularity of location information”. As to the new use cases, the following conclusions are reached: “- NWDAF supports the following new Analytics IDs: - Analytics ID that supports collecting UE location when the UE is exactly in the requested area instead of the translated TA/cell list. - Analytics ID ‘Relative Proximity’ provides support for finer granularity of location information at NWDAF via relative proximity information of nearby Ues. The output analytics contain relative proximity and Time to Collision information. - Analytic ID that supports the traffic flow statistics use case. - Analytic ID that supports location accuracy estimate use case. NOTE 1: That new Analytics IDs to support these use cases are required (instead of enhancing Analytics IDs) will be confirmed during normative work. ” This contribution provides the analytics detail of the traffic flow use case based on the finer granularity location information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Analytics ID for the traffic flow use case in the TS 23.288. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for the “traffic flow” of “Enhancement of NWDAF with the finer granularity of location information” in R18 eNA study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new )* * * *

6.x	Traffic Flow Analytics

6.x.1	General

The Traffic Flow analytics provide analytics information (statistics or predictions) regarding the location, direction and velocity of UEs  during a analytics target periodin a target area.The statistics of traffic flow collects a map of UE movements in a certain region in a certain period of time. In addition, the consumers can obtain the congestion situation in the target area through statistics.

The NWDAF provides traffic flow analytics to an AF.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Traffic flow";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the traffic flow analytics to the provided area. The AOI may be described as shown in clause 5.5 of 3GPP TS 23.273 [39];

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred granularity of location information: "below-cell level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both")"; and

-	Optionally, maximum number of objects.

6.x.2	Input data

The NWDAF collects the UE location and velocity information within the area of interest from the sources listed in Table 6.x.2-1.

Table 6.x.2-1: Data collection by NWDAF for "Traffic flow "

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1 …max) | GMLC | UE locations |
| …>Timestamp |  | Time information associated with the collected information. |
| …>LCS QoS |  | The achieved LCS QoS accuracy (see TS 23.273 [39]) |
| …>Location estimate |  | GAD shape or location coordinates (see TS 23.032 [34]) |
| …>Velocity estimate |  | Speed and direction information |
| …>Indoors/outdoor indication |  | Indicate the location estimate is indoor or outdoor |
| …>Age of location |  | Valid period of the location estimate |

6.x.3	Output analytics

The output analytics of Traffic flow provided by NWDAF is defined in Table 6.x.3-1 and Table 6.x.3-2.

Table 6.x.3-1: Traffic Flow Statistics

| Information | Description |
| --- | --- |
| Applicable Area | A geograohical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users | Total number of users in the area of interest |
| > Average speed | Average speed of all users in the area of interest. |
| > Speed threshold (1...max) | Threshold utilized to fiter the users |
| >> Number of users | The number of users whose speed is faster than the speed threshold |
| >> Ratio | Percentage of users whose speed is faster than the speed threshold |
| > Direction (1…max) | Heading directions of the traffic in the target area, such as north, south, east, west etc. |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |

Table 6.x.3-2: Traffic Flow Predictions

| Information | Description |
| --- | --- |
| Applicable Area | A geograohical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users | Total number of users in the area of interest |
| > Average speed | Average speed of all users in the area of interest. |
| > Speed threshold (1...max) | Threshold utilized to fiter the users |
| >> Number of users | The number of users whose speed is faster than the speed threshold |
| >> Ratio | Percentage of users whose speed is faster than the speed threshold |
| > Direction (1..max) | Heading directions of the traffic in the target area, such as north, south, east, west etc. |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |
| Confidence | Confidence of the prediction |

6.x.4	Procedures

Figure 6.x.4-1 depicts the procedure for traffic flow analytics provided by NWDAF.

Figure 6.x.4-1: "Traffic flow" analytics provided by NWDAF

1.	The NWDAF receives an analytics request from the NWDAF service consumer (as defined in clause 6.1.1.1) or from an AF via NEF (as defined in clause 6.1.1.2) to request traffic flow analytics within a target area for any UE.

The Analytics ID is set to "Traffic flow". The target for analytics reporting is set to be any UE. Analytic filters may be provided as shown in clause 6.x.1. The Area of Interest is the target area that may be defined as a geographical area or as a geopolitical name of an area, in such case, the NEF will translate it as the identities of one or more radio cells or tracking areas.

The Consumer NF can request statistics or predictions or both for a given Analytics target period.

2.	The NWDAF collects input data from AMF as defined in Table 6.x.2-1 via the AMF event exposure service.

3.	The NWDAF collects the UE location information as defined in Table 6.x.2-1 via Ngmlc_Location services.

Based on the analytics target period, the NWDAF may trigger the immediate 5GC-MT-LR procedure as defined in clause 6.1.2 or the defered 5GC-MT-LR procedure in clause 6.3.1 in TS 23.273 [39] to obtain UE’s location information.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics as defined in Table 6.x.3-1 and Table 6.x.3-2 using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe_Notify, depending on the service used in step 1.

# * * * * End of changes * * * *