SA WG2 Meeting #156E (e-meeting)	S2-2304689

April 17 – 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0118 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FQDN detection during EAS Discovery Procedure with Local DNS for HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_ph2 |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In section 6.7.2.4, there is an ENs to describe how to local DNS to identify the FQDN and to do the offloading: Editor's note: It is FFS how the Local DNS server can detect which FQDN(s) are allowed for HR-SBO offloading. Usage of this configuration may not fit with HR-SBO. Due to in agreed CR S2-2303660, in step 4a, if the DNS query does not match the FQDN range eligible for HR-SBO related offload, UPF in VPLMN delivers the DNS query via H-PSA through N9 and H-PSA delivers the DNS query to the DNS server of HPLMN. It means the UPF should have the capability to identify the FQDN. So, one of the possible way to solve this ENs is, the V-SMF configures the detection rule (FQDN or FQDN range) in V-UPF (that can route to local DNS server). The FQDN in DNS query from UE accords with this detection rule will be sent to local DNS server to local handle. The FQDN in DNS query from UE doesn’t accord with this detection rule will be sent to HPLMN to handle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The V-SMF configures the detection rule (FQDN or FQDN range) in V-UPF (that can route to local DNS server). The FQDN in DNS query from UE accords with this detection rule will be sent to local DNS server to local handle. The FQDN in DNS query from UE doesn’t accord with this detection rule will be sent to HPLMN to handle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the FQDNs in DNS query may not be allowed to locally handle in local DNS server, and also local DNS server in VPLMN doesn’t know which FQDN(s) are allowed for HR-SBO offloading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.7.2.4	EAS Discovery Procedure with Local DNS for HR-SBO

Figure 6.7.2.4-1: Procedure for EAS Discovery with local DNS for HR-SBO roaming scenario

EAS Discovery procedure with local DNS are corresponding with the procedure Figure 6.2.3.2.3-1. The steps 1 to 5 are the same as the steps 1 to 6 of Figure 6.2.3.2.2-1 with following differences:

-	SMF is replaced with V-SMF.

-	UE, (R)AN, AMF, ULCL/BP UPF, L-PSA UPF, V-SMF, Local DNS Resolver/Server are located in VPLMN.

-	UPF, H-SMF, C-DNS are located in HPLMN.

0.	The HR-SBO PDU Session is established. See the procedure in clause 6.7.2.2.

1,	ULCL/BP insertion. See the step 1 of the procedure in Figure 6.2.3.2.3.

According to the VPLMN offload policy from HPLMN, the V-SMF configures the traffic detection rule that including FQDNs or FQDN range in V-UPF that can route to local DNS server. The FQDN(s) are allowed for HR-SBO offloading will be detected by the V-UPF and route to local DNS server, and the FQDN(s) are not allowed for HR-SBO offloading will be detected by the V-UPF and route to HPLMN.

2,	After ULCL/BP insertion is performed, the V-SMF sends new local DNS server address to the UE by performing PDU Session Modification procedure as in TS 23.502 clause 4.3.3.3 with following additions:

-	V-SMF sends Local DNS Server/Resolver to the H-SMF in the step 1a of the procedure as in clause 4.3.3.3 of TS 23.502 [3].

-	H-SMF sends the Local DNS Server/Resolver to be sent to the UE via PCO to the V-SMF in the step 3 of the procedure in clause 4.3.3.3 of TS 23.502 [3].

3.	See the step 4 of the procedure in Figure 6.2.3.2.3.

According to the VPLMN offload policy from HPLMN, the V-SMF configures the traffic detection rule that including FQDNs or FQDN range in V-UPF that can route to local DNS server. The FQDN(s) are allowed for HR-SBO offloading will be detected by the V-UPF and route to local DNS server, and the FQDN(s) are not allowed for HR-SBO offloading will be detected by the V-UPF and route to HPLMN.

4.	See the step 5 of the procedure in Figure 6.2.3.2.3.

If the FQDN(s) in DNS query from UE are allowed for HR-SBO offloading, this DNS query will be detected by the V-UPF and route to local DNS server to handle.

If the FQDN(s) in DNS query from UE are not allowed for HR-SBO offloading, this DNS query will be detected by the V-UPF and route to V-EASDF to handle.

5.	See the step 6 of the procedure in Figure 6.2.3.2.3.

* * * End of Change * * *