3GPP TSG-WG SA2 Meeting #156e	S2-2304717r02

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Title:	[DRAFT] LS Reply on Design of RTP Header Extension for PDU Set handling

Response to:	LS (S2-2303974/S4-230419) from SA4

Release:	Release 18

Work Item:	XRM, 5G_RTP

Source:	[Huawei to be] SA WG2

To:	SA WG4

Cc:	RAN WG2

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui DOT ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA4 for the liaison (S2-2303974/S4-230419) on the design of RTP Header Extension for PDU Set handling.

Regarding the questions raised by SA4, SA2 would like to provide following replies:

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

Reply#1: In current release, a single QoS Flow only contains PDU Sets which needs PDU Set based handling. The unmarked PDUs and PDUs marked with PDU Set header will not co-exist in one QoS Flow.

SA2 would like to note that, for PDUs that the application does not want to designate as belonging to a PDU Set (e.g. RTCP PDUs, STUN PDUs), the application needs not to mark them with different service data flow template (e.g. different port number).

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Reply#2: As stated in the previous LS to SA4 (S2-2303849), SA2 kindly asks SA4 to provide guidelines on identification of the Data Burst related information (e.g. the start and or end of data burst, different Identifier of PDU Sets in a Data Burst) via the 3GPP specific RTP header extension if possible.

Furthermore, SA2 would appreciate guidance on how to detect which PDUs belong to a data burst in scenarios where RTP header extensions are not present and cannot be specified. For example, in case it is possible to configure the UPF to detect which PDUs belong to a data burst in scenarios where RTP header extensions are not present. And if the UPF can detect which PDUs belong to a data burst in scenarios where RTP header extensions are not present, whether the specific RTP header extension for the Data Burst is really needed.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE