3GPP TSG-WG SA2 Meeting #156e	S2-2304717r05

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Title:	[DRAFT] LS Reply on Design of RTP Header Extension for PDU Set handling

Response to:	LS (S2-2303974/S4-230419) from SA4

Release:	Release 18

Work Item:	XRM, 5G_RTP

Source:	[Huawei to be] SA WG2

To:	SA WG4

Cc:	RAN WG2

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui DOT ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA4 for the liaison (S2-2303974/S4-230419) on the design of RTP Header Extension for PDU Set handling.

Regarding the questions raised by SA4, SA2 would like to provide following replies:

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

Reply#1: In current release, a single QoS Flow only contains PDU Sets which needs PDU Set based handling. The unmarked PDUs and PDUs marked with PDU Set header will not co-exist in one QoS Flow.

SA2 would like to note that, for PDUs that the application does not want to designate as belonging to a PDU Set (e.g. RTCP PDUs, STUN PDUs), the application needs to mark them with different traffic characteristics  (e.g. different port number) and mapped  into different QoS flows by UPF.

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Reply#2: As stated in the previous LS to SA4 (S2-2303849), SA2 kindly asks SA4 to provide guidelines on identification of the End of Data Burst indication, and support for the End of Data Burst Indication marked by the application server via RTP header information (e.g. the timestamp, sequence number and "M" bit ) and define the 3GPP specific RTP header extension if the current RTP header cannot provide such data burst information.

In SA2 TR conclusion, there is no agreement about whether it is needed to specify which PDUs belong to a data burst, so the discussion may further considered in future release.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE