3GPP TSG-WG SA2 Meeting #156e	S2-2304717r06

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] LS Reply on Design of RTP Header Extension for PDU Set handling

# Response to:	LS (S2-2303974/S4-230419) from SA4

# Release:	Release 18

# Work Item:	XRM, 5G_RTP

Source:	[Huawei to be] SA WG2

To:	SA WG4

Cc:	RAN WG2

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui DOT ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 thanks SA4 for the liaison (S2-2303974/S4-230419) on the design of RTP Header Extension for PDU Set handling.

Regarding the questions raised by SA4, SA2 would like to provide following replies:

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

Reply#1: Mixing of PDUs marked with PDU Set header extension with unmarked PDUs can happen because in both cases the QoS Flow binding is determined only by the Packet Detection Information in the Packet Detection Rule (refer to TS 23.501 Table 5.8.5.3-1. The Protocol Description which indicates the service protocol used by the service data flow (e.g. H.264/RTP, SRTP) is additional information that is used to enable PDU Set-level marking in the UPF, but is not used for QoS Flows binding.

If there are any DL packets that match the Packet Detection Information of a QoS Flow in a specific Packet Detection Rule (PDR), but do not match the Protocol Description in this PDR, then they will be carried as unmarked packets within the same QoS Flow.

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Reply#2: As stated in the previous LS to SA4 (S2-2303849), SA2 kindly asks SA4 to provide guidelines on End of Data Burst Indication marked by the application server via RTP header information (e.g. the timestamp, sequence number and "M" bit ) or define the 3GPP specific RTP header extension if the current RTP header cannot provide such information to determine End of Data Burst Indication.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE