3GPP TSG-WG SA2 Meeting #156e	S2-2304717r12

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Title:	[DRAFT] LS Reply on Design of RTP Header Extension for PDU Set handling

Response to:	LS (S2-2303974/S4-230419) from SA4

Release:	Release 18

Work Item:	XRM, 5G_RTP

Source:	[Huawei to be] SA WG2

To:	SA WG4

Cc:	RAN WG2

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui DOT ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA4 for the liaison (S2-2303974/S4-230419) on the design of RTP Header Extension for PDU Set handling.

Regarding the questions raised by SA4, SA2 would like to provide following replies:

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

Reply#1: All downlink PDUs arriving on N6 (regardless whether they are subject to PDU Set handling or not) undergo QoS Flow matching which is determined by the Packet Detection Rule (refer to TS 23.501 Table 5.8.5.3-1). Any PDUs that match a specific Packet Detection Rule of a QoS Flow will be carried within the same QoS Flow.

The Protocol Description provided by the application (indicating the service protocol used by the service data flow (e.g. H.264/RTP, SRTP)) has been included in PDR.

So it is impossible for a single QoS flow to include both marked and unmarked PDUs.

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Reply#2: As stated in the previous LS to SA4 (S2-2303849), for Rel-18 SA2 kindly asked SA4 to provide guidelines on End of Data Burst Indication marked by the application server via RTP header information (e.g., the timestamp, sequence number and "M" bit ) or define the 3GPP specific RTP header extension if the current RTP header cannot provide such information to determine End of Data Burst Indication.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE