3GPP TSG-WG SA2 Meeting #156-e	S2-2304784r05

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0127 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 Coordination among SMFs for Common EASDNAI determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Edge_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. update the procedure of “Synchronization of Common EAS/DNAI for SMFs”. 2. resolve the ENs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. make clarification on the procedure of “Synchronization of Common EAS/DNAI for SMFs”. 2. resolve the following ENs: a. Editor’s Note: If and how any of the SMFs can update the common DNAI when UE is moving far away from common DNAI is FFS. Proposal A: any SMF is allowed to update the common DNAI. b. Editor’s Note: how the 5GC determined common EAS/DNAI will be stored in UDR needs further discussion. Proposal B: there are two ways for store the common EAS/DNAI as part of AF traffic influence information in UDR: 1) differentiate the AF provided common EAS/DNAI and 5GC determined common EAS/DNAI, e.g AF_common EAS/DNAI, and 5GC_common EAS/DNAI; 2) using the same IE, e.g. common EAS/DNAI. Because, in case AF provides the common EAS/DNAI, 5GC will use the common EAS/DNAI directly, and will not update the common EAS/DNAI; in case 5GC determined common EAS/DNAI, the common EAS/DNAI may be changed, so it propose 1). c. Editor’s Note: Multiple SMFs support is FFS. Proposal C: Multiple SMFs are allowed, and interact via UDR. d. Editor’s Note: How the UDR locking works is FFS. Proposal D: locking is only for update, but other operations are still allowed, e.g. delete. e. Editor’s Note: Whether more than one UE can be considered in common DNAI selection is FFS. Proposal E: It is up to SMF implementation, the SMF can take multiple UEs it served into consideration. 3. replace PCF with NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common EAS/DNAI selection procedure will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.7, 6.2.3.2.6, 6.2.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 1001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.2.7	Coordination among SMFs for Common EAS/DNAI determination

For the case that the common EAS/DNAI is determined by 5GC and UEs in the set of UEs are served by multiple SMFs, coordination is needed among SMFs for determining the common EAS/DNAI. 5GC determines a common EAS/DNAI and stores it as 5GC determined common EAS/DNAI. The 5GC determined common EAS/DNAI is stored as part of AF traffic influence request information in UDR.

The SMF could be any of the SMFs that serving the UE(s) in the Set of UEs in the figure 6.2.3.2.7-1. Once SMF gets the common EAS/DNAI for the set of UEs, it will not update the common EAS/DNAI in UDR.

Figure 6.2.3.2.7-1: Coordination of Common EAS/DNAI for SMFs

1.	In case a new EAS/DNAI has been determined for the set of UEs, SMF sends the EAS/DNAI, Traffic Correlation ID to NEF by invoking Nnef_CommonEASDNAI_Create Request with an EAS/DNAI included, triggering the NEF to update the 5GC determined common EAS/DNAI for corresponding AF traffic influence request information with the Traffic Correlation ID in UDR.

NOTE Y: If multiple UEs in the UE set are served by the SMF, SMF could take multiple UEs into consideration for determining the common EAS/DNAI.

2. NEF determines AF traffic influence request information based on Traffic Correlation ID, and updates the 5GC determined common EAS/DNAI for corresponding AF traffic influence request information in UDR using the EAS/DNAI provided by SMF.

3. If the update of 5GC determined common EAS/DNAI is successful, a successful response will be sent to NEF.

4. NEF responses to SMF with the 5GC determined common EAS/DNAI.

5. In case the 5GC determined common EAS/DNAI and/or 5GC determined DNAI list is created, then they are notified to SMF(s) serving the set of UE(s) by using Nudr_DM_Notify and Npcf_SMPolicyControl_UpdateNotify. The 5GC detetermined common EAS/DNAI is used as common EAS/DNAI for the set of UEs. SMF(s) may initiate the EAS rediscovery procedure as defined in clause 6.2.3.3 for selecting the new common EAS or EAS corresponding to the new common DNAI.

# * * * * Second change * * * *

6.2.3.2.5	Common EAS discovery for a set of UEs

The following is the procedure for discovery common EAS for set of UEs accessing the same application. Different UEs can be served by different SMFs.

The common EAS IP address for the set of UEs can be provided by AF or determined by 5GC. AF can provide the common EAS IP via AF Traffic influence procedure as defined in clause 4.3.6.2 of TS 23.502 [3], AF may determine the common EAS IP address based on candidate DNAI(s) reported by SMF as described in clause 4.3.6.3 of TS 23.502 [3]. Also, the common EAS IP address can be determined by SMF using EAS discovery procedure, and SMF stores the common EAS IP address in UDR as part of AF traffic influence request information.

Figure 6.2.3.2.5-1: Common EAS discovery for a set of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request selecting the common EAS for a set of UEs accessing the application as identified in the AF Request.

AF may use External/Internal Group ID(s) or a list of UEs or any UE as Target UE Identifier(s) and additionally Spatial Validity Condition to identify the set of UEs for correlated selection of common EAS.

The following information may be included in AF request as defined in clause 5.6.7.1 of TS 23.501 [2]:

-	An EAS Correlation indication may be provided for indication of selecting the same EAS for the set of UEs accessing the same application.

-	A Traffic Correlation ID may be provided for identification of the set of UEs accessing the application identified by the Traffic Description in AF request.

-	A Common EAS to be accessed by the set of UEs may be included in AF request, if it is determined by AF.

-	FQDN(s) may be included which is corresponding to the application traffic identified by Traffic Description in AF request.

-	Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or a UE list or group ID can be provided for defining set of UEs accessing the same EAS.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [3], PCF determines the UEs influenced by the AF Request, and for each UE, based on AF request, PCF creates PCC rule with Traffic Correlation ID and EAS Correlation indication and/or Common EAS and FQDN(s) and sends the PCC rule to the SMF.

2.	Based on steps 1-19 in figure 6.2.3.2.2-1, with the following updates:

In step 9:

If FQDN in Neasdf_DNSContext_Notify Request is corresponding to the application indicated in PCC rule, e.g. the FQDN is included in the FQDN(s) in the PCC rule and if EAS Correlation indication is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accessing the application and determines the UE connects to the common EAS for the set of UEs. If FQDN(s) is included in PCC rule, the SMF can use the FQDN(s) in PCC rule and the FQDN in Neasdf_DNSContext_Notify Request to match the FQDN with the PCC rule, i.e. the matched PCC rule includes the FQDN(s) containing the FQDN.

If the common EAS is not available for the set of UEs, or if the 5GC determined common EAS is available but Edge Relocation is needed for the common EAS:

Steps 10-15 are used for discovering of common EAS. In step 15 SMF(s) coordinate for determining the common EAS IP according to clause 6.2.3.2.7.

Else if the common EAS is available and to be used for the set of UEs:

Steps 10-15 are skipped.

In step 16:

SMF may determine the DNAI based on the common EAS instead of EAS information received from the EASDF in Neasdf_DNSContext_Notify.

In step 17:

SMF sends DNS message handling rule including IP address for the common EAS and the Forwarding Action "Respond directly to the DNS request" for instructing EASDF to return the Common EAS IP address in a DNS response to UE directly.

In step 19:

If received IP address of the common EAS and instructed to respond directly in step 17, EASDF sends DNS response with the IP address of the common EAS to UE.

# * * * * Third change * * * *

6.2.3.2.6	EAS discovery corresponding to Common DNAI for a set of UEs

The common DNAI for the set of UEs can be provided either by AF or determined by 5GC. When the AF determines the common DNAI, the AF provide the common DNAI for the set of UEs via AF Traffic influence procedure as defined in clause 4.3.6.2 of TS 23.502 [3]. The  AF may determine the common DNAI based on candidate DNAI(s) reported by SMF as described in clause 4.3.6.3 of TS 23.502 [3].

When the 5GC determines the common DNAI, SMF determines the common DNAI  and SMF notifies the NEF to store the common DNAI in UDR as part of AF traffic influence request information. In case of multiple SMF(s) serving a set of UEs, the first SMF decides on the common DNAI.

The following is the procedure for discovery EAS corresponding to a Common DNAI for set of UEs accessing the same application.

Figure 6.2.3.2.6-1: EAS discovery corresponding to Common DNAI for a set of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request selecting the common DNAI for a set of UEs accessing the application as identified in the AF Request.

AF may use External/Internal Group ID(s) or a list of UEs or any UE as Target UE Identifier(s) and additionally Spatial Validity Condition to identify the set of UEs for correlated selection of common DNAI.

The following information may be included in AF request as defined in clause 5.6.7.1 in TS 23.501 [2]:

-	An indication of traffic correlation may be provided for indication of selecting the same DNAI (i.e. selecting EAS corresponding to the same DNAI) for the set of UEs accessing the same application.

-	A Traffic Correlation ID may be provided for identification of the set of UEs accessing the application identified by the Traffic Description in AF request.

-	A Common DNAI to be accessed by the set of UEs can be included in AF request, if it is determined by AF.

-	FQDN(s) may be included which is corresponding to the application identified by Traffic Description in AF request.

-	Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or a UE list or group ID can be provided for defining set of UEs accessing the same DNAI.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [3], PCF determines the UEs influenced by the AF Request, and for each UE, based on AF request, PCF creates PCC rule with Traffic Correlation ID and indication of traffic correlation and Common DNAI and FQDN(s) and sends the PCC rule to the SMF.

2.	Based on steps 1~19 in figure 6.2.3.2.2-1, with the following changes:

In step 10:

If FQDN in Neasdf_DNSContext_Notify Request is corresponding to the application indicated in PCC rule, e.g. the FQDN is included in the FQDN(s) in the PCC rule and if indication of traffic correlation is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accessing the application and determines the UE connects to EAS corresponding to the common DNAI for the set of UEs. If FQDN(s) is included in PCC rule, the SMF can use the FQDN(s) in PCC rule and the FQDN in Neasdf_DNSContext_Notify Request to match the FQDN with the PCC rule, i.e. the matched PCC rule includes the FQDN(s) containing the FQDN.

If the common DNAI is not available, or if the 5GC determined common DNAI is to be changed, SMF determines a DNAI, andSMF(s) coordinate for determining the common DNAI according to clause 6.2.3.2.7.

Once the common DNAI is available and to be used, for Option A, SMF provisions EASDF with the information to build EDNS Client Subnet option that refers to a location that is topologically close to the common DNAI; for Option B, SMF provisions EASDF with Local DNS server related to the common DNAI.

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *