3GPP TSG-SA WG2 Meeting #156e 	S2-2304810r06

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: Procedures for SL positioning without NAS under 5GC NF involvement.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to implement Procedures for SL positioning for target UE without NAS connection with located UE.

1. Introduction/Discussion

This paper tries to implement the Procedures for SL positioning for target UE without NAS connection with located UE which mainly focuses on the 5GC NF involvement case. Additionally, this paper also gives out the structure for the Procedures for SL positioning for target UE without NAS connection with located UE.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

### 6.5.2	Procedures for SL positioning for target UE without NAS connection using Located UE with network connection

The procedures for SL positioning for Target UE without NAS connection using Located UE with network connection apply to the following cases:

-	when the Target UE initiates 5GC-MO-LR request to obtain its own absolute location and cannot establish a NAS connection or cannot be served by a Ranging/SL positioning capable LMF

-	when the target UE detects an event configured by the network during an initial 5GC-MT-LR procedure when the Target UE has the NAS connection

Figure 6.5.2-1: Procedures for SL positioning for target UE without NAS connection using Located UE with network connection

1.	The Target UE performs step1~5 of the SL-MO-LR procedure in clause 6.x.z of TS23.273[x] with following adaptations:

-	UE1 is the Target UE that cannot set up a NAS connection or that cannot be served by a Ranging/SL positioning capable LMF, and UE2/…/UEn are Located UEs or SL Reference UEs, whereby at least one of the UE2/…/UEn are in coverage and can be served by a Ranging/SL positioning capable LMF.

2. Target UE sends a SL positioning information request to the Located UE(s) to request for its absolute location. In this request, it may include the routing identifier, the QoS requirement, the Target UE ID (e.g., GPSI).

3.	Located UE(s) determine to proceed the SL-MO-LR with step6~22 in clause 6.x.z of TS23.273[7] with following adaptions:

-	Step 6: In addition to what is stated in step 6, UE1 further determines, based on information obtained in earlier steps, that at least one of the discovered UE2/…/UEn are in network coverage and can be served by a Ranging/SL positioning capable LMF. If so, it requests one of the UE2/…/UEn (i.e. UEx) to initiate a SL-MO-LR on its behalf.

Editor's note:	It is FFS whether and how the SL-MO-LR is performed on behalf of Target UE.

Steps 7 to 21 are performed by UE2/…/UEn (i.e., Located UE) in the respective steps, with the following additional clarifications:

-	Step 8:SL-MO-LR indicates that absolute location is required and may indicate the location calculation assistance is needed. Located UE (UE2/…/UEn) may also report the ranging/SL positioning measurement or estimation result and a deferred routing identifier to LMF via the serving AMF of each Located UE. The LMF uses the received information (i.e., ranging/SL positioning measurement or estimation result with the same deferred routing identifier) to calculate the location of the Target UE.

Editor's note:	It is FFS how each Located UE gets the deferred routing identifier towards to a same LMF.

Editor's note:	It is FFS whether step 10-18 can be skipped or not.

4.	The Located UE(s) sends the SL positioning information provide message including the Target UE location to the Target UE.

Editor's note: It is FFS the SL positioning information request message in step1 and the SL positioning information provide message in step4 are RSPP message or PC5-S message like SL positioning client UE initiated Ranging service invocation in clause 6.6.1.1.

# * * * * End of changes * * * *