3GPP TSG-SA WG2 Meeting #156e 	S2-2304810r06

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: Procedures for SL positioning without NAS under 5GC NF involvement.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to implement Procedures for SL positioning for target UE without NAS connection with located UE.

# 1. Introduction/Discussion

This paper tries to implement the Procedures for SL positioning for target UE without NAS connection with located UE which mainly focuses on the 5GC NF involvement case. Additionally, this paper also gives out the structure for the Procedures for SL positioning for target UE without NAS connection with located UE.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

### 6.5.2	Procedures for SL positioning for target UE without NAS connection using Located UE with network connection

The procedures for SL positioning for Target UE without NAS connection using Located UE with network connection apply to the following cases:

-	when the Target UE initiates 5GC-MO-LR request to obtain its own absolute location and cannot establish a NAS connection or cannot be served by a Ranging/SL positioning capable LMF

-	when the target UE detects an event configured by the network during an initial 5GC-MT-LR procedure when the Target UE has the NAS connection

Figure 6.5.2-1: Procedures for SL positioning for target UE without NAS connection using Located UE with network connection

1.	The Target UE performs step1~2 of the SL-MO-LR procedure in clause 6.x.z of TS23.273[x] with following adaptations:

-	UE1 is the Target UE that cannot set up a NAS connection or that cannot be served by a Ranging/SL positioning capable LMF, and UE2/…/UEn are Located UEs or SL Reference UEs, whereby at least one of the UE2/…/UEn are in coverage and can be served by a Ranging/SL positioning capable LMF.

2. Target UE sends a SL positioning information request to the Located UE(s) to request for its absolute location. In this request, it may include the routing identifier, the QoS requirement, the Target UE ID (e.g., GPSI).

Editor's note: It is FFS if the SL positioning request is sent to each Located UE and what this request includes.

3.	Located UE(s) (i.e., UE2/…/UEn not the UE#1) proceed the SL-MO-LR with step3~15 in clause 6.x.z of TS23.273[7] with following adaptions:

-	Step 4:SL-MO-LR is sent by each Located UE and indicates that absolute location is required and may indicate that the location calculation assistance is needed. A deferred routing identifier may be included along with the SL-MO-LR.

Editor's note: It is FFS if the SL-MO-LR is sent by each Located UE.

-	Step5: the AMF select the LMF taking the deferred routing identifier (i.e., to select same LMF for different Located UE)

-Step13: The LMF uses the received SL measurement data/result and the Located UE position in step12 to calculate the location of the Target UE.

Editor's note:	It is FFS how each Located UE gets the deferred routing identifier towards to a same LMF.

4.	The Located UE(s) sends the SL positioning information provide message including the Target UE location to the Target UE.

Editor's note: It is FFS the SL positioning information request message in step1 and the SL positioning information provide message in step4 are RSPP message or PC5-S message like SL positioning client UE initiated Ranging service invocation in clause 6.6.1.1.

Editor's note: It is FFS whether and how to proceed the deferred MT-LR.

# * * * * End of changes * * * *