3GPP TSG-WG SA2 Meeting #156-e	S2-2304864r01

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0349 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location service continuity between EPS and 5GS (bi-direction) for deferred MT-LR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an EN in the TS for the location service contunity between EPS and 5GS for deferred MT-LR Editor's note: Location service continuity from 5GS to EPS and from EPS to 5GS for deferred location request are FFS. Based on previous discussion, there are two different service contunity scenarios which need to be considered for periodic or triggered MT-LR case: - After AMF/MME just received a deferred MT-LR request, UE may handover (i.e., with CONNECTED state) between 5GS and EPS, which means UE has not received the LCS Periodical-Triggered Invoke Request but handover happens. - After UE received the LCS Periodical-Triggered Invoke Request, UE may move between 5GS and EPS (with IDLE sate or CONNECTED state), which means UE has reveived related information (e.g., including LDR reference number and H-GMLC address) for peridodical or triggered event report. For the case 1), the principle in clause 6.19.1 for Immediate Location Request can be reused, i.e., the AMF or MME in the source system can notify GMLC to re-initiate a deferred MT-LR session in the target system. For the case 2), the principle in TR conclusion can be used, particularly for multiple QoS class case: - The LMF should determine whether UE can move between 5GS and EPS and notify UE the corresponding location QoS can be applied to EPS. - After the location event is triggered, the UE should determine and send the suitable event triggered message and carry the suitable location QoS which can be applicable to EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new procedure clause to define the Location service continuity between EPS and 5GS (bi-direction) for defferred MT-LR request as per the conclusion. Remove the EN in clause 6.19.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location service continuity for defferred MT-LR request between EPS and 5GS (bi-direction) is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19.0, 6.19.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.19	Location Service Continuity between EPS and 5GS

6.19.0	General

Clause 6.19.1 describes the location service continuity procedure from 5GS to EPS for Immediate Location Request. Clause 6.19.2 describes the location service continuity procedure from EPS to 5GS for Immediate Location Request. Clause 6.19.x describes the location service continuity procedure for deferred MT-LR and also includes the 5GS to EPS case and EPS to 5GS case. GMLC in can be a single co-located EPC-GMLC and 5GC-GMLC or a separate EPC-GMLC and 5GC-GMLC with the non-standardized Lr' interface.

# * * * * Second change * * * *

6.19.x	Location Service Continuity between EPS and 5GS (bi-direction) for deferred MT-LR

This clause shows the procedure to support location service continuity between EPS and 5GS (bi-direction) for deferred MT-LR. Clause 6.19.x.1 shows the procedure for 5GS to EPS handover case and clause 6.19.x.2 shows the procedure for the EPS to 5GS handover case.

6.19.x.1 Location Service Continuity from 5GS to EPS

Figure 6.19.x-1: LCS Continuity Solution for 5GS to EPS Mobility

1.	Same as step 1 of deferred 5GC-MT-LR procedure in clause 6.3.1.

NOTE 1:	Multiple QoS class is supported in 5GS but not in EPS, if the deferred LCS service request in this step contains multiple location QoS, QoS mapping is performed in step 3a or on UE to support service continuity in EPS.

2.  Target UE positioning starts in 5GS with steps 2~15 of deferred 5GC-MT-LR procedure in clause 6.3.1.

-	If handover happens before step 16 in clause 6.3.1, the UE has not received the LCS Periodic-Triggered Invoke Request message. After handover complete, the AMF notifies 5GC-GMLC. The 5GS to EPS handover notification also includes the MME ID for the target UE. The 5GC-GMLC sends the location request with the mapped location QoS to EPC-GMLC to trigger the deferred/periodic MT-LR in EPS (i.e., steps 3-11 in clause 6.19.1 are performed with the enhancement in step 8 that 5GC-GMLC perform location QoS mapping, then obtain and forward the mapped location QoS which can be applicable to EPS to EPC-GMLC, the mapping may be performed by choosing the most stringent values from the "LocationQoS" and set the "LcsQoSClass" in "LocationQoS" as the "Best Effort"), and steps 3a-9 are not performed;

-	If handover or UE movement with IDLE state happens after step 16 in clause 6.3.1, the UE has received the LCS Periodic-Triggered Invoke Request message. The following steps 3a-9 are performed;

3a. [Conditional] In the periodic or triggered 5GC-MT-LR case, if the LCS QoS class in location request is "multiple QoS class" and the LMF determines the access type allowed for the UE for event reporting includes "E-UTRAN connected to EPC", the LMF may determine the mapped location QoS which can be applicable to EPS based on the original multiple QoS. The mapping may be performed by choosing the most or least stringent values from the "LocationQoS" and set the "LcsQoSClass" in "LocationQoS" as the "Best Effort" or ‘Assured’ based on operator’s policy.

3b~3c. [Conditional] In the deferred periodic or triggered 5GC-MT-LR case, LMF sends the LCS Periodical-Triggered Invoke Request to UE. If step 3a is performed, besides the multiple location QoS, the request also includes the mapped location QoS used for EPS. LDR reference number and 5GC/EPC (H)GMLC address are also transferred to UE in step 3b. UE response to LMF with the LCS Periodical-Triggered Invoke Return Result.

4.	[Conditional] LMF sends the Nlmf_Location_DetermineLocation Response to AMF.

5. 	[Conditional] UE may stay in 5GC over a period of time and the deferred 5GC-MT-LR procedure can continue in 5GS.

6.	[Conditional] 5GS to EPS handover happens or UE moves to EPS with IDLE state.

7-8. [Conditional] For the deferred periodic or triggered MT-LR case, when the event is detected by UE in EPS, the UE shall send an LCS MO-LR Invoke message for event report to the MME. If the location QoS is multiple QoS in 5GS, the UE shall include the mapped location QoS which is applicable to EPS in the LCS MO-LR Invoke message. The UE may use the location QoS values and QoS class based on received mapped QoS in step 3a if the mapping is performed by LMF or UE perfoms the mapping by selecting the most stringent location QoS value and set the "LcsQoSClass" as "Best Effort" by default if it does not receive mapped QoS from LMF in step 3.

9. [Conditional] For the deferred periodic or triggered MT-LR case, the procedure continues with step 15-23 as described in Figure 9.1.19.1-1 of TS 23.271 [4]

10-11. [Conditional] EPC (H)GMLC sends LCS Service Response to 5GC (H)GMLC and 5GC (H)GMLC performs the step 30 of Figure 6.3.1-1.

NOTE 2:	If UE detects handover is not complete but the event is triggered, i.e. step 8 happens before handover complete, the UE waits for handover complete to send the triggered LCS MO-LR Invoke message.

# * * * * Second change * * * *

6.19.x.2 Location Service Continuity from EPS to 5GS

Figure 6.19.x.2-1: LCS Continuity Solution for EPS to 5GS Mobility

1.	Target UE positioning starts in EPS with steps 1-16 of deferred EPC-MT-LR procedure for Periodic and Triggered Location procedure in Figure 9.1.19.1-1 of TS 23.271[4].

-	If handover happens before step 9 in clause 9.1.19.1 of TS 23.271, the UE has not received the LCS Periodic-Triggered Event Invoke message. After handover complete, the MME notifies EPC-GMLC. The EPS to 5GS handover notification also includes the AMF ID for the target UE. The EPC-GMLC sends the location request to 5GC-GMLC to trigger the deferred/periodic MT-LR in 5GS (i.e., steps 3-8 in clause 6.19.2-1 of are performed), and steps 2-5 are not performed;

-	If handover or UE movement with IDLE state happens after step 9 in clause 9.1.19.1, the UE has received the LCS Periodic-Triggered Event Invoke message which transferred LDR reference number and 5GC/EPC (H)GMLC address are to UE. The following steps 2-5 are performed;

2.	[Conditional] 5GS to EPS handover happens or UE moves to EPS with IDLE state.

3-5.  [Conditional] For the deferred periodic or triggered MT-LR case, when the event is detected by UE in 5GS, the UE should send an Event Report message to the LMF through AMF.

For the deferred periodic or triggered MT-LR case, the procedure continues with step 26-29 as described in clause 6.3.1.

6-7. [Conditional] 5GC (H)GMLC sends LCS Service Response to EPC (H)GMLC and EPC (H)GMLC sends the response to LCS client.

NOTE 1:	If UE detects handover is not complete but the event is triggered, i.e. step 3 happens before handover complete, the UE waits for handover complete to send the event report message.

# * * * * End of changes * * * *