SA WG2 Meeting #S2-156E	S2-2304871

17 - 21 April 2023, Electronic, Elbonia	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0231 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN on the MOCN enhancement for the location dependent MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following ENs need to be resolved: Editor’s Note: For location dependent broadcast service, whether the Associated Session ID is sufficient for the NG-RAN to identify multiple broadcast MBS Sessions via different CNs delivering the same content is FFS. Editor´s notes: How to support multiple broadcast MBS Sessions via different CNs to deliver the same content for location-dependent MBS sessions is FFS. In the last meeting, the S2-2302822 and S2-2302976 are proposing to remove the EN about “How to support multiple broadcast MBS Sessions via different CNs to deliver the same content for location-dependent MBS sessions is FFS.” But there is no consensus in the last meeting. The Discussion papers S2-2304870 and S2-2305362 further analyze the the issue and conclude that no signalling extensions are required. The CR reflects the proposal |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN and add Notes to clarify, this is up to RAN implementation Adding a bulletin to descibe the Location dependent MBS session for the MOCN enhancement. Modify the procedure to reflect this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN on the MOCN enhancement for the location dependent MBS session is not resoved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5, 6.18, 7.3.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.5.5	Associated Session ID

In the case of network sharing, an Associated Session ID may be used as specified in clause 6.17. When the AF creates multiple broadcast MBS Sessions via different CNs to deliver the same content, it may provide the Associated Session ID which enables the NG-RAN to identify the multiple MBS Sessions delivering the same content.

Source Specific IP Multicast Address specified in clause 6.5.3 may be used as Associated Session ID.

Next CHANGE

## 6.18	Resource sharing across broadcast MBS Sessions during network sharing

In network sharing scenario as specified in clause 5.18 of TS 23.501 [2], the same MBS broadcast service may be delivered via multiple operators' CN participating in the network sharing to a shared NG-RAN, and the shared NG-RAN nodes may broadcast the MBS data only once for resource efficiency.

When the AF creates multiple broadcast MBS sessions via multiple CNs to deliver the same content, the shared NG-RAN allocates radio resource for one of broadcast MBS Sessions instead of allocating radio resource for all the broadcast MBS Sessions.

NOTE 1:	The same QoS requirements are assumed to be provided by the AF for the broadcast MBS Sessions via multiple CNs delivering the same content.

The NG-RAN determines the broadcast MBS sessions delivering the same content in one of the following ways:

-	Based on the Associated Session ID (see clause 6.5.X) provided by the AF to the NG-RAN via 5GCs when creating broadcast MBS sessions. or

NOTE 2:	For the location dependent MBS session (see Clauses 6.2.3),, the different PLMNs use the different Area session IDs for the same service area. How the shared NG-RAN can determine multiple broadcast MBS Sessions via different CNs to deliver the same content for location-dependent MBS session is up to RAN implementation, e.g. According to Associated area session ID + Service area information of area session.

-	Based on the association of MBS session identifiers (i.e. TMGIs) configured in NG-RAN, the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting the same content for the same MBS service. For the location dependent MBS session, the existing MBS session identifiers are used to identify multiple broadcast MBS Sessions via different CNs delivering the same content.

NOTE 3:	 For the location dependent MBS session (see Clauses 6.2.3),, NG-RAN can also use Service area information of area session to identify multiple broadcast MBS Sessions via different CNs delivering the same content.

NOTE 4:	One possibility for configuring the association between TMGIs is configuring a range of TMGIs for each PLMN participating in network sharing, and associating TMGIs with the same offset in the ranges. Example for TMGI range mapping: If the PLMN 1 TMGI range x1 to xn is configured to be mapped to TMGI range y1 to yn in PLMN 2, x1 is mapped to y1, x2 is mapped to y2, and so forth.

Editor's note:	For the association of MBS session identifiers (i.e. TMGIs) configured in NG-RAN, it is FFS whether AFs can provide additional information (e.g. TMGI index) to request the allocation of a TMGI from a range of TMGIs for shared MBS services in an MB-SMF.

NOTE 5:	When the association of MBS session identifiers is configured in NG-RAN, there is no requirement on the AF to provide an Associated Session ID.

-	For the location dependent MBS session (see Clauses 6.2.3),, the same location dependent service area is mapped to the same shared cells in each PLMN which share the same node. Besides the shared cells, the same location dependent service area may also contain the non-shared cells in each PLMN participating the RAN sharing.

NOTE X:	Different Area session IDs will be used by the different core networks for the same service area.

Illustrated in Figure 6.18-1 is an example that the AF creates broadcast MBS Sessions via 5GC Operators A, B and C respectively to deliver the same content and N3mb unicast transport is used from 5GC to the NG-RAN. Based on operator policy in the NG-RAN node, the N3mb tunnel may be established from the 5GC of only one operator (i.e., Operator A in Figure 6.18-1) to the shared NG-RAN, or the N3mb tunnels may be established from the 5GCs of all the operators to the shared NG-RAN. Over the Uu interface, the NG-RAN allocates radio resource for only one of the established broadcast MBS Sessions regardless of the number of N3mb tunnels established to deliver the MBS packets.

Figure 6.18-1: Example of Resource sharing across multiple broadcast MBS Sessions via different CNs to deliver the same content during network sharing

For a location dependent MBS session (see Clauses 6.2.3), the AF(s) that create the location dependent area sessions towards the participating PLMNs shall supply service areas that contain the same shared radio cells (but may contain different non-shared radio cells).

Next CHANGE

### 7.3.1a	MBS Session Start for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions during network sharing (see clause 6.17), in addition to clause 7.3.1, the following enhancement applies:

-	Step 2: The MB-SMF includes Associated Session ID in the N2 SM information in Namf_MBSBroadcast_ContextCreate.

-	Step 4: A shared NG RAN node identifies associated MBS sessions via a configured association of TMGIs or via the same Associated Session Identifier being received for the associated sessions. The associated MBS sessions shares radio resource configuration based on the existing Broadcast MBS Session context. For the location dependent MBS session (see Clauses 6.2.3 and 7.3.4), NG-RAN identifies multiple broadcast MBS Sessions via different CNs to deliver the same content for location-dependent MBS session as described in clause 6.18

Editor's note:	Additions with respect to N3mb tunnel management is FFS.

-	Step 5: If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content based on the Associated Session ID, the NG-RAN may skip joining the multicast group based on operator policy, i.e., step 5 can be skipped.

-	Step 9: If the NG-RAN determines that radio resource has been allocated for another broadcast MBS Session delivering the same content based on the Associated Session Identifier or the configured association of TMGIs, the NG-RAN advertises the TMGI of the broadcast MBS session, and reuses such already allocated radio resource for the MBS data transmission for the broadcast MBS session.

NOTE:	The 5G QoS Profile values for the broadcast MBS sessions are assumed to be pre-agreed amongst the multiple operators participating in the network sharing, and are the same irrespective of the PLMN from which the MBS Session Start is sent.

-	Step 10 ~ step 12:

The handling is the same as steps 5, 6, 7 and 8.

-	Step 14: If the user plane of a broadcast MBS session from its MB-UPF to the shared NG-RAN is not established, the NG-RAN will not receive the MBS packets from the MB-UPF.

-	Step 15: If the NG-RAN receives multiple DL media streams for the same content represented by the same Associated Session ID via multiple CNs, the NG-RAN should deliver only one DL media stream.

END OF CHANGES