3GPP TSG-WG SA2 Meeting #156-e	S2-2304890r02

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0790 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the End-to-end data volume transfer time analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Application Server Instance can represent a specific application server. As listed in Table 6.4.2-1 of TS 23.288, the Application Server Instance can be collected from AF, and it can be represented by the IP address of the Application Server. Therefore, the following EN can be removed. Editor's note: Which AF event can be used to collect Server location is FFS. It should be clarified that the validity period in output analytics is the subset of corresponding time slot. The E2E data volume transfer time UL/DL should indicate the time over the corresponding time slot, not Analytics target period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the following EN: Editor's note: Which AF event can be used to collect Server location is FFS. Clarify that the validity period is the subset of corresponding time slot. Clarify that the E2E data volume transfer time UL/DL indicates the time over the corresponding time slot. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN for collecting Server location still exists. Unclear descriptions for End-to-end data volume transfer time analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.2, 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.18.2	Input Data

The NWDAF supporting analytics on E2E data volume transfer time shall be able to collect E2E data volume transfer time information from AF, OAM and 5GC NFs.

More detailed information collected by the NWDAF from the OAM is defined in the Table 6.18.2-1, from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.18.2-2, and from AF is defined in Table 6.18.2-3.

Table 6.18.2-1: Input data from OAM related to E2E data volume transfer time

| Information | Source | Description |
| --- | --- | --- |
| RAN part delay | OAM TS 28.554 [10] | Average packet transmission delay through the RAN part to the UE, per timeslot, per 5QI and per S-NSSAI. |
| Timestamp | OAM | A time stamp associated with the collected information. |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per QCI per UE packet delay as specified in clause 5.2.1.1 of TS 37.320 [20] and per DRB per UE packet delay as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per QCI per UE packet loss rate as specified in clause 5.2.1.1 of TS 37.320 [20] and the per DRB per UE packet loss rate as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| Distribution of UL/DL/round trip packed delay between PSA UPF and UE | OAM | The distribution of UL/DL/round trip packet delay between PSA UPF and UE as captured in TS 28.552 [8]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.18.2-1 by using the services provided by OAM as described in clause 6.2.3.

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s)).

NOTE 2:	User consent checking from UDM can apply for these analytics.

Table 6.18.2-2: Service Data from 5GC NFs for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | Location of the UE(s) needs to be selected via AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be selected via GMLC instead. |
| UE ID | AMF | (list of) SUPI(s). |
| 5QI | SMF | A reference to 5G QoS characteristics. |
| QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions. |

Table 6.18.2-3: Service Data from AF for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AF | Timestamp of the collected information. |
| Application ID | AF | Identifier of the application at the AF. |
| UE ID(s) | AF | Internal or External UE IDs (i.e. SUPI or GPSI, respectively). |
| Transmitted data volume | AF | The volume of the transmitted data. |
| Transmission time duration | AF | The time duration (start and end time) needed for sending a volume of data. |
| Application Server Instance information | AF | The IP address of the Application Server that the UE(s) had a communication session when the measurement was made. |

# * * * * Next change * * * *

### 6.18.3	Output Analytics

The NWDAF supporting E2E data volume transfer time analytics provides the analytics results to consumer NFs, e.g. AF, or NEF. The analytics results provided by the NWDAF could be E2E data volume transfer time statistics as defined in Table 6.18.3-1 or predictions as defined in table 6.18.3-2.

Table 6.18.3-1: E2E data volume transfer time statistics

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the statistic applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer time information (1…max) | List of E2E data volume transfer time statistics per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> UE location | Indicating the UE location information when the UE service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the Application. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time statistics as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time statistics applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time UL indicators. Statistics of E2E data volume transfer time for uplink over the time slot (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Statistics of E2E data volume transfer time for downlink over the time slot (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL. |
| > Classified E2E data volume transfer time information for a list of UEs (NOTE 1) | Classified E2E data volume transfer time statistics for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time statistics as defined in clause 6.1.3. |
| >> Spatial Validity Condition | Area where the E2E data volume transfer time statistics applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Table 6.18.3-2: E2E data volume transfer time predictions

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the statistic applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer time information (1…max) | List of E2E data volume transfer time prediction per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot. |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> UE location | Indicating the UE location information when the UE service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the Application. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time prediction as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time prediction applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time indicators. Predictions of E2E data volume transfer time for uplink over the time slot (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Predictions of E2E data volume transfer time for downlink over the time slot (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL |
| > Classified E2E data volume transfer time information for a list of UEs (NOTE 1) | Classified E2E data volume transfer time prediction for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
|  | Ratio of UEs. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time prediction as defined in clause 6.1.3. |
| >> Spatial Validity Condition | Area where the E2E data volume transfer time prediction applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

NOTE:	When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics transfer time result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

# * * * * End of changes * * * *