3GPP SA WG2 Meeting #156E	S2-2304945r01

April 17 - 21, 2023, Elbonia                  	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0272 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on 5G ProSe UE-to-Network Relay Reselection and Path Switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G ProSe UE-to-Network Relay reselection and path switching between two Relays are related functions and procedures. This CR is submitted to clarify the procedures for 5G ProSe UE-to-Network Relay reselection and path switching. The procedure for path switching between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-3 UE-to-Network Relay with N3IWF is not addressed in clause 6.5.5.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the procedures for 5G ProSe UE-to-Network Relay reselection and path switching; and clarify that for path switching between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the application layer procedures are used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sequence and relation of 5G ProSe UE-to-Network Relay reselection and path switching is not clear. The procedure for path switching between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-3 UE-to-Network Relay with N3IWF is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3, 6.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.5.3	5G ProSe UE-to-Network Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE keeps performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-Network Relay for relay reselection.

The 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 are used for 5G ProSe UE-to-Network Relay reselection to discovery available 5G ProSe UE-to-Network Relays. The principal for 5G ProSe UE-to-Network Relay reselection is described in clause 5.15.

SECOND CHANGE

6.5.5.2	Procedures for Communication Path Switching between UE-to-Network Relays

This clause specifies the communication path switching procedures between two UE-to-Network Relays.

When performing the path switching between two 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, or between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-2 UE-to-Network Relay, or between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the application layer procedures are used for service continuity support which is out of scope of this specification.

Path switching between two 5G ProSe Layer-3 UE-to-Network Relay with N3IWF is performed using the mechanism specified for Local mobility anchor within untrusted non-3GPP access networks using MOBIKE in clause 6.2.9 of TS 23.501 [4].

When a 5G ProSe-enabled UE as a 5G ProSe Layer-2 Remote UE in CM-CONNECTED state switches the communication path to Layer-3 UE-to-Network Relay with N3IWF, 5G ProSe-enabled UE's PDU session(s) can handover to the indirect path via Layer-3 UE-to-Network Relay with N3IWF by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

When a 5G ProSe-enabled UE as a 5G ProSe Layer-3 Remote UE switches the communication path from Layer-3 UE-to-Network Relay with N3IWF to an indirect path via Layer-2 UE-to-Network Relay, 5G ProSe-enabled UE's PDU session(s) can handover to the indirect path via Layer-2 UE-to-Network Relay by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

Editor's note:	The procedures for Layer-2 Remote UE in CM-CONNECTED state switching the communication path to another Layer-2 UE-to-Network Relay will be aligned with RAN WG specifications.

END OF CHANGES