3GPP TSG SA WG2 Meeting #156E	S2-2304968r02

# April 17 - 21, 2023, Elbonia														(Revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1301 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Bootstrap and application data channel setup procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-03-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The annex for IMS data channel needs to be updated on data channel signalling procedure for bootstrap and application data channel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedures of boostrap DC and application DC are updated to be clearer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

AC.2.2	Functional entities

AC.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores the verified data channel applications which are retrieved by the DCSF when required.

**** Second Change ****

AC.7	Data Channel Procedures

AC.7.1	Bootstrap Data Channel Setup Signalling procedure

Figure AC.7.1-1depicts a signalling flow diagram for establishing a bootstrap data channel in a person-to-person use case. The DCMF anchors the bootstrap data channel, and the originating network is offering a bootstrap data channel to the remote peer as well for application download.

In the call flow the two UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

NOTE 1:	Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

# Figure AC.7.1-1: Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains offers for the bootstrap data channel establishment requirement with bootstrap DC stream ID. In this example procedure, the SDP contains both bootstrap data channel offers for originating side and terminating side.

NOTE 2:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup.

2.	IMS AS validates user subscription data to determine whether the data channel call request should be notified to DCSF.

If the IMS AS determined, based on the user profile, that data channel call request needs to be notified to DCSF, the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS determined, based on the user profile, that the data channel call request need not to be notified to DCSF, or DCSF decides that DC request is not allowed, the IMS AS proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel bootstrapping, by deleting DC related media information and sending the updated SIP INVITE to the originating S-CSCF.

3.	IMS AS notifies the DCSF of the call event sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, CallingID, CalledID, SessionCase, Event initiator, MediaInfoList service operation to the DCSF.

4.	After receiving the DC control request, the DCSF determines the policy about how to process the bootstrap data channel establishment requirement based on the related parameters in the Data Channel control request and/or operator strategy.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on its policies reserves originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE), which are used to receive UE request for application downloading from DCMF or enhanced MRF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (SessionID, MediaInstructionSet) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with DCMF both for originating and terminating side. The MediaInstructionSet provided by the DSCF, includes  its MDC1 media endpoint addresses created in step 5 and the replacement HTTP URL representing the application list offered via the MDC1 interface.

In this scenario, the DCSF instructs the IMS AS to terminate bootstrap data channel establishment requirement on originating DCMF, and initiate remote bootstrap (targeting remote UE) as well as forwarding remote bootstrapping of served user (targeting remote DCSF) establishment towards terminating network..

7.	The IMS AS selects a DCMF based on local configuration or discovers and selects a DCMF instance or enhanced MRF supporting DC media function via NRF.

8.	IMS AS invokes Ndcmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct DCMF to allocate required media resources. IMS AS request creation of two different Media Terminations, one representing the local bootstrap media to be terminated and the other representing the remote bootstrap media to be offered to remote UE. Each Media Termination includes information required to allocate resources in both Mb and the MDC1 interfaces.  The DCMF responds with the reserved data channel media resource information to IMS AS. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

9.	IMS AS invokes Ndcmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct DCMF to allocate a Mb resource to terminate the media descriptor specified by the DCSF for the terminating side. The DCMF responds with the reserved data channel media resource information to IMS AS. For If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources. IMS AS also includes DCSF MDC1 media endpoint address for terminating side and the replacement HTTP URL received from DSCF in step 7.

NOTE 3:	The DCMF media resource allocation in step 8 and 9 could be done with one or multiple service invocations.

10.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and includes also MDC1 media resource information on DCMF received from DCMF.

11.	The DCSF stores the media resource information and responds to the Notify Request received in step 3.

12-14.	IMS AS sends the INVITE which includes the updated SDP offer adding media information of DCMF or enhanced MRF via the originating S-CSCF to remote network side and UE#2. In this scenario, the SDP offer for bootstrap data channel to UE#2 is included.

15.	UE#2 and terminating network returns an 18X response with the SDP answer to bootstrap data channel to originating network.

According to the received SDP answer, DCMF or enhanced MRF may update data channel media resource for UE#2.

16-17:	UE#2 and terminating network returns a 200 OK response.

18.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, SessionID, MediaInfoList) to DCSF.

19.	The DCSF responds to the notification.

20.	200 OK forwarded to UE#1 which indicates the bootstrap data channel has been established.

21-24:	The bootstrap data channels have been established between originating DCMF or MRF and UE#1/UE#2. The UEs send application request messages to DCMF or enhanced MRF to request a data channel application or an application list if multiple DC applications are available, via bootstrap data channel with its data channel capabilities. The DCMF or MRF replaces the root URL with the replacement URL received in steps 8 and 9 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 and UE#2 based on their data channel capabilities and their choices through DCMF or MRF.

The bootstrap data channels have also been established between terminating DCMF or MRF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF

AC.7.2	Application Data Channel Setup Signalling Procedure

AC.7.2.1	Person to Person (P2P) Application Data Channel Setup

Figure AC.7.2.1-1 depicts a signalling flow diagram for establishing an application data chanel in a person-to-person use case. In this scenario, the DCMF is not used to anchor the Application data channel.

In the call flow the UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

# Figure AC.7.2.1-1: Person-to-Person Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.	IMS session and bootstrap data channel have been established. Data channel applications have been downloaded to UE#1 and UE#2.

1.	UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the bootstrap data channel information, as well as the application data channel establishment requirement per TS 26.114 [76].

2.	The IMS AS validates user subscription data to determine whether the media change request event should be notified to the DCSF.

3.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (MediaChangeRequest Event, Session ID, Event Direction, Event initiator, Media Info List) of the media change request event.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the notification and/or operator policy.

5.	The DCSF determines that the added application data channel media descriptor of the SDP offer takes UE#2 as target endpoint and does not require anchoring on the local DCMF or enhanced MRF. If DCMF or enhanced MRF needs to anchor application data channel, DCSF would have used the Nimsas_MediaControl service operation to instruct IMS AS to allocate data channel media resources of the DCMF or enhanced MRF.

6.	DCSF responds to the notification received in step 3.

7-8.	IMS AS sends the re-INVITE to the originating S-CSCF and then to the terminating network side and UE#2.

9-11.	UE#2 and terminating network returns a 200 OK response with SDP answer for application data channel to originating network.

12.	IMS AS notifies the DCSF of the successful data channel modification.

13.	DCSF responds to the notification.

14-15.	The IMS AS sends 200 OK response to the originating S-CSCF and P-CSCF.

16.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

17.	P-CSCF returns the 200 OK response to UE#1.

18.	UE#1 sends ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established. In this example, it is not anchored in DCMF/ MRF.

AC.7.2.2	Person-to-Application (P2A) Application Data Channel Setup

Figure AC.7.2.2-1 depicts a signalling flow diagram for establishing an Application Data Chanel in a person to application use case.

# Figure AC.7.2.2-1: Person-to-Application (P2A) Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0-3.	Steps 0-3 of clause AC.7.2.1 applies.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the notification and/or operator policy.

5.	The DCSF determines that the added Application Data Channel media of the offer takes DC Application Server as target endpoint and requires to anchor in the DCMF or MRF.

6.	The DCSF invokes Nimsas_MediaControl service operation to instruct IMS AS to terminate the media flow of the originating UE to DCMF. The instruction also includes information to be consumed by the DCMF that the data channel media shall be relayed via the MDC2 interface.

7.	The IMS AS invokes Ndcmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct DCMF on application data channel establishment and data channel media resource reservation based on the DC media information received from DCSF. If enhanced MRF is used, IMS AS uses Mr'/Cr to enhanced MRF to reserve data channel media resources.

8.	The IMS AS notifies the MediaControl instruction control response to DCSF.

9.	The DCSF stores the media resource information and sends a P2A application data channel establishment request (including the MDC2 SDP offer received from DCMF) to the DC Application Server via DC3/DC4.

10.	DC Application Server accepts the P2A application data channel establishment request, returning an MDC2 SDP answer and is prepared for UE#1 traffic through MDC2.

NOTE:	Details on how DCSF communicates with the DC Application Server is out of scope of this Release.

11.	DCSF requests IMS AS to update the DCMF resource with MDC2 media endpoint information of DC Application Server.

12.	IMS AS updates the DCMF resource.

13.	IMS AS notifies the MediaControl instruction control response to DCSF.

14.	DCSF replies to the Nimsas notification request.

15-16.	IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, and which does not include the SDP for the application data channel.

17-19:	UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

20.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

21.	DCSF replies to the notification.

22-23.	The IMS AS includes SDP answer for application data channels to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

24.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

25.	CSCF returns the 200 OK response to UE#1.

26.	UE#1 send ACK to the terminating network.

27.	The application data channel between UE#1 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#1 and DC Application Server based on MDC2 media point information received in step 9 and 12.

AC.7.3	Person-to-Application and Application-to-Person (P2A2P) Procedure

Editor's note:	The relationship between clauses AC.7.3 and AC.7.2 needs further clarification.

This procedure enables originating and terminating UE to establish application data channels for the same application to communicate with the same Data Channel Application Server.

# Figure AC.7.3-1: Symmetric Application Data Channel Establishment

0.	IMS session and bootstrap data channels are established. Data channel applications are downloaded to UE#1 and UE#2.

1.	UE#1 sends SIP re-INVITE request with an updated SDP to IMS AS. The updated SDP contains the bootstrap data channel and the application data channel information.

2.	IMS AS validates the data channel media description information and/or user subscription data to determine whether the DCSF needs to be notified.

3.	IMS AS selects and notifies the DCSF about the call event and data channel establishment request.

4.	The DCSF determines how to process the application data channel establishment request based on the parameters in the notification from DCSF and/or operator policies.

5.	DCSF determines that the added application data channel media in the SDP offer requires the DC Application Server is the endpoint for both originating and terminating UE and that the application DC must be anchored at the DCMF or enhanced MRF. DCSF communicates with the DC Application Server for DC resource control. Once the application data channel is established, the DC Application Server will send/receive traffic to/from UE#1 and UE#2 through the MDC2 interface.

NOTE:	Details on how the DCSF communicates with the DC Application Server are out of scope of Release 18.

6.	DCSF invokes Nimsas_MediaControl service to send data channel control request to IMS AS, including information how to relay data channel media via the MDC2 interface.

7.	IMS AS reserves data channel media resources at the DCMF via DC2 or at the enhanced MRF via Mr'/Cr based on the DC media information received from DCSF.

8.	IMS AS notifies the DCSF about MediaControl instruction control response.

9.	The DCSF stores the media resource information and establishes MDC1 connections with DCMF or enhanced MRF for application DC traffic to UE#1 and replies to the Nimsas notification request.

10-11.	IMS AS sends re-INVITE which include the SDP offer from DCMF or enhanced MRF for the application data channel to the originating S-CSCF and then to the remote network and UE#2. The SDP offer to remote network and UE#2 should include the application data channel establishment information to terminating side.

12-15.	UE#2 and terminating network return 200 OK with SDP answer for audio/video and for the application data channel. The terminating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

16. IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

17.	DCSF replies to IMS AS.

18-19.	The IMS AS includes SDP answer for application data channel to UE#1 in 200 OK and sends 200 OK to S-CSCF and P-CSCF.

20.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

21.	P-CSCF returns the 200 OK to UE#1.

22.	UE#1 sends ACK to the terminating network.

23.	The application data channel between UE#1 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#1 and DC Application Server via MDC2.

24.	The application data channel between UE#2 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#2 and DC Application Server via MDC2.

**** End of Changes ****