3GPP TSG-WG SA2 Meeting #156E	S2- 2305018

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4458 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing ULI of IAB-UE for services using location reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the reply LS(R3-231011) from RAN WG3 on FS_VMR solutions review, the functionality to provide ULI of the IAB-UE to the AMF serving the UE for services is to be supported within Rel-18 as follows. - For point#7 (regarding KI#6): RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the following Editor's note with clarification that the cell ID/TAC of MBSR can be provided for services with the ULI of the IAB-UE to the CN. Editor's note: The description of how to provide the MBSR cell ID/TAC for certain services will be enhanced if needed based on RAN WG work progress. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear whether and how the cell ID/TAC of MBSR is provided for services that infer the UE locations taking IAB-UE ULI into account. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.35A.6	Providing cell ID/TAC of MBSR for services

The TAC and cell ID broadcasted by the MBSR cell(s) are configured as specified in TS 38.470 [165].

If the mobile IAB indication was received during RRC connection establishment, the IAB-donor gNB shall provide the User Location Information for the IAB-UE including NR CGI/TAI based on a cell ID/TAC managed by the IAB-donor gNB to the AMF serving the UE. The AMF serving the UE may take ULI of IAB-UE together with ULI of UE into account to determine the UE location for services that use cell ID/TAC as the location reference (e.g. for Mobility Management, Registration Area management, etc).

* * * * End of Changes * * * *