3GPP TSG-WG SA2 Meeting #156-e	S2-2305080

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4475 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the EN on supporting TSN TN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are still some ENs in TS 23.501: Editor's Note 2: It is FFS if the interface between the SMF/CUC and TN CNC could allow the SMF/CUC to indicate the TEID and QFI of the given QoS Flow to the TN CNC. As defined in IEEE specs, there is no TEID or QFI in the interface between CUC and CNC. Besides, there is no need to transfer such information as 5GS can allocate different addresses/port for different QoS Flow based on implementation. So it is to remove the EN directly. Editor's Note 6: it is FFS if and how the Service Data Flows can be used to identity streams in the TN. The parameters are generated based on QoS Flow but not SDF, no need to mention SDF in N3. It is proposed to remove the EN directly. Editor’s Note 7: How SMF/CUC derives the input information based on the TimeAwareOffset and where this input information is mapped to Gate Control configuration information is FFS. There are 2 options: one is the SMF just transfers the TimeAwareOffset to the Talker (i.e. RAN or UPF) and TL will know the exact window by related information, e.g periodicity, TimeAwareOffset, and EarlistTransmit offset by itself. This option is aligned with Annex U.1.1. The other is SMF/CUC calculates the window by itself and send the related information, e.g. adminbase time directly to the TLs. In the second case, the SMF still needs to provide per stream level information to the TLs. i.e. It is still required to configure the traffic class related information. It is proposed to go with the first option. It is simpler and gives more flexibility to the TLs. To generate the Gate Control Information, the Talker needs to know the MaxFrameSize. (As MaxFrameSize is calculated per interval, the MaxFramesPerInterval can be considered as 1 in this case.) It is proposed to remove the EarlestTransmitoffset and LatestTransmitOffset. Besides, it add some clarifications on the parameters: The value of LatestTransmitOffset subtracted by EarliestTransmitOffset shall be up to Buffer capability. But currently this is described in MaxLatency. It should be also described in LatestTransmitOffset part. And LatestTransmitOffset shall be larger or equal than Earlesr Transmitoffset. MaxFrameSize: the PCF does not transfer the Burst Size of QoS Flow directly to the SMF but generate MDBV based on it. To make it precise, the PCF shall set MDBV equal to the Burst Size if the interworking with a TSN network deployed in the transport network is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the EN and add clarifications as above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Still FFS in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex M.1, 5.27.3, 5.28a.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex M (normative):
Interworking with TSN deployed in the Transport Network

M.1	Mapping of the parameters between 5GS and TSN UNI

Editor's note:	The information provided here is a guidance for better understanding of the function. Content of this Annex will be updated to a functional level detail once stage 3 work is stable.

The details of the parameters in the TSN UNI are specified in IEEE Std 802.1Qcc [95]) and IEEE P802.1Qdj [146].

The SMF/CUC derives the End Station related information for the stream requirements towards the TN CNC for the QoS Flow as follows:

a)	For the Talker group:

‐	StreamID: can be generated by the SMF/CUC based on the End Station MAC address acting as Talker, and PDU Session ID and QFI. The MAC address is either pre-configured at the SMF/CUC or provided by the AN-TL or CN-TL to the SMF/CUC (e.g. as part of the EndStationInterfaces information).

-	StreamRank: set to zero for ARP priority values 1-8; set to one for other ARP values.

‐	EndStationInterfaces: If the AN-TL and CN-TL are supported, the SMF/CUC receives the EndStationInterfaces (MacAddress, InterfaceName) from the AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported the SMF/CUC sets the information based on pre-configuration.

-	DataFrameSpecification (optional): When it is present it specifies how the TN can identify packets of the TN stream in order to apply the required TSN configuration. If the AN-TL and CN-TL are supported, the SMF/CUC does not include the DataFrameSpecification to the TN CNC.

If the AN-TL and CN-TL are not supported, the SMF/CUC derives the DataFrameSpecification based on the N3 tunnel end point addresses/ports that are used for the QoS Flow. The SMF/CUC instruct the UPF and NG-RAN to assign a separate N3 tunnel end point address for each QoS Flow. This ensures the TN can distinguish the QoS Flows based on the N3 tunnel destination IP addresses.

NOTE 1:	IPv6 can be used in the N3 tunnel end point addresses to provide sufficient address space.

If the DataFrameSpecification is provided to the TN CNC, the TN CNC configures the edge bridge to perform the stream transformation based on the provided the DataFrameSpecification

Editor's note:	It is FFS if the interface between the SMF/CUC and TN CNC could allow the SMF/CUC to indicate the TEID and QFI of the given QoS Flow to the TN CNC.

‐	TrafficSpecification elements:

‐	Interval: derived from the Periodicity of the traffic as indicated in the TSCAI.

‐	MaxFramesPerInterval: specifies the maximum number of frames that the Talker transmits in one Interval.

‐	MaxFrameSize: derived from the MDBV of the QoS Flow. If the PCF determines interworking with a TSN network deployed in the transport network is supported based on the DNN/S-NSSAI of the PDU Session, the PCF generates MDBV based on the Burst Size as described in clause 5.27.3 and the PCF transfers the MDBV to the SMF/CUC. The SMF/CUC sets MaxFrameSize based on the following formula: MDBV of the QoS Flow - the framing bits which is not used for transferring in 5GS, (e.g. CRC + the GTP-U tunnel overhead).

-	TransmissionSelection: specifies the algorithm that the Talker uses to transmit the Stream's traffic class. If no algorithm is known, the value zero (strict priority) is used.

‐	TSpecTimeAware group (optional, present only if the traffic in the QoS Flow is time-synchronized):

‐	EarliestTransmitOffset: the earliest offset within the Interval.

For uplink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in UL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UE-DS-TT Residence Time + 5G-AN PDB - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in UL direction + UE-DS-TT Residence Time + 5G-AN PDB > M x Interval duration.

would be true.

For downlink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in DL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UPF Residence Time - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in DL direction + UPF Residence Time > M x Interval duration.

would be true.

‐	LatestTransmitOffset: the last chance within an interval should leave enough time to transfer a packet with MaxFrameSize. Derived from the end of the interval, subtracted by the sum of Jitter and the time to transfer a packet with MaxFrameSize. The value of LatestTransmitOffset shall be larger or equal than EarliestTransmitOffset and the value of LatestTransmitOffset subtracted by EarliestTransmitOffset shall be up to Buffer capabilityof the Talker.

‐	Jitter: derived based on local configuration.

‐	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one (no redundancy) or other value (if redundancy is required).

‐	MaxLatency: set to CN PDB subtracted by maximum possible buffer duration in Talker. Maximum possible buffer duration is set to LatestTransmitOffset subtracted by EarliestTransmitOffset (which is up to Buffer capability sent by the Talker).

-	InterfaceCapabilities (optional): If the AN-TL and CN-TL are supported, the SMF/CUC collects InterfaceCapabilities from AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported, the SMF/CUC leaves the InterfaceCapabilities empty.

b)	For the Listener group:

-	Stream ID and Stream Rank: that were generated for the Talker of the TN stream are also used by the SMF/CUC for the Listener.

-	EndStationInterfaces: derived as with the corresponding information for the Talker group.

-	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one.

‐	MaxLatency: derived as with the corresponding information for the Talker group.

-	InterfaceCapabilities: derived as with the corresponding information for the Talker group.

c)	For the Status group: The Status group contains the end station communication-configuration provided by TN CNC to the SMF/CUC:

‐	Stream ID.

‐	StatusInfo.

‐	AccumulatedLatency: If the AccumulatedLatency is included from TN CNC to SMF/CUC for a stream in DL direction, the SMF/CUC may use the AccumulatedLatency to update the TSCAI BAT to the NG-RAN; the SMF sets the TSCAI Burst Arrival Time in downlink direction as the sum of the TSCAC BAT value in downlink direction and AccumulatedLatency and the buffer duration in Talker in CN-TL. The buffer duration in CN-TL is zero if TimeAwareOffset for the Talker group is not present, and TimeAwareOffset - EarliestTransmitOffset if the TimeAwareOffset is present for the Talker group.

‐	InterfaceConfiguration (optional):

‐	MAC Address (optional, present only if the respective InterfaceCapability contains a value for Active Destination MAC and VLAN Stream identification in CB-StreamIdenTypeList, and stream transformation is performed in AN-TL and CN-TL).

‐	VLAN Tag (optional, present only if the respective InterfaceCapability contains that it is VlanTagCapable and a value for Active Destination MAC and VLAN Stream identification in CB-StreamIdenTypeList, and the stream transformation is performed in AN-TL and CN-TL).

‐	IPv4/IPv6 Tuples (optional, but not supported in this release of the specification).

‐	TimeAwareOffset (optional, present only if the traffic is time-synchronized, AN-TL and CN-TL is supported, and TSpecTimeAware elements were provided in the stream requirements).

If the InterfaceConfiguration is included from TN CNC to SMF/CUC, and if the AN-TL/CN-TL are supported, and AL-TL/CN-TL support the Stream Transformation as described in IEEE Std 802.1Qcc [95], the SMF/CUC can instruct the UPF and NG-RAN to assign for each QoS Flow an individual TSN Transport address by providing the InterfaceConfiguration to the AN-TL/CN-TL via TL-Container. The Talker in AN-TL/CN-TL shall use the indicated InterfaceConfiguration, e.g. source MAC address, multicast destination MAC address, VLAN ID, as assigned by the TN CNC for the data stream in a QoS Flow. The TN can identify the streams based on the Stream Transformation that is applied in the Talker in the AN-TL/CN-TL. This allows to use a single GTP-U tunnel as defined for non-TSN Transport networks.

Editor's note:	It is FFS if and how the Service Data Flows can be used to identity streams in the TN.

If the TimeAwareOffset is included from TN CNC to SMF/CUC, the SMF/CUC should sent the TimeAwareOffset to the AN-TL (for streams in UL direction) or the CN-TL port (for streams in the DL direction). The AL-TL/CN-TL derive Gate Control information (i.e., AdminBaseTime, AdminCycleTime, AdminControlListLength, and AdminControlList) based on the TimeAwareOffset as defined in IEEE Std 802.1Q [98] at the AN-TL (for streams in UL direction) and the CN-TL port (for streams in the DL direction). The AN-TL or CN-TL acting as Talker buffers the data burst until the time indicated in the TimeAwareOffset is reached.

If the SMF/CUC receives a TimeAwareOffset from TN CNC for a downlink stream (i.e. for a Talker in the UPF/CN-TL), the SMF/CUC adds the received TimeAwareOffset value to the TSCAI BAT in the DL direction in the TSCAI and updates the NG-RAN for the new TSCAI.

‐	FailedInterfaces (optional) provides a list of one or more physical ports of failed end stations or bridges to locate the interfaces in the physical topology that caused the failure. It is up to implementation how the SMF reacts when it receives FailedInterfaces.

NOTE 2:	It is assumed that the end station communication-configuration will contain at least the same information as defined for the status.

# * * * * Second change * * * *

5.28a.2	Transfer of TL-Container between SMF/CUC and AN-TL and CN-TL

When NG-RAN and UPF has signalled the support of AN-TL and CN-TL, the SMF/CUC may use the TL-Container to send a:

1)	get-request.

2)	set-request: submits the following elements of the Status group to the AN-TL or CN-TL:

-	list of InterfaceConfiguration(s) (one InterfaceConfiguration is associated with each QFI in the N3 tunnel)

submits the following elements of Talker group to the AN-TL or CN-TL, as derived by the SMF/CUC:

-	Interval.

-	MaxFrameSize.

The AN-TL or CN-TL may use the TL-Container to send a:

1)	get-response: indicates the following elements of the Talker or Listener group from the AN-TL or CN-TL:

-	EndStationInterfaces: list of InterfaceIDs, one InterfaceID is associated with each Local F-TEID.

-	InterfaceCapabilities.

-	Buffer capability: maximum possible buffer duration supported by the Talker.

2)	set-response: acknowledges the reception of the Status group to the SMF/CUC.

The SMF may request the UPF to report the support for AN-TL or CN-TL by including a TL-Container with a get-request to the AN-TL or CN-TN, respectively. The get-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 of TS 23.502 [3].

If the NG-RAN/UPF supports AN-TL/CN-TL, the NG-RAN/AN-TL or UPF/CN-TL responds with a TL-Container including the elements defined for the get-response. The elements are associated with the corresponding N3 Tunnel Info (Local F-TEID) in the NG-RAN or UPF.

The SMF may submit TL-Container including a set-request the elements defined for the set-request to NG-RAN/AN-TL or UPF/CN-TL. The set-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 of TS 23.502 [3]. The InterfaceConfiguration is associated with the corresponding QFI in the N3 tunnel in the NG-RAN or UPF, respectively. The AN-TL or CN-TL uses the provided configuration for the traffic in the QoS Flow of the given QFI as described in Annex M.

# * * * * Third change * * * *

5.27.3	Support for TSC QoS Flows

TSC QoS Flows use a Delay-critical GBR resource type and TSC Assistance Information. TSC QoS Flows may use standardized 5QIs, pre-configured 5QIs or dynamically assigned 5QI values (which requires signalling of QoS characteristics as part of the QoS profile) as specified in clause 5.7.2. For each instance of Periodicity, within each Period (defined by periodicity value), TSC QoS Flows are required to transmit only one burst of maximum size MDBV within the 5G-AN PDB. Known QoS Flow traffic characteristics provided in the TSCAI may be used to optimize scheduling in the 5GS.

The following is applicable for the QoS profile defined for TSC QoS Flows:

1.	The TSC Burst Size may be used to set the MDBV as follows:

The maximum TSC Burst Size is considered as the largest amount of data within a time period that is equal to the value of 5G-AN PDB of the 5QI. The maximum value of TSC Burst Size should be mapped to a 5QI with MDBV that is equal or higher. When integration with IEEE TSN applies, this 5QI also shall have a PDB value that satisfies the bridge delay capabilities (see clause 5.27.5 for more details) reported for the corresponding traffic class. For TSC QoS Flows, the Maximum Burst Size of the aggregated TSC streams to be allocated to this QoS Flow can be similarly mapped to a 5QI with MDBV value that is equal or higher. If interworking with a TSN network deployed in the transport network is supported, the maximum value of TSC Burst Size should be mapped to a 5QI with MDBV that is equal.

2.	The PDB is explicitly divided into 5G-AN PDB and CN PDB as described in clause 5.7.3.4. Separate delay budgets are necessary for calculation of expected packet transmit times on 5G System interfaces. For the TSC QoS Flow, the5G-AN PDB is set to value of 5QI PDB minus the CN PDB as described in clause 5.7.3.4. The CN PDB may be static value or dynamic value and is up to the implementation of 5GS bridge.

3.	When integration with IEEE TSN applies, the Maximum Flow Bitrate calculated by the TSN AF as per Annex I.1 may be used to set GBR. In this case, MBR is set equal to GBR.

4.	ARP is set to a pre-configured value.

5.	5QI value is derived using QoS mapping tables and TSN QoS information as described in clause 5.28.4 in the case of integration with IEEE TSN network, or using QoS Reference parameters and Requested PDB, Burst Size, Priority parameters as described in clause 4.15.6.6 or clause 4.15.6.6a of TS 23.502 [3] in the case of AF requested Time Sensitive Communication.

# * * * * End of changes * * * *