3GPP TSG-WG SA2 Meeting #156-e	S2-2305082r02

# Albonia, April 17 – 21, 2023		(revision of S2-2302925)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3930 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Hierarchical NSACF architecture procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, NEC, Xiaomi, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | eNS-Ph3(TR23.700-41) concluded the option of hierarchical NSACF architecture to support multiple service area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the service operation of NSAC for hierarchical NSACF architecture option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support the service operation of NSAC for hierarchical NSACF architecture option. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.1, 4.2.11.2a (new), 4.2.11.4a (new), ,4.2.11.x(new), 5.2.21.1, 5.2.21.2.2, 5.2.21.2.w (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.11	Network Slice Admission Control Function (NSACF) procedures

4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2]. If charging needs to be enabled, the NSACF may act as a NF (CTF) and interact with the CHF to support the Event based charging as defined in TS 32.290 [42].

Depending on the NSAC architecture deployed in the network, three options of NSAC procedures are defined:

- Option 1: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on non-Hierarchal NSAC architecture. The corresponding procedures are described in clause 4.2.11.2 and clause 4.2.11.4 respectively.

- Option 2: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on centralized NSAC architecture. The corresponding procedures are described in clause 4.2.11.2 and clause 4.2.11.4 respectively.

- Option 3: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on hierarchical NSAC architecture. The corresponding procedures are described in clause 4.2.11.2a and clause 4.2.11.4a respectively.

# * * * * Next change * * * *

4.2.11.2a	Hierarchical NSACF-based Number of UEs per network slice availability check and update procedure

Figure 4.2.11.2a-1: Hierarchical NSACF-based Number of UEs per network slice availability check and update procedure

For an S-NSSAIs subject to counting of the number of registered UEs, if hierarchical NSACF architecture is deployed in the network the enforcement of maximum number of UEs registered for an S-NSSAI is performed as follows:

1.	Same as the step 1 defined in clause 4.2.11.2.

2.	In addition to the information included in the Nnsacf_NSAC_NumOfUEsUpdate_Request as described in the step 2 of clause 4.2.11.2, the AMF may provide UE already registered indication to the NSACF if the registered S-NSSAI has already been registered in another service area before. The AMF determines the indication based on the received the Allowed NSSAI information from source AMF (in case of inter AMF handover) or from SMF+PGW-C (in case of mobility from EPS to 5GS).

3.	The NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the AMF indicates increase, the following applies:

- For NSACF which support UE admission quota based control:

-	If the local maximum number of UEs is not reached yet, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

-	If the local maximum number of UEs is reached, the NSACF sends a delegation request of NSAC to the Primary NSACF. Steps 4-8 are executed.

- For NSACF which support UE admission threshold-based control:

-	If the UE admission is below the threshold level, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

-	If the UE admission is at or above the threshold level and the local maximum number has not been reached, the NSACF checks whether the UE already registered indication is present.

- If the UE already registered indication is not present then the NSACF immediately rejects the NSAC request. Steps 4-8 are skipped.

- If the UE already registered indication is present the NSACF executes the same action as the step 3 defined in clause 4.2.11.2 in order to allow for service continuity. Steps 4-8 are skipped.

-	If the local maximum number has been reached and the UE already registered indication is present then the NSACF sends a delegation request of NSAC to the Primary NSACF in order to allow for service continuity. Steps 4-8 are executed. Otherwise, if the UE already Registered indication is not present, the NSACF immediately rejects the NSAC request and steps 4-8 are skipped.

If the update flag parameter from the AMF indicates decrease, the following applies:

- If the UE entry to be deleted is stored at the NSACF, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

- If the UE entry to be deleted is not stored at the NSACF, the NSACF sends a delegation request of NSAC to the Primary NSACF. Steps 4-8 are executed.

4.	If the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF which manages the entire PLMN service area per clause 6.3.22 of TS23.501 [2].

5.	The NSACF invokes Nnsacf_NSAC_NumOfUEsUpdate_Request to the Primary NSACF. The request includes the NSAC request information received from AMF, which may include the UE already Registered indication only if it is received from AMF and the UE admission type is quota-based.

6.	The Primary NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the NSACF indicates increase, the following applies:

- If the Primary NSACF decided to delegate the NSAC update request to the NSACF, per the applied UE admission type of the network, the Primary NSACF adjusts the local maximum number for UE admission quota based or the UE admission threshold for UE admission threshold based in its response  to the NSACF. The Primary NSACF does not create a new entry associated with the UE ID in the received NSAC request.

NOTE 1: When NSACF applies local maximum number update to Primary NSACF, the Primary NSACF either increase local maximum number at NSACF or reject the application.

- For quota-based admission type, and if the Primary NSACF decided not to delegate the request to the NSACF, and the UE already Registered indication is not included, the Primary NSACF rejects the NSAC request. If the UE already Registered indication is included, and if the Primary NSACF decided to store the UE entry, it creates a new entry associated with the UE ID within the received NSAC. If the Primary NSACF is not able to store the UE entry, it rejects the request.

- For threshold-based admission, and if the Primary NSACF decided not to delegate the request to the NSACF, the same action as step 3 in clause 4.2.11.2 is executed with the replacement of NSACF with Primary NSACF.

NOTE 2:	To support the session continuity across different service area, the Primary NSACF always keep part of the global maximum number, i.e. the whole global maximum number is not distributed to all contacted NSACF(s).

If the update flag parameter from the NSACF indicates decrease and the UE entry is managed by the Primary NSACF, the same action as step 3 in clause 4.2.11.2 is executed with the replacement of NSACF with Primary NSAC. This applies to both admission types.

7.	The Primary NSACF returns the Nnsacf_NSAC_NumOfUEsUpdate_Response message to the NSACF. The response may include the indication (e.g., rejection).

If the Primary NSACF determines to adjust the configured value stored at the NSACF, the updated local maximum number of UEs or UE admission threshold is also included in the response respectively.

8.	The NSACF checks the response from Primary NSACF.

If the response includes the updated configured value,

- The NSACF, which supports UE admission quota based control, replaces the existing local maximum number of UEs with the received updated value. The same action is executed per step 3 in clause 4.2.11.2 based on the updated configured value.

- The NSACF, which supports UE admission threshold based control, replaces the existing UE admission threshold with the received updated value. The same action is executed per step 3 in clause 4.2.11.2 based on the updated configured value.

If the response does not include the updated configured value, the NSACF returns the response to AMF based on the received NSAC response from Primary NSACF.

9.	Same as the step 4 defined in clause 4.2.11.2.

# * * * * Next change * * * *

4.2.11.4a	Hierarchical NSACF-based Number of PDU Sessions per network slice availability check and update procedure

Figure 4.2.11.4a-1: Hierarchical NSAC-based number of PDU Sessions per network slice availability check and update procedure

For an S-NSSAI subject to counting of the number of PDU sessions, if hierarchical NSACF architecture is deployed in the network the enforcement of maximum number of PDU Session established for an S-NSSAI is performed as follow:

1-2. Same as the steps 1-2 defined in clause 4.2.11.4.

3.	The NSACF performs NSAC for the indicated S-NSSAI.

If the PDU session ID entry update at the NSACF is possible, e.g. create a new entry associated with the received NSAC request for increase case, the same action as step 3 defined in clause 4.2.11.4 is executed. Steps 4-8 are skipped.

If the PDU session ID entry at the NSACF is impossible, i.e. by admitting the PDU session the local maximum PDU session number is exceeded, the NSACF applies an updated local maximum PDU session number from the Primary NSACF.

4.	If the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF, which manages the global service area as per clause 6.3.22 of TS23.501 [2].

5.	The NSACF invokes  Nnsacf_NSAC_LocalNumberUpdate request to the Primary NSACF. The request includes the S-NSSAI and the current local maximum number of PDU Sessions.

6.	The Primary NSACF checks the global maximum PDU session number and determines whether accept or reject the requested the updated local maximum PDU session number from NSACF, i.e. whether the update of the local maximum PDU session number of NSACF is accepted or not.

NOTE 1: When NSACF applies local maximum number update to Primary NSACF, the Primary NSACF either increase local maximum number at NSACF or reject the application.

7.	The Primary NSACF returns the Nnsacf_NSAC_LocalNumberUpdate response. The response includes a new allocated local maximum PDU sessions number or the indication to reject the request of updated local maximum PDU session number.

8.	If the primary NSACF provides an updated local maximum number, the NSACF replaces the local maximum PDU session number with the received local maximum PDU sessions number value. The same action is executed per step 3 in clause 4.2.11.4 based on the updated configured value.

9.	 Same as the step 4 of clause 4.2.11.4.

# * * * * Next change * * * *

# * * * * Next change * * * *

4.2.11.X	Update of local maximum number in Hierarchical NSAC Architecture

Figure 4.12.11.X-1: Update of local maximum number in hierarchical NSAC architecture

At any time the Primary NSACF may update the allocated local Maximum number of UE or PDU session configured at the NSACFs as follow:

1.	The Primary NSACF subscribes the slice event exposure service from all the related NSACFs supporting the requested S-NSSAI to get the status of the current registered number of UE or established PDU session number at NSACFs by invoking Nnsacf_SliceEventExposure_Subscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) service operation. The Event Reporting for the subscription is either a threshold based (e.g. the notification is triggered when the current number of UEs or PDU Sessions with a network slice reaches a defined threshold value) or a periodic reporting.

The Primary NSACF may request immediate reporting. If immediate reporting is requested, the subscription response contains the immediate report, i.e. the current registered number of UE or established PDU session number.

2.  When the reporting condition for a subscribed event is fulfilled, the NSACF triggers a notification towards the Primary NSACF.

3.	The NSACF invokes Nnsacf_SliceEventExposure_Notify (Event ID, Event Reporting information) service operation and notifies the status of the current registered number of UE or established PDU session number to Primary NSACF.

4.	Per the received current registered UE/PDU session number at NSACFs and operator's policy, the Primary NSACF decides to update the local maximum number of UE or PDU session values at the NSACF(s), i.e. the configured value at NSACF(s).

5.	The Primary NSACF invokes Nnsacf_NSAC_LocalNumberUpdate Request to the NSACF(s). The message includes the new configured value of local Maximum number of UE or PDU sessions.

NOTE 1: The new configured value(s) of local maximum number given by the Primary NSACF can be lower than the existing local maximum number configured at the NSACF(s).

6.	The NSACF replaces the local maximum number with the received new local maximum number value.

7.	The NSACF returns the Nnsacf_NSAC_LocalNumberUpdate Response to the Primary NSACF.

# * * * * Next change * * * *

5.2.21	NSACF services

5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NSAC | NumOfUEsUpdate | Request/Response | AMF, SMF, (NOTE 1) |
|  | NumOfPDUsUpdate |  | SMF |
|  | EACNotify |  | AMF |
|  | LocalNumberUpdate |  | NSACF, Primary NSACF |
| Nnsacf_SliceEventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF, AF (NOTE 2) |
|  | Unsubscribe |  | NEF, NWDAF, AF |
|  | Notify |  | NEF, NWDAF, AF |
| NOTE 1: If EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NumberOfUEs Update services operation and Nnsacf_NumberOfPDUsUpdate at PDN connection establishment procedure. NOTE 2: The AF can access NSACF services either via NEF to NSACF in case of untrusted AF or directly in case of trusted AF. |  |  |  |

# * * * * Next change * * * *

5.2.21.2.2	Nnsacf_NSAC_NumOfUEsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfUEsUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Availability Check (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NSAC_EACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice via the same Access Type configured in the NSACF is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), NF ID, Access Type, update flag.

Inputs, Conditional: Notification endpoint for EAC Notification for the S-NSSAI, UE already registered

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs registered with the network slice. The NSACF also takes Access Type into account for increasing and decreasing the number of UEs per network slice as described in clause 5.15.11.1 of TS 23.501 [2].

The NF ID parameter is the NF instance ID of the NF (e.g. AMF or SMF + PGW-C) sending the request to the NSACF.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

-	increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

-	decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

The UE already registered parameter indicates that the UE has already been registered in another NSACF service area before.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the maximum number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

Outputs, Required: Result indication.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of UEs for the S-NSSAI not reached' or 'maximum number of UEs for the S-NSSAI reached'.

Outputs, Optional: Updated local maximum number of UEs, Updated UE admission threshold.

# * * * * Next change * * * *

# * * * * Next change * * * *

5.2.21.2.w	Nnsacf_NSAC_LocalNumberUpdate service operation

Service Operation name: Nnsacf_NSAC_LocalNumberUpdate

Description: (from NSACF to Primary NSACF) the NSACF applies the Primary NSACF to get an updated local maximum number of UEs or PDU Sessions of the network slice for NSAC. (from Primary NSACF to NSACF) the Primary NSACF, use this service operation to update local maximum number of UEs or PDU sessions of the network slice at NSACFs. The Primary NSACF may use to update the admission threshold when threshold based approach is used.

In case of roaming

- From NSACF to Primary NSACF, the NSACF and the Primary NSACF are replaced with the Primary (or central) NSACF in VPLMN and the Primary (or central) NSACF in HPLMN, respectively.

- From Primary NSACF to NSACF, the Primary NSACF and the NSACF are replaced with the Primary (or central) NSACF in HPLMN and the Primary (or central) NSACF in VPLMN, respectively.

Inputs, Required: S-NSSAI, Update type .

Inputs, Optional: Updated local number (if the service consumer is Primary NSACF), PLMN ID of serving or home PLMN (for inter PLMN case)

The S-NSSAI is the network slice for which the NSACF is applying the updated local number update.

The update type parameter indicates whether the update of the local maximum number is increase or decrease.

The updated local number indicates the updated local maximum number of UEs, the updated UE admission threshold, or the updated local maximum number of PDU Sessions of the S-NSSAI.

Output, Required: Result indication.. .

Outputs, Optional: Updated local number (from NSACF to Primary NSACF)

# * * * * End of changes * * * *