3GPP TSG-WG SA2 Meeting #156-e	S2-2305082r01

# Albonia, April 17 – 21, 2023		(revision of S2-2302925)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3930 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Hierarchical NSACF architecture procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, NEC, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | eNS-Ph3(TR23.700-41) concluded the option of hierarchical NSACF architecture to support multiple service area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the service operation of NSAC for hierarchical NSACF architecture option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support the service operation of NSAC for hierarchical NSACF architecture option. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.1, 4.2.11.2, 4.2.11.2a (new), 4.2.11.4, 4.2.11.4a (new), 4.15.3.2.10, 4.15.3.2.10a (new),5.2.21.1, 5.2.21.2.2, 5.2.21.2.4, 5.2.21.2.w (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS23.501 CR3959 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.11	Network Slice Admission Control Function (NSACF) procedures

#### 4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2]. If charging needs to be enabled, the NSACF may act as a NF (CTF) and interact with the CHF to support the Event based charging as defined in TS 32.290 [42].

Depending on the NSAC architecture deployed in the network, three options of NSAC procedures are defined:

- Option 1: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on non-Hierarchal NSAC architecture. The corresponding procedures are described in clause 4.2.11.2 and clause 4.2.11.4 respectively.

- Option 2: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on centralized NSAC architecture. The corresponding procedures are described in clause 4.2.11.2 and clause 4.2.11.4 respectively.

- Option 3: The NSAC procedure for number of UEs or PDU sessions for an S-NSSAI is based on hierarchical NSAC architecture. The corresponding procedures are described in clause 4.2.11.2a and clause 4.2.11.4a respectively.

- * * * * Next change * * * *

#### 4.2.11.2a	Hierarchical NSACF-based Number of UEs per network slice availability check and update procedure

Figure 4.2.11.2a-1: Hierarchical NSACF-based Number of UEs per network slice availability check and update procedure

For an S-NSSAIs subject to counting of the number of registered UEs, if hierarchical NSACF architecture is deployed in the network the enforcement of maximum number of UEs registered for an S-NSSAI is performed as follows:

1.	Same as the step 1 defined in clause 4.2.11.2.

2.	In addition to the information included in the Nnsacf_NSAC_NumOfUEsUpdate_Request as described in the step 2 of clause 4.2.11.2, the AMF  may provide UE already registered indication  to the NSACF.

3.	The NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the AMF indicates increase, the following applies:

- For NSACF which support UE admission quota based control:

-	If the local maximum number of UEs is not reached yet, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

-	If the local maximum number of UEs is reached, the NSACF sends a delegation request of NSAC to the Primary NSACF. Steps 4-8 are executed.

- For NSACF which support UE admission threshold-based control:

-	If the UE admission is below the threshold level, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

-	If the UE admission is at or above the threshold level and the local maximum number has not been reached, the NSACF checks whether the UE already registered indication is present.

- If the UE already registered indication is not present then  the NSACF immediately rejects the NSAC request. Steps 4-8 are skipped.

- If the UE already registered indication is present the NSACF executes the same action as the step 3 defined in clause 4.2.11.2 in order to allow for service continuity. Steps 4-8 are skipped.

-	If the local maximum number has been reached and the UE already registered indication is present then the NSACF sends a delegation request of NSAC to the Primary NSACF in order to allow for service continuity. Steps 4-8 are executed.

If the update flag parameter from the AMF indicates decrease, the following applies:

- If the UE entry to be deleted is stored at the NSACF, the NSACF executes the same action as the step 3 defined in clause 4.2.11.2. Steps 4-8 are skipped.

- If the UE entry to be deleted is not stored at the NSACF, the NSACF sends a delegation request of NSAC to the Primary NSACF. Steps 4-8 are executed.

4.	If the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF which manages the global service area per clause 6.3.22 of TS23.501 [2].

5.	The NSACF invokes Nnsacf_NSAC_NumOfUEsUpdate_Request to the Primary NSACF.

The request includes the NSAC request information received from AMF. In addition it may include the current local maximum number of UEs or the current UE admission threshold respectively, which is the last value received from the Primary NSACF.

6.	The Primary NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the NSACF indicates increase, the following applies:

- The Primary NSACF creates a new entry associated with the UE ID within the received NSAC request due to the remaining global maximum number is only managed by Primary NSACF and not distributed to the NSACF(s) and executes the same action as step 3 in clause 4.2.11.2 is executed with the replacement of NSACF with Primary NSACF. Alternatively, the updated local maximum number of the UEs can be send to the NSACF by the Primary NSACF, for the admission control locally by the NSACF.

NOTE:	To support the service continuity across different service area, the Primary NSACF always keep part of the global maximum number, i.e. the whole global maximum number is not distributed to all contacted NSACF(s).

If the update flag parameter from the NSACF indicates decrease and the UE entry is managed by the Primary NSACF, the same action as step 3 in clause 4.2.11.2 is executed with the replacement of NSACF with Primary NSACF.

7.	The Primary NSACF returns the Nnsacf_NSAC_NumOfUEsUpdate_Response message to the NSACF. The response may include the indication (e.g., rejection) of NSAC delegation request for the S-NSSAI.

If the Primary NSACF determines to adjust the configured value stored at the NSACF, the updated local maximum number of UEs or UE admission threshold is also included in the response respectively.

8.	The NSACF checks the response from Primary NSACF.

If the response includes the updated configured value,

- The NSACF, which supports UE admission quota based control, replaces the existing local maximum number of UEs with the received updated value. The same action is executed per step 3 in clause 4.2.11.2 based on the updated configured value.

- The NSACF, which supports UE admission threshold based control, replaces the existing UE admission threshold and/or the maximum number of Registered UEs with the received updated value. The same action is executed per step 3 in clause 4.2.11.2 based on the updated configured value.

If the response does not include the updated configured value, the NSACF returns the response to AMF based on the received NSAC response from Primary NSACF.

9.	Same as the step 4 defined in clause 4.2.11.2.

# * * * * Next change * * * *

#### 4.2.11.4a	Hierarchical NSACF-based Number of PDU Sessions per network slice availability check and update procedure

Figure 4.2.11.4a-1: Hierarchical NSAC-based number of PDU Sessions per network slice availability check and update procedure

For an S-NSSAI subject to counting of the number of PDU sessions, if hierarchical NSACF architecture is deployed in the network the enforcement of maximum number of PDU Session established for an S-NSSAI is performed as follow:

1-2. Same as the steps 1-2 defined in clause 4.2.11.4.

3.	The NSACF performs NSAC for the indicated S-NSSAI.

If the PDU session ID entry update at the NSACF is possible, e.g. create a new entry associated with the received NSAC request for increase case, the same action as step 3 defined in clause 4.2.11.4 is executed. Steps 4-8 are skipped.

If the PDU session ID entry at the NSACF is impossible, i.e. by admitting the PDU session the local maximum PDU session number is exceeded, the NSACF proxies the request to the Primary NSACF for an updated local maximum PDU session number from the Primary NSACF.

4.	If the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF, which manages the global service area as per clause 6.3.22 of TS23.501 [2].

5.	The NSACF invokes  Nnsacf_NSAC_NumOfPDUsUpdate request to the Primary NSACF. The request includes the S-NSSAI and the current local maximum number of PDU Sessions.

6.	The Primary NSACF checks the global maximum PDU session number and determines whether accept or reject the requested the updated local maximum PDU session number from NSACF, i.e. whether the update of the local maximum PDU session number of NSACF is accepted or not.

7.	The Primary NSACF returns the  Nnsacf_NSAC_NumOfPDUsUpdate response. The response includes a new allocated local maximum PDU sessions number or the indication to reject the request of updated local maximum PDU session number.

8.	If the primary NSACF provides an updated local maximum number, the NSACF replaces the local maximum PDU session number with the received local maximum PDU sessions number value. The same action is executed per step 3 in clause 4.2.11.4 based on the updated configured value.

9.	 Same as the step 4 of clause 4.2.11.4.

# * * * * Next change * * * *

##### 4.15.3.2.10a	Number of UEs and PDU Sessions per network slice notification procedure in Hierarchical NSACF architecture

This procedure depicts the case of an AF subscribing to receive the registered number of UEs, or the number of PDU sessions in a specific S-NSSAI. The procedure handles the case when hierarchical NSACF architecture is deployed.

Figure 4.15.3.2.10a-1: Number of UEs and PDU Sessions per network slice notification procedure in Hierarchical NSACF architecture

1-2. Same as the steps 1-2 defined in clause 4.15.3.2.10

3.	The NEF may query the NRF to find the Primary NSACF as per the clause in.

4.	If the NEF has not already subscribed to the event from the NSACF for the requested S-NSSAI, the NEF initiates the request Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) to the Primary NSACFs supporting the requested S-NSSAI. The NEF stores the AF requested Event Reporting Information.

5.	The Primary NSACF confirms with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Response message to the NEF. This message may include the event reporting if available and immediate reporting or one-time reporting was requested by the NEF.

6.	If the Primary NSACF has not already subscribed to the event from the NSACF for the requested S-NSSAI, the Primary NSACF initiates the request Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) to the NSACFs supporting the requested S-NSSAI. The Primary NSACF may set the Event Reporting Information to periodic in its request to the NSACFs. The Primary NSACF also sets the Event ID and Event Filter identical to the received request from the AF.

NOTE 1:	The period chosen is selected by the Primary NSACF based on its internal logic.

7.	The NSACF(s) confirms with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Response message to the Primary NSACF.

8.	When the reporting condition for a subscribed event is fulfilled, the NSACFs trigger notification towards the Primary NSACF.

9.	The NSACF sends the Nnsacf_SliceEvent Exposure_Notify (Event ID, Event Reporting information) message to the Primary NSACF. If the subscription is for event based notification (e.g. based on the monitored event reaching a threshold value), the Event Reporting information parameter contains confirmation for the event fulfilment. If the subscription is for periodic notification or for immediate reporting, the Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the UEs established on the network slice).

10.	The Primary NSACF performs the aggregation from reporting NSACF(s) and maintain the overall usage of the S-NSSAI for the selected NSACFs as long as the subscription is active. The Primary NSACF includes the count for the current number of UEs registered or for the number of PDU session while aggregating the reports.

NOTE 2:	During UE’s mobility when the quota for the local maximum number is exceeded at NSACF then Primary NSACF may admit the request and increase the count for the current number of UEs registered or for the number of PDU session. Primary NSACF considers this usage while aggregating the report.

NOTE 3:	The Primary NSACF continuously updates the aggregated information to be able to fulfil the incoming subscription request from the NEF.

11.	When the reporting condition for a subscribed event by the NEF is fulfilled, the Primary NSACF sends Nnef_EventExposure_Notify (Event ID, Event Reporting information) message towards the NEF.

12. The NEF sends Nnef_EventExposure_Notify (Event ID, Event Reporting information) message towards the AF. In the case of untrusted AF; the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

# * * * * Next change * * * *

# * * * * Next change * * * *

##### 5.2.21.2.2	Nnsacf_NSAC_NumOfUEsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfUEsUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Availability Check (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NSAC_EACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice via the same Access Type configured in the NSACF is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), NF ID, Access Type, update flag.

Inputs, Conditional: Notification endpoint for EAC Notification for the S-NSSAI, UE already registered, current local maximum number, current UE admission threshold.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs registered with the network slice. The NSACF also takes Access Type into account for increasing and decreasing the number of UEs per network slice as described in clause 5.15.11.1 of TS 23.501 [2].

The NF ID parameter is the NF instance ID of the NF (e.g. AMF or SMF + PGW-C) sending the request to the NSACF.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

-	increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

-	decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

The UE already registered parameter indicates that the UE has already been registered in another NSACF service area. If this parameter is included, and the local NSACF cant handle the request, the local NSACF triggers a request to the Primary NSACF to handle the request.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the maximum number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

Outputs, Required: Result indication.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of UEs for the S-NSSAI not reached' or 'maximum number of UEs for the S-NSSAI reached'.

Outputs, Optional: Updated local maximum number of UEs, Updated UE admission threshold.

# * * * * Next change * * * *

##### 5.2.21.2.4	Nnsacf_NSAC_NumOfPDUsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfPDUsUpdate

Description: Updates the number of PDU Sessions established on a network slice (e.g. increase or decrease). Also, if the number of PDU Sessions on the network slice is to be increased, the NSACF first checks whether the number of the PDU Sessions on that network slice has reached the maximum number of PDU Sessions per network slice. If the maximum number of PDU Sessions on the network slice has already been reached, the PDU Session Establishment procedure is rejected.

Inputs, Required: S-NSSAI, UE ID, PDU Session ID, Access Type, update flag, current local maximum number.

The S-NSSAI parameter is the network slice for which the number of PDU Sessions established on a network slice is to be updated.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs that has established PDU sessions with the network slice.

PDU Session ID parameter is used by the NSACF to maintain for each UE ID, the PDU Session ID(s) for established PDU Sessions.

The Access Type parameter indicates over which access network type the PDU Session is established. In the case of MA PDU Session, one or multiple Access Types may be included for a PDU Session ID.

The update flag input parameter indicates 'increase', 'decrease' or 'update' as specified in clause 4.2.11.4.

Inputs, Optional: None.

Outputs, Required: Result indication, Access Type.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of PDU Sessions for the S-NSSAI not reached' or 'maximum number of PDU Sessions for the S-NSSAI reached'.

The Access Type parameter is associated with the Result indication parameter.

Outputs, Optional:  Updated local maximum number of PDUs.

# * * * * End of changes * * * *