3GPP TSG-WG SA2 Meeting #156E 	S2-2305092

April 17 – 21, 2023, Elbonia	(revision of S2-230xxxx)

Source:	Philips International B.V.

Title:	Updates related to SL Positioning Server

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper updates and clarifies several aspects related to the SL Positioning Server.

# 1. Introduction/Discussion

This paper proposes the following changes:

- The text in clause 5.3.1 related to the SL Positioning Server UE about the LMF involving the Target UE or Reference UE for result calculation is confusing. It also does not seem to be in line with the wording in the conclusion for KI#4 on this aspect. Since mixing terms and definitions should be avoided, we propose to use the term SL Positioning Server UE instead of Target UE and Reference UE. The last sentence of that paragraph can be combined with the previous sentence.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

### 5.3.1	General

Any UE supporting Ranging/SL Positioning, e.g. Target UE, Reference UE, Sidelink Positioning Server UE, shall have a Ranging/SL Positioning layer, which is above the AS layer and handles service requests received from application layer to control the Ranging/SL Positioning operation. The Ranging/SL Positioning layer provides the support of authorization and provisioning as described in clause 5.1, the UE discovery and selection as described in clause 5.2, the Ranging/SL Positioning Protocol (RSPP) defined in TS 38.xxx [x], and the protocols between UE and LMF for Ranging/SL Positioning. Ranging/SL Positioning layer supports the Ranging/SL Positioning service sessions for one time or periodic ranging, ranging for distance or direction measurement or both.

Editor’s note: The specification number for RSPP (SLPP for RAN) will be added based on RAN WG input.

Editor’s note: The protocol used between UE and LMF will be decided by RAN and could be an extension of LPP, a new protocol or both.

The RSPP is exchanged over SR5 over PC5-U reference point between UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE) to manage the Ranging/SL Positioning service sessions between UEs or among a group of UEs. Multiple UEs may be involved in a single Ranging/Sidelink Positioning session. When the Ranging/SL Positioning service session is among a group of UEs, the RSPP may provide group related identifier information for the PC5 transport layer handling. The transport of the RSPP over PC5-U is specified in clause 5.3.2.  The RSPP supports the following functions:

-	exchange the Ranging/Sidelink Positioning capability;

-	exchange the Ranging/Sidelink Positioning assistant data;

-	exchange Ranging/Sidelink positioning measurement data/result.

NOTE 1:	Group management is out-of-scope of this specification.

NOTE 2: 		RSPP signalling negotiation determines whether measurement results or Ranging/Sidelink Positioning results are exchanged over the SR5.

A LMF shall be involved when at least one of the Target UE and the Reference UE are in the network coverage and the serving network is capable to support ranging functionalities. The details of Network assisted Ranging/SL Positioning operation is described in detail in clause 5.5. When Network assisted Ranging/SL Positioning is used, the use of RSPP is necessary for Ranging/SL Positioning control.

A SL Positioning Server UE can be discovered and selected for result calculation, method determination, assistant data distribution and SL Reference UE selection in case of out-of-coverage or for UE-only Operation if the serving network does not support Ranging/SL Positioning. If the LMF capable for Ranging/SL Positioning is reachable by Target UE and/or Reference UE, the LMF can still decide that a SL Positioning Server UE (e.g. co-located/integrated with a Target UE or Reference UE) executes the result calculation.

Editor’s note:	Functionalities of the SL Positioning Server UE will be determined by RAN WGs.

# * * * * End of changes * * * *