3GPP TSG-WG SA2 Meeting #156-e	S2-2305099r05

# Elbonia, April 17 – 21, 2023		(revision of S2-2305099r01)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2097 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G-RG ID provided in Trusted Non-3GPP access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For trusted and untrusted non-3GPP access, UE connects to the overlay 5G network using the trusted and 3GPP access approaches as illustrated in AnnexA of TS 23.316 and clause 4.10 of TS 23.316 respectively. In addition to UE being connected to the underlay 5G network, the 5G-RG also acts as TNAP with respect to the TNGF in the overlay network i,e it has an established Ta reference point with the TNGF. Base on the TR conclusion, if 5G-RG acts as a TNAP, it shall provide its 5G-RG ID as TNAP ID to the TNGF over Ta, then by the TNGF to the PCF of the UE as ULI carried via AMF and SMF |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is mandate that when the 5G-RG acts as a TNAP , the 5G-RG send its ID as TNAP ID to TNGF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of UE connecting to TNGF via a 5G-RG, if the TNAP ID does not corresponds to the 5G-RG, but to the BSSID or civic addrss, the PCF is not able to differentiate the QoS of UEs connecting behind the 5G-RG since it can do derive to which 5G-RG the UE is conencted to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The clause in NOTE 5 is a new clause defined in 23.316 CR2086 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.10	UE behind 5G-RG and FN-RG

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5GC capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a UE, the control and user plane packets of the UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an IWF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 4.10-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

Figure 4.10-2 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

NOTE 1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered in this specification as it is assumed that FN-RG does not support EAP-based access control (e.g. 802.1X).

Figure 4.10-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via 5G-RG, NG-RAN or via both accesses.

Figure 4.10-2: Architecture for UE behind 5G-RG using untrusted N3GPP access

The TNGF and Ta reference point are defined in TS 23.501 [2]. In addition to the requirements described in TS 23.501 [2], the Ta reference point should be able to carry the TNAP ID to the TNGF.

NOTE 2:	The reference architecture in figure 4.10-1 only shows the architecture and the network functions directly connected to W-5GAN or TNGF, and other parts of the architecture are the same as defined in clause 4.2 of TS 23.501 [2].

NOTE 3:	The reference architecture in figure 4.10-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 4:	The two N2 instances in Figure 4.10-1 apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and W-5GAN.

NOTE 6:	For trusted non-3GPP access, UE connects to the overlay 5G network using the trusted non-3GPP access approach. In addition to being connected to the underlay 5G network, the 5G-RG also acts as TNAP with respect to the TNGF in the overlay network i.e. it has an established Ta reference point with the TNGF.

For NWu scenario, the overlay and the underlay network as described in clause D.7 of TS 23.501 [2] may be administrated by different operator's domain, hence an SLA can be used (as defined clause D.7 of TS 23.501 [2]). If the same operator deploys both overlay and the underlay network, e.g. as in the case for trusted non-3GPP access (NWt), there is no need for SLA but that operator's network configuration determines how to handle the DSCP markings in the underlay network.

A 5G-RG acting as a TNAP shall provide its TNAP ID. to the TNGF and the TNGF provides this TNAP ID as part of ULI (user Location Information) sent to the 5GC; this information is propagated to the PCF that may use it to determine PCC rules depending on whether an UE is using a 5G-RG as a host or as a guest ;

#### NOTE 6: QoS and charging differentiation based on user location (e.g., home or guest users) can be applied when the user is connected via a TNGF reached over a 5G-RG. The PCF may use the TNAP ID, which is available to it as a part of ULI. For example, if the TNAP ID is included in the UE’s policy control subscription information (the UE is considered a home user. Alternatively, the PCF may use TNAP ID provided by an AF using the Service Specific parameter provisioning as defined in clause 4.10.X

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *