3GPP TSG-WG SA2 Meeting #156-e	S2-2305172r02

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0370 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SL-MO-LR Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion of FS_Ranging_SL, there shall be a procedure specified for SL positioning procedure with LMF involved. It shall be supported SL positioning with LMF in Mobile Originated Service Request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SL-MO-LR procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobile Originated SL Positioning service is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.X	Procedures of SL-MO-LR involving LMF

Figure 6.X-1. SL-MO-LR procedure

0. UE1 and other UEs are authorized for Ranging/SL positioning service and are provisioned with policy/parameters on Ranging/SL positioning service as specified in clause 6.2 of TS 23.586 [x].

1.	UE1 may be triggered for SL-MO-LR. The location request may include its destination for example an LCS client or AF. And the location request may include requested location service (e.g., absolute location, relative location, etc) and QoS requirements (e.g., accuracy, response time, LCS QoS Class).

2.	UE1 discovers SL Reference UEs (UE2, UE3, UE4,…) and select number of SL reference UEs as candidate SL reference UEs among discovered UEs for example, signal strength, QoS requirement, etc. UE1 may setup PC5 connections with the selected SL reference UEs.

3.	If UE1 is in CM-IDLE state, UE1 initiates a UE triggered Service Request in order to establish a signalling connection with its serving AMF.

4.	UE1 sends AMF a UL NAS TRANSPORT message including SL-MO-LR request. The SL-MO-LR Request includes its destination information for example an LCS client or AF and requested location service and QoS information, and information of SL reference UEs which are selected in step 1.

5.	AMF select a LMF to handle the location request from UE1.

6.	AMF sends an Nlmf_Location_DeterminedLocation service operation toward the selected LMF including SL-MO-LR request.

7.	LMF, UE1 and SL reference UEs may communicate to perform SL positioning capability negotiation.

LMF may select SL positioning method based on QoS information included in the location request, negotiated UE1’s capability and available reference UEs. LMF may downselect SL reference UEs based on decided SL positioning methods.

LMF may decide whether LMF, UE1 or other UE perform calculation of location result.

8.	LMF may send assistance information to UE1 and selected SL reference UEs.

9.	LMF or NG-RAN may make resource assignment of UE1 and SL reference UEs for SL positioning.

NOTE : It is assumed that serving LMF of UE1 and SL reference UEs are same.

Editor’s Note : It is FFS how LMF may communicated with SL reference UE when serving AMF of SL reference UEs are different or when some SL reference UEs are out of coverage of serving LMF.

10. LMF sends a request for location information to UE1 to trigger ranging/SL positioning procedure in step 11.

11.	UE1 and SL references UEs perform Ranging/SL positioning procedures.

12.	UE1 sends Ranging/SL positioning result to the LMF.

If location calculation is performed in UE, the calculation result is shared in step 11. Otherwise, LMF calculates location based on the received result.

LMF may determine UE1’s location based on received SL positioning result and known location of reference UEs. If needed, LMF may perform the retrieval of a new location of reference UE(s) which is available for Uu positioning with the LMF.

Editor’s Note : It is FFS whether LMF to obtain measurement data from SL reference UEs.

13.	LMF sends AMF Nlmf_Location_DetermineLocation Response which include location result.

14.	AMF may send location result to the indicated destination based on step 7-12 of clause 6.2

15. AMF sends DL NAS TRANSPORT message including SL-MO-LR response to UE1. If the UE1 is requesting its own location, the response carries the location result received from LMF.

# * * * * End of changes * * * *