3GPP TSG-SA2 Meeting #156E	  S2-2305238

# Elbonia, April 17 – 21, 2023										   						(revision of S2-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4192 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | High level feature description for AIMLsys |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The high-level description of the features described in 23.501 are focused to provide the enablers for the feature to work, and the enablers as such are defined somewhere else, including other TSs. The list of enablers is incomplete, as the Editor´s Note shows that there may be other enablers needed: Editor’s note: It is FFS with additional clauses to summarize the other respective 5GC assistance functionalities for the AI/ML operations in the application layer and the references to the relevant normative clauses once the normative work becomes stable. The respective 5GC assistance functionalities are: - Network resource performance and monitoring - Information exposure to authorized 3rd party - External parameter provisioning - Planned group data transfer support with QoS for application traffic - QoS and Policy requirements - Member Selection Assistance functionality - Network data analytics support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rather than an overview, the clause describes enablers to support certain AIML operations. These AIML operations are listed, as stage 1 requirements are evolving, a stable design base is required. The following enablers are listed, and references to the clause that describe them is added. - AF request for a list of UEs that fulfil certain conditions. - AF request to provide an estimation of the time window to transfer data with QoS requirements to a list of UEs. - AF request to provide QoS for a list of UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | General archirecture principles are not captured in description of the item. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.46.1	General

This clause describes the list of 5GC enablers to support the following AI/ML operations in the Application layer over the 5G System:

- AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing

-	Distributed/Federated Learning

NOTE:	Requirements on AI/ML operations are specified in clause 6.40 of TS 22.261 [2].

The AF may subscribe to NEF event as described in clause 4.15.3.2.3 in TS 23.502 [3] to assist its application AI/ML operation. For example, the AF may subscribe to session inactivity time monitoring event to assist the AI/ML application server in scheduling available UE(s) to participate in the AI/ML operation (e.g. Distributed/Federated Learning). In addition, the AF may subscribe to NEF to be notified on the traffic volume shared between the UE and the AI/ML application server as described in clause 4.15.3.2.3 in TS 23.502 [3]. This may help the AI/ML application server to determine, e.g., how large the model should be for certain AI/ML operation.

The AF that aims to provide an AI/ML operation e.g. Distributed/Federated Learning, may request assistance from the 5GC as described in clause 5.46.2 by providing a list of UE, then subscribing to the NEF to be notified about the subset list of UEs that fulfil certain filtering criteria. Details of the procedures are described in clause 4.15.13 of TS 23.502 [3]. This subset list of UEs may become the list of candidate UEs for this AI/ML operation depending on the AF filtering criteria. Alternatively, the AF may select the list of UEs for the AI/ML operation e.g. Distributed/Federated Learning, without NEF involvement as described in Annex I of TS 23.502 [3]: in this case, the AF may determines a list of UEs without any assistance from the 5GC and proceeds with the AI/ML operation over 5G system.

The AF that has requested a list of candidate UEs, e.g. for an AI/ML operation of Distributed/Federated Learning, may request the network to provide the preferred time window for the AI/ML operation using the Planned Data Transfer with QoS (PDTQ) requirements and procedures as described in clause 6.1.2.7 of TS 23.503 [45] and in clause 4.15.16 of TS 23.502 [3].

At the time or during the AI/ML Operation. e.g. Distributed/Federated Learning, the AF requests the serving NEF to provide QoS for the list of candidate UEs, each UE identified by its UE IP address, that were selected as described in clause 4.15.13 of TS 23.502 [3]. The AF may subscribe to QoS Monitoring which may include also Consolidated-MBR monitoring  as described in clause 5.45 and in clause 4.15.6.13 of TS 23.502 [3] for those AF requests for QoS that result in a successful resource allocation. The AF provides QoS parameters that are derived from the performance requirements listed in clause 7.10 of TS 22.261 [2].

As result of the subscription to NEF to provide the subset list of UEs that fulfil certain filtering criteria, the AF may be notified about changes in the subset list of UEs, as such the AF may request a new preferred time window for the AI/ML operation using the Planned Data Transfer with QoS (PDTQ) as described in clause 6.1.2.7 of TS 23.503 [45] and in clause 4.15.16 of TS 23.502 [3]. The AF may request the NEF to provide QoS for the updated list of candidate UEs, each identified by UE IP addresses, that results in QoS resources previously allocated to some UE IP addresses may be released, while other UE IP addresses may require QoS for resource allocation and QoS monitoring.

AF may continue to leverage Network Data Analytics as defined in TS 23.288 [86], such as End-to-end data volume transfer, DN performance analytics, Network performance analytics, UE mobility analytics, WLAN performance analytics etc. to assist its AI/ML operations.

In addition, the following principles apply when 5GS assists the AI/ML operation at the application layer:

-	AF requests to the 5G System in the context of 5GS assistance to AI/ML operations in the application layer shall be authorized by the 5GC using existing mechanisms.

NOTE:	In this Release, assistance to AI/ML operations in the application layer is not supported to roaming UEs.

-	UPF can perform traffic detection for AI/ML traffic as defined in clause 5.8.2.4 and the network needs to support charging for AI/ML traffic.

Editor's note:	How the network and/or AF can differentiate AI/ML traffic is FFS.

-	Application AI/ML decisions and their internal operation logic reside at the AF and UE application client and is out of scope of 3GPP.

-	Based on application logic, it is the application decision whether to request assistance from 5GC e.g. for the purpose of selection of UEs that participate in certain AI/ML operation.

-	In this Release, 5GS assistance to AI/ML operations in the application layer is conducted within a single slice.

Editor's note:	It is FFS with additional clauses to summarize the other respective 5GC assistance functionalities for the AI/ML operations in the application layer and the references to the relevant normative clauses once the normative work becomes stable. The respective 5GC assistance functionalities are:
	
-	Information exposure to authorized 3rd party.
-	External parameter provisioning.

* * * End of Changes * * *