3GPP TSG-SA2 Meeting # 156E	S2-2305266

# Elbonia, April 17-21, 2023															 (revision of S2-XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2099 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Differentiated service for NAUN3 devices connected behind a 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, Differentiated QoS and charging for NUAN3 devices behind 5G-RG is not supported. This CR proposes QoS and charging differentatiation for UE(s) behind 5G-RG using the following - Grouping NUAN3 known as “connectivity group” and providing QoS and charging to the group based on URSP rules. - URSP enhancement to provide “connectivity group” rules to the 5G-RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing QoS and charging differentiation for NAUN3 devices behind 5G-RG and URSP enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature as defined in the conclusion is not currently supported in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.316 CR 2099. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text) * * *

### 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices cannot be authenticated by 5GC but may be locally authenticated by the 5G-RG using pre-shared secret etc. . Differentiated QoS servivces for NAUN3 devices are provided using local configuration in the 5G-RG and based on DNN/S-NSSAI for the 5G-RG’s PDU sessions.

The 5G-RG can set up distinct PDU Session for each group of NAUN3 devices based on the local configuration in the 5G-RG. For example, local configuration of the 5G-RG can allow the local admin to group NAUN3 devices based on 5G-RG logical ports and set different QoS levels/priority levels for a group of NAUN3 devices. Based on these local configuration and operator provided policies, the 5G-RG can map each group of NAUN3 devices to a distinct PDU Session.

The NAUN3 devices are classifed into “connectivity groups” where each group connects to the 5G-RG using a separate physical or virtual port on the 5G-RG. These ports could for example, refer to separate physical ethernet ports and/or a separate WLAN SSIDs and the groups may also be separted on the LAN behind the RG using VLANs. The devices that connect to a certain logical port are considered part of the same connectivity group. How this configuration is done is out of scope of this specification.

Each connectivty group is then mapped to a separate PDU Sesssion that is established by 5G-RG based on the procedures defined in clause 7. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69, TR-360 and TR-181. URSP rules can be provided to the RG to indicate how to map a Connectivity Group ID to e.g. DNN, S-NSSAI etc. Alternatively, the 5G-RG can be manually configured by the admin to form the connectivity groups.

The mapping between virtual ports and DNN/S-NSSAI can be configured using URSP, where e.g. VLAN IDs and Connectivity Group ID can be used as Traffic Descriptors, provided that the 5G-RG has a corresponding configuration of virtual ports with the same VLAN IDs or Connectivity Group ID.

In addition, the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69/TR-181.

How the NAUN3 devices are configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG is out of scope of this specification.

Differentiation of charging and QoS is provided via existing mechanisms. The solution also allows isolation of the connectivity groups into separate network slices, i.e. with separate N-SSAIs for each PDU Session.

* * * End of Changes * * *