# SA WG2 Meeting #S2-156e	S2-2305287

# 17 - 21 Apr, 2023, Electronic meeting

# Title:	[draft] Reply LS to LS to SA2 on Sidelink positioning procedure

# Response to:	LS to SA2 on Sidelink positioning procedure (S2-2303952/R2-2302285)

# Release:	Release 18

# Work Item:	Ranging_SL

Source:	SA2

To:	RAN2

Cc:	SA3

Contact Person:	Sherry (Yang) Shen

shenyang6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: S2-230xxxx (pCR for clause 6.7 of TS 23.586), S2-230xxxx (clause 6.x&6.y of TS 23.273)

1. Overall Description:

SA2 thanks RAN2 for informing agreements of the overall signalling procedure for PC5-only positioning and the further questions.

SA2 has discussed the questions asked by RAN2 with the following answers:

Question 1: Does SA2 have any comments on RAN2 agreed PC5-only positioning procedure?

SA2 Answer: SA2 has no comments on the procedure.

Question 2: Regarding the step 1 trigger event, RAN2 would like to understand whether SA2 will specify the triggering event for an SLPP session?

SA2 Answer: As defined in procedures for UE-only Operation and Network-assisted Operation in the attachments, the service request received from Ranging/SL Positioning application layer (triggered by a SL Positioning Client UE over PC5 or by any application on the UE)and the service request received from LMF triggers the UE discovery and then the SLPP session.

Regarding to UE (e.g. Located UE and SL Positioning UE) discovery and selection, SA2 has decided to include a RSPP (i.e. SLPP) metadata field during discovery to enable RAN2 to add information it deems necessary to be exchanged during discovery to aid in SL positioning. This also implies that SA2 expects RAN2 to include the supported UE roles (e.g. SL Reference UE, Located UE, SL Positioning Server UE). In SA2 we distinguish between SL Reference UE and Located UE roles, and hence sincerely ask RAN2 to define such roles (or an equivalent thereof) to be discovered using the RSPP metadata field.

In addition, SA2 has the following questions regarding UE discovery and selection:

Question 1: Whether and how LMF can select a Located UE from the list of candidate Located UEs for the Sidelink Positioning of the Target UE based on e.g. the supported Sidelink Positioning methods and the required QoS? Same question for the Target UE.

Question 2: What are the criteria from RAN2 perspective for the selection of a Located UE and SL Positioning Server UE?

Question 3: What are the criteria from RAN2 perspective to determine if the location of a UE is sufficiently stable and accurate in order for the UE to announce itself as a “Located UE” for the discovery?

2. Actions:

To RAN2

ACTION: 	Take SA2 answers into consideration and give feedback to SA2 on the SA2 further questions.

3. Date of Next SA2 Meetings:

SA2#157	22th - 26th May 2023	Berlin, DE

SA2#158	21st - 25th Aug 2023	Goteborg , SE